| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0133 |
| --- | --- |

| Title: | SCCE1: Test 3.5 - Block vector difference coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, peter.chuang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of SCCE1 Test 3.5 on the proposed method in JCTVC-Q0095 for coding the block vector prediction difference associated with the Intra block copy (BC) coding mode. The proposed method re-uses the existing binarization scheme employed for coding the motion vector difference in the current HEVC. However, an improved context modeling scheme has been developed for entropy coding the resulting bin string. Using the original context modeling scheme in JCTVC-Q0095 with 15 additional contexts, it is reported to achieve 4.0%, 2.0%, and 1.5% Luma BD-rate savings for category YUV, text & graphics with motion, 1080p sequences under the AI, RA, and LB, settings, respectively, of the SCC CTCs. Using the reduced context modeling scheme with 4 additional contexts, it is reported to achieve 3.7%, 1.8%, and 1.4% Luma BD-rate savings for category YUV, text & graphics with motion, 1080p sequences under the AI, RA, LB settings, respectively, of the SCC CTCs.

# Introduction

A new intra block copy (Intra BC) coding tool, indicated by a new syntax flag intra_bc_flag, has been adopted into Screen content coding test model (SCM) 1. When intra_bc_flag is equal to 1, the current CU is coded in the Intra BC mode and is predicted by the reconstructed block region before deblock & SAO filtering in the current frame, followed by residual coding. The location of the reference block is indicated by a block vector, specifying the coordinate offsets in the horizontal and vertical directions, respectively, from the current block.

According to the current HEVC Text Draft 6, the block vector of the current Intra BC CU is predicted by the block vector of the last Intra BC CU. The resulting prediction difference of the block vector is then coded by re-using the existing syntax for coding the motion vector difference in an Inter CU. The absolute value of each component of vectors is binarized by the truncated unary code, cascaded with the 1st order of Exp-Golomb (EG1) code when the absolute value is greater than 2. The first two bins of the resulting bin string are encoded in the CABAC mode. For the absolute value of syntax component greater than one, the absolute value minus 2 is binarized by the EG1 code and encoded in the bypass mode.

# Proposed method

This contribution evaluates the method for coding the block vector difference associated with the Intra BC coding mode first proposed in [1]. The proposed method represents the absolute value, | d |, of each component of the block difference vector by its most significant bit (MSB) index followed by its refinement bits. The MSB index for syntax value 0 is assigned to -1. The MSB index plus one, referred to as msb_plus_one, is binarized by the unary code. The remaining refinement bits are represented in the fixed-length binary code with the codeword length determined by the MSB index. Incidentally, this binarization scheme leads to exactly the same bin string as that of the binarization scheme employed in the current HEVC for coding motion vector difference, as compared in Table 1.

The resulting bin stings are coded by context adaptive binary arithmetic coding (CABAC). The three new syntax elements abs_bvd_msb_idx_p1, abs_bvd_refinement_bits, and bvd_sign_flag are introduced for representing msb_plus_one, refinement bits and sign flag of a block vector component difference, respectively. The bin string for abs_bvd_msb_idx_p1 is entropy coded in the CABAC mode if the bin index is smaller than a threshold and in the bypass mode, otherwise.

The context selection is conditioned on bin index and vector component index. In the original context modeling scheme proposed in [1], each bin is assigned a single context with threshold value equal to 8 & 7 for the horizontal component and 7 for the vertical component, respectively. The total 15 contexts are thus added, 8 for the horizontal component and 7 for the vertical component.

An additional context modeling method with reduced contexts is also evaluated in this SCCE test. Table 2 provides the mapping table for context selection. The first 2 contexts are shared with the existing contexts for MVD coding and the total 4 contexts are added.

The refinement bins and sign flags are coded in the bypass mode. The block vector difference Bvd[ compIdx ] for component compIdx = 0..1 is derived as follows:

- If abs_bvd_msb_idx_p1[ compIdx] is equal to 0,

Bvd[ compIdx ] = 0.

- Otherwise (abs_bvd_msb_idx_p1[ compIdx] is greater than 0),

Bvd[ compIdx ] =
		(  ( 1 << ( abs_bvd_msb_idx_p1[compIdx] – 1 ) )   +  abs_bvd_refinement_bits[compIdx] ) * 	( 1 − 2 * bvd_sign_flag[ compIdx ] )

Table 1: The bin strings generated by the proposed method and HEVC mvd coding

|  | Proposed Method |  | HEVC mvd_coidng() |  |  |
| --- | --- | --- | --- | --- | --- |
| ¦ d ¦ | msb_plus_one | refinement bins | TU prefix | EG1 prefix | EG suffix |
| 0 | 0 | - | 0 | - | - |
| 1 | 10 | - | 10 | - | - |
| 2 | 110 | 0 | 11 | 0 | 0 |
| 3 | 110 | 1 | 11 | 0 | 1 |
| 4 | 1110 | 00 | 11 | 10 | 00 |
| 5 | 1110 | 01 | 11 | 10 | 01 |
| 6 | 1110 | 10 | 11 | 10 | 10 |
| 7 | 1110 | 11 | 11 | 10 | 11 |

Table 2: Context assignment for entropy coding abs_bvd_msb_idx_p1 of the BVD components

| bin index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | > 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Horizontal component | 5 | 1 | 1 | 2 | 3 | 4 | 4 | bypass |
| Vertical component | 0 | 1 | 1 | 2 | 3 | 4 | 4 | bypass |

# Experimental results

The proposed method has been implemented using reference software HM-14.0+RExt-7.0+SCM-1.0 as the code base. The simulation is conducted according to the common test condition specified in [2]. The simulation results are summarized in Tables 3 and 4 for lossy and lossless coding, respectively, using the original context modeling scheme proposed in JCTVC-Q0095. The simulation results with reduced contexts are summarized in Tables 5 and 6 for lossy and lossless coding, respectively. The detailed simulation results can be found in the included spread sheet files.

Table 3: BD-rate results versus the anchor, JCTVC-Q0095

Table 4: Lossless bit-rate results versus the anchor, JCTVC-Q0095

Table 5: BD-rate results versus the anchor, reduced contexts

Table 4: Lossless bit-rate results versus the anchor, reduced ontexts

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei, “AHG8: Coding the prediction differences of the intra BC vectors,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q0095, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.