| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0136 |
| --- | --- |

| Title: | Run coding of the palette index map using a universal entropy coding scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, peter.chuang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution develops a new method for coding syntax palette_run of palette coding for improved compression efficiency. The proposed method utilizes a universal entropy coding scheme representing a syntax value by its most significant bit (MSB) index and refinement bits. It is reported to achieve average Luma BD-rate savings of 0.9%, 0.4%, and 0.3% for the AI, RA, and LB settings, respectively, using IBC with a full-frame search range and 1.3%, 0.6%, and 0.4% for the AI, RA, and LB settings, respectively, using IBC with a 2-CTU search range in the YUV, text & graphics with motion, 1080p category compared with the related results of the SCCE3 palette base software under the SCC common test conditions.

# Proposed method

This contribution proposed a new method for coding syntax palette_run for coding the palette index map by using a universal entropy coding scheme. This entropy coding method, previously employed in [3] for coding IBC block vector difference, represents a syntax value by sending its most significant bit (MSB) index followed by its refinement bits. The bin string for a binarized syntax value is thus a concatenation of a prefix bin string and a suffix bin string. The prefix string represents the MSB index plus 1, or msb_id_plus1, of a syntax value in a unary code, or a truncated unary code if the maximal syntax value is known. msb_id_plus1 of a unsigned syntax x is given by

msb_id_plus1 =  Floor ( Log2( x ) ) + 1,  	if x > 0; 
                                      0,   	otherwise.

The suffix bin string represents the refinement bits and is present if msb_id_plus1 > 1. The value of refinement bits, denoted by refinement_bits, is coded in a fixed-length binary code with the string length equal to msb_id_plus1 – 1. The decoded syntax value x is given by

x  =  (1 << ( msb_id_plus1-1)) + refinement_bits,  if msb_id_plus1 > 1;

msb_id_plus1,  otherwise.

Context modeling for entropy coding the bin string of binarized msb_id_plus1 of palette_run is conditioned on the bin index and the two separate context sets are employed, corresponding to palette_run_type_flag equal to COPY_ABOVE_MODE and INDEX_MODE, respectively. Inspired by the observation that palette_run is often equal to 0 when palette_run_type_flag is equal to INDEX_MODE and the current palette index is large, context modeling for bin index 0 is further conditioned on the palette index for the INDEX_MODE run type. The context index is assigned to 0 if palette_index <= 1, 1 if 1 < palette_index <=7, or 2 otherwise. Table 1 provides the mapping table for context selection currently employed by our proposal.

Table 1: Mapping table for context selection for coding palette_run

# Experimental results

The proposed method has been implemented using the SCCE3 palette reference software [1] as the code base. The simulation is conducted according to the common test condition specified in [2]. The results of the proposed method compared with the SCCE3 palette base software using IBC with the full-frame search range are summarized in Tables 2 & 3 for lossy and lossless coding, respectively. The results of the proposed method compared with the SCCE3 palette base software using IBC with the 2-CTU search range are summarized in Tables 4 & 5 for lossy & lossless coding, respectively. The detailed simulation results can be found in the included spread sheets.

Table 2: BD bitrate results of the proposal compared with the SCCE3 palette anchor using IBC with the full-frame search range for lossy coding

Table 3: Bitrate savings of the proposal compared with the SCCE3 palette anchor using IBC with the full-frame search range for lossless coding

Table 4: BD bitrate results of the proposal compared with the SCCE3 palette anchor using IBC with the 2-CTU search range for lossy coding

Table 5: Bitrate savings of the proposal compared with the SCCE3 palette anchor using IBC with the 2-CTU search range for lossless coding

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-W. Huang, et al., “Description of Screen Content Core Experiment 3 (SCCE3): Palette mode,”  Document of Joint Collaborative Team on Video Coding, JCTVC-Q1123, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, 17th Meeting: Valencia, ES, 27 March – 4 April 2014.

- Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei, “SCCE1: Test 3.5 - Block vector difference coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0133, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014.