| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0139r2 M33776 |
| --- | --- |

| Title: | Modulation channel information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fabrice Le Léannec, Sébastien Lasserre, Edouard François, David Touzé, Pierre Andrivon, Philippe Bordes, Yannick Olivier 975 avenue des champs blancs CS 17616-35576 Cesson-Sévigné Cedex, France | Tel: Email: | +33 (0)2 99 27 38 07 fabrice.leleannec@technicolor.com sebastien.lasserre@technicolor.com edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes a new SEI message aiming at using a modulation channel in combination with a limited bit-depth channel. The intent of this modulation channel is to give access to high / very-high bit-depth formats while re-using existing limited bit-depth HEVC implementation (typically of 8 or 10 bits).

# Introduction

Proposals JCTVC-N0142, JCTVC-O0090, JCTVC-P0162 and JCTVC-P0173 address the issue of high-bit depth coding using limited bit-depth implementations. In those approaches the signal is split into two additive parts (roughly MSBs and LSBs). In this proposal, the decomposition is based on the modulation of two multiplicative signals. This solution is an alternate solution that enables supporting even higher bit-depth signals, up to HDR representations.

The proposed approach splits the signal into a Modulation channel and a Low Bit-depth (LBD) channel, that can be processed by existing low bit-depth HEVC implementations. Its implementation is illustrated with two options: option1 enables to generate an LBD channel that can be rendered on ‘conventional’ rendering devices (conform to BT.709 specification); option2 generates an LBD signal that cannot be directly displayed as is. Option2 potentially offers higher coding performance.

Different possible options to convey the modulation channel in the bitstream are also proposed: Auxiliary picture, Frame packing arrangement, additional SEI message. The proposed ”Modulation channel information SEI message” embeds the parameters required for the regeneration of the output high bit-depth signal from the decoded modulation and LBD signals.

# Proposed solution

The principle of the proposed approach is that, even if a picture is globally of high bit-depth or dynamic range, it can be decomposed in local portions of low bit-depth. The solution consists in a pre-processing step prior to the encoding, splitting the input signal into two separate low bit-depth signals, one modulation signal (called modulation picture) which consists in a low-frequency version of the input signal, and one limited bit-depth signal (called LBD picture) corresponding to the remaining part once the low-frequency part has been extracted. At the decoder side, a symmetrical post-processing step is applied to the reconstructed modulation and LBD pictures to generate the output high bit-depth picture.

The modulation picture is accompanied by a new SEI message (Modulation Channel Information SEI) which embeds the parameters required to perform the recomposing process at the decoder, from the decoded modulation picture and the decoded LBD picture. The message also indicates the format of the output signal to be reconstructed.

## Decomposition process overview

The decomposition pre-processing step is summarized in Figure 1. The process applies to a linear 4:4:4 RGB or XYZ signal. A conversion step (dashed border box in Figure 1) is therefore possibly required prior to the decomposition process. The output of this decomposition process are a modulation signal and a LBD signal, both of low bit-depth. A detailed description of the process is provided in document JCTVC-R0267.

A low frequency signal extraction is first performed from the linear 4:4:4 signal to generate the modulation signal. Then the modulation signal is converted by a mapping process to an adapted low bit-depth representation. A downsampling can also be applied to reduce the resolution of this signal before its encoding.

The input signal demodulation consists in a division of the input linear 4:4:4 signal by the modulation signal previously extracted, to generate the LBD signal. A mapping process then applies to the LDR signal, to represent it into a low bit-depth format. Chroma format downsampling and colour transform are possibly applied depending on the input signal format, to adapt the signal format to the target output format. This demodulation process can be applied in closed-loop, that is, using the reconstructed modulation signal after decoding / upsampling / inverse modulation mapping.

The resulting LBD signal is coded using HEVC. For the coding of the modulation signal, as explained in section 2.3, three options are considered.

Figure 1: pre-processing sketch at encoder side.

## Recomposing process overview

The recomposing post-processing step is summarized in Figure 2. Its inputs are the decoded modulation signal and the decoded LBD signal. Its output is a reconstructed HBD signal, the format of which is indicated in the accompanying SEI message.

The modulation signal is possibly upsampled (to reach the full-resolution of the target output signal) then inverse mapped. The LBD signal is processed through a color resampling and color transform, then an inverse mapping step. The two resulting signals are multiplied and the resulting signal has to be converted to the output format. The implementation of these 2 last steps can be combined into one single step (dashed black border box in Figure 2) that depends on the target output format.

Figure 2: post-processing sketch at decoder side.

The process works in two possible modes, namely YCbCr mode and Lab mode, indicated in the Modulation channel information SEI message. In YCbCr mode, the decoded LBD picture is conform to BT.709 or BT.2020 recommendation. In Lab mode, the LBD picture cannot be correctly displayed on a rendering device conform to BT.709 or BT.2020 recommendation.

The implementation details of the recomposing process are given in Annex. In the following sub-sections, the principles are shortly described.

### Generation of the reconstructed modulation picture

As shown in the upper left part of Figure 2 (dashed red line box), the decoded modulation picture is converted after possible upsampling (if its resolution is lower than the output picture) to the reconstructed modulation picture by an inverse mapping process.

The inverse mapping process applied to each sample corresponds mathematically to an inverse log transfer function.

### Generation of the reconstructed LBD picture

This process corresponds to the lower left part of Figure 2 (dashed green line box). It converts the decoded LBD picture, after chroma resampling to the 4:4:4 chroma format, to the reconstructed LBD picture, using an inverse mapping process.

Transfer function description

The inverse mapping is based on the inverse of a transfer function defined as a log-like correction (noted GLog), defined as follows: GLog(x) = a0 * ln( x + b0 ) + c0. This transfer function enables to finely quantize the dark luminance ranges while avoiding too harsh high light saturations.

The parameters of the GLog curves are determined such that 0 and 1 are invariant (GLog(0) = 0 and GLog(1) = 1) and the derivative is continuous in 1 (GLog’(1) = γ). This leads to the parameters shown in the Table 1.

Table 1: GLog parameters for different γ values.

|  | a0 | b0 | c0 |
| --- | --- | --- | --- |
| 1/2.0 | 0.6275 | 0.2550 | 0.8575 |
| 1/2.4 | 0.4742 | 0.1382 | 0.9386 |
| 1/2.8 | 0.3861 | 0.0811 | 0.9699 |

In Figure 3, the gamma curve with γ=1/2.4 and several GLog curves are plotted for 1/2.0, 1/2.4 and 1./2.8. It can be observed that high lights are lowered much more aggressively with a GLog curve than with a simple gamma curve.

Figure 3: GLog and gamma curves.

Inverse mapping in YCbCr mode

In YCbCr mode, the inverse mapping process is made of the following steps:

- Color space conversion from YCbCr to RGB

- Inverse transfer function: it applies to each R, G and B sample and mathematically corresponds to the following operation (the control parameters Scal, LumaFact, a0, b0 and c0, are signaled in the Modulation channel information SEI message):

stf = LumaFact * GLog-1( s * Scal )

- Color space conversion from RGB to XYZ

Inverse mapping in Lab mode

In Lab mode, the three decoded components of the decode LBD picture are represented in a locally perceptual Lab-based color space. Let’s note (L,a,b) the color samples at a given pixel. The inverse mapping process applied to (L,a,b) is made of the following steps:

- Rescaling by a multiplicative factor

L1 = L / scal         a1 = a  / scal       b1 = b  / scal

- Inverse transformation from the locally perceptual color space

Ĉ² = a1² + b1²   	C =  ( exp( k * Ĉ ) - 1 ) / k        a2 = a1 * C / Ĉ           b2 = b1 * C / Ĉ

- Inverse transformation from the Lab-like space

Y’ = L1 / 116        X’ = a2 / 500 + Y’            Z’ = b2 / 500 + Y’

- Inverse transfer function: it applies to each X, Y and Z sample and mathematically corresponds to the following operation:

stf = GLog-1( s )

### Generation of the output HBD picture

The final recombining process mathematically corresponds to the following operation, with (Xtf ,Ytf , Ztf) being the sample of the reconstructed LBD picture, and modrec the corresponding sample of the reconstructed modulation picture:

In YCbCr mode:

XHBD = Xtf * modrec	YHBD = Ytf * modrec	ZHBD = Ztf * modrec

In the Lab mode, a scaling factor, taking into account the D65 illuminant, applies as follows:

XHBD = Xtf * 0.9505 * modrec	YHBD = Ytf * modrec	ZHBD = Ztf * 1.0890 * modrec

The signal has then to be converted to the output format indicated in the Modulation channel information SEI message.

## Containers for the modulation channel

Three different options for conveying the modulation signal are considered: auxiliary picture, frame packing, additional SEI message.

- Auxiliary picture: the concept is actually close to the Alpha channel concept, which also involves a multiplication of a signal by the alpha channel picture. The proposed approach can be considered as a generalization of this concept. Thanks to SHVC framework, the synchronization between the primary coded picture and the auxiliary picture is easy to handle.

- Frame packing: another option is to use the frame packing, which simplifies the implementation of the synchronization between the modulation and LBD signals. It involves either reducing the resolution of the main coded picture in order to embed in a full-resolution picture the modulation and the LBD pictures, or the need of coding/decoding devices supporting higher resolutions and therefore higher operational points. Frame packing is already practically used for 3D/multiview services.

- Additional SEI message: this third option would consist in embedding the modulation channel into an SEI message (one per picture). This is only valuable if the size of the modulation channel is small, that is, resulting from a significant downsampling process of the modulation signal.

The high level syntax/semantics changes for the 2 first options are more detailed in the attached documents.

## Description of the Modulation Channel Information SEI message

This section provides the main elements of the proposed Modulation channel information SEI message. The message embeds the syntax elements required for the recomposing process of the output picture from the decoded modulation and LBD pictures. The Figure 4 indicates in red font the usage, for the recomposing process, of the Modulation channel information SEI message data.

Figure 4: Modulation channel information SEI message in the recomposing process.

In the following of this section, we shortly describe the syntax. The message is defined in Table 2. It is made of three parts. The first part describes the format of the output signal after the recomposing process. The second part contains parameters related to the modulation picture format and processing. The third part contains parameters related to the LBD picture format and processing. In particular, parameters required for the inverse mapping process are provided. These different parameters are described in details in attached documents JCTVC_R0139_SyntaxSemanticsAuxPict.doc and JCTVC_R0139_SyntaxSemanticsFramePack.doc.

Table 2: syntax of the Modulation channel information SEI message.

| modulation_channel_info( payloadSize ) { | Descriptor |  |
| --- | --- | --- |
| mod_channel_cancel_flag | u(1) | related to the output signal format |
| if ( !mod_channel_cancel_flag ) { |  |  |
| mod_chroma_format_idc | ue(v) |  |
| mod_sample_format_idc | ue(v) |  |
| if( mod_sample_format_idc = = 1 ) { |  |  |
| mod_bit_depth_luma_minus8 | u(3) |  |
| mod_bit_depth_chroma_minus8 | u(3) |  |
| } |  |  |
| mod_video_full_range_flag | u(1) |  |
| mod_colour_primaries | u(8) |  |
| mod_transfer_characteristics | u(8) |  |
| mod_matrix_coeffs | u(8) |  |
| mod_modpic_bit_depth_minus8 | u(3) | related to the modulation picture processing |
| mod_lbd_bit_depth_luma_minus8 | u(3) | related to the LBD picture processing |
| mod_lbd_bit_depth_chroma_minus8 | u(3) |  |
| mod_lbd_format | u(2) |  |
| if( ( mod_lbd_format = = 1 ) ¦ ¦ ( mod_lbd_format = = 2 ) ) { |  |  |
| lbd_glog_a0 | u(15) |  |
| lbd_glog_b0 | u(14) |  |
| lbd_glog_c0 | u(10) |  |
| lbd_scaling_factor | u(10) |  |
| lbd_luminance_balance_factor | u(10) |  |
| } |  |  |
| } |  |  |
| mod_channel_persistence_flag | u(1) |  |
| } |  |  |

# Experiments

To test the proposed approach, several 16 bit XYZ sequences have been created from the openEXR sequences available on the hannover ftp site (testsequences/half-float-sequences). The openEXR RGB sequences are converted to the XYZ color space, normalized with a peak luminance of 10000 nits, mapped using a gamma function of  = 0.45 (~1/2.2), then quantized to 16 bits.

These 16 bit XYZ sequences are then processed as follows : demodulation, encoding of the modulation and LBD videos, decoding of the modulation and LBD bitstreams, modulation. The performance is evaluated between the input XYZ content and the reconstructed XYZ content regenerated from the modulation process.

Figure 5 : test coding chain.

Simulations have been performed for the Auxiliary picture and Frame packing implementations. The Auxiliary picture implementation is based on the SHM7.0. The Frame packing implementation is based on the HM13.0_RExt6.0rc1. Simulations are made in the Lab mode.

The corresponding results are provided in the attached file JCTVC_R0139_simuResults.xls.

# Conclusion

This contribution proposes a new SEI message aiming at using a modulation channel in combination with a limited bit-depth channel. The intent of this modulation channel is to give access to high / very-high bit-depth formats while re-using existing limited bit-depth HEVC implementation (typically of 8 or 10 bits).

One important target property of the proposed approach is to potentially support a limited bit-depth channel that can be directly displayed with ‘conventional’ rendering devices. This is one important difference with the approach proposed in JCTVC-N0142, JCTVC-O0090, JCTVC-P0162 and JCTVC-P0173.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: implementation details of the recomposing process

The recomposing process is summarized in the Figure 6.

Figure 6: recomposing process at decoder side.

The process takes as input the decoded modulation picture and the decoded LBD picture. These pictures are converted to a reconstructed modulation picture and a reconstructed LBD picture. Then the reconstructed modulation picture and a reconstructed LBD picture are combined, and a final conversion process applies to generate the output HBD picture of format specified in the Modulation channel information SEI message.

The process works in two possible modes, namely YCbCr mode and Lab mode, indicated by the syntax element mod_lbd_format of the Modulation channel information SEI message. In YCbCr mode (mod_lbd_format=1 or 2), the decoded LBD picture is conform to BT.709 or BT.2020 recommendation. In Lab mode (mod_lbd_format=0), the LBD picture cannot be correctly displayed on a rendering device conform to BT.709 or BT.2020 recommendation.

Notations:

- Width and Height are the dimensions of the Y component of the decoded LBD picture

- BDmod is the bit-depth of the decoded modulation picture

- BDLBD is the bit-depth of the decoded LBD picture

## Generation of the reconstructed modulation picture

The decoded modulation picture is processed before its recombining with the LBD picture, resulting in the reconstructed modulation picture.

The following steps apply:

- Upsampling. The decoded modulation picture is first upsampled (if necessary) to the resolution of the output signal (specified in the Modulation Channel Information SEI message). The resampling process used in SHVC for inter-layer prediction can be used.

- Inverse mapping. Each sample m of the picture is inverse mapped to m’ by the following successive steps:

m’ = ( m * ( maxmod – minmod ) + ( minmod * (1 << BDmod ) ) + (1 << (BDmod  – 1 ) ) ) >> BDmod

m’ = pow2_int_s9[ ( ( m’ + 4096 ) + 2 ) >> 3 ] >>1

m’ = ( m’ + 128 ) >> 8

where

  - pow2_int_s9[ i ] = Round( 16384.* power( 2. , ( 1. / 512. ) * i ) )    for i = 0 ... 511

  - minmod and maxmod are defined as follows:

| mode | YCbCr | Lab |
| --- | --- | --- |
| minmod | 4096 | -4096 |
| maxmod | 27213 | 44916 |

the sample m’ is finally clipped as follows:

  - In YCbCr mode:	m’ = Clip3( 128 , 6400 , m’ )

  - In Lab mode:		m’ = Max( 32 , m’ )

## Generation of the reconstructed LBD picture

The decoded LBD picture is processed before its recombining with the modulation picture, resulting in a reconstructed LBD picture. The process also uses the reconstructed modulation picture as input.

YCbCr mode

The following steps apply:

- Chroma resampling. Applied (if necessary) to convert the decoded LBD picture to 4:4:4 chroma format.

- Conversion to RGB. Each (y,cb,cr) samples of the 4:4:4 LBD picture is converted into (r,g,b) as follows:

cb = cb – ( 1 << ( BDLBD – 1 ) )

cr = cr – ( 1 << ( BDLBD – 1 ) )

r = ( 4096 * y + 5743 * cr + 512 ) >> 10

g = ( 4096 * y − 1410 * cb − 2925 * cr + 512 ) >> 10

b = ( 4096 * y + 7258 * cb + 512 ) >> 10

- Inverse mapping. Each sample s of the r, g or b samples is inverse mapped to s’ by the following successive steps:

s' = ( ( s * Scaling + 128 ) >> 8 ) – GLogc0

s’ = ( s’ * GLoga0 + 8192 ) >> 14

s’ = ( ( ( pow2_int_s8[ s’ + 1024 ] + 8 ) >> 4 ) – GLogb0 + 16 ) >> 5

s’ = ( s’ * LumaFact + 512 ) >> 10

where

  - The parameters GLoga0, GLogb0, GLogc0, Scaling, LumaFact are transmitted in the Modulation Channel Information SEI message

  - pow2_int_s8[ i ] = Round( 16384. * power( 2. , ( 1. / 256. ) * i ) )   for i = 0 ... 255

Note:	this process corresponds to a transfer function that can be simply replaced by a 1D LUT 
(s’ = Yuv_Mapp[ s ]).

- Conversion to XYZ color space. The modified r’, g’ and b’ samples of the R, G and B arrays are finally converted to samples x, y and z in the XYZ color space as follows:

x = ( 422 * r’ + 366 * g’ + 185 * b’ + 128 ) >> 10

y = ( 218 * r’ + 732 * g’ +  74 * b’ + 128 ) >> 10

z = (  20 * r’ + 122 * g’ + 973 * b’ + 128 ) >> 10

Lab mode

In Lab mode, the process uses the reconstructed modulation picture and the decoded LBD picture as inputs.

The following steps apply:

- Chroma resampling. Applied (if necessary) to convert the decoded LBD picture to 4:4:4 chroma format.

- Inverse mapping. The inverse mapping process converts the signal from a local perceptual space (that takes into account the local luminance) to a global Lab perceptual space. Then the signal is reconverted from global Lab perceptual space to XYZ color space.

The process works as follows. For each sample position (x=0…Width-1,y=0…Height-1), the following applies with m the sample of the reconstructed modulation picture, (L, a, b) the samples of the 3 components of the decoded LBD picture:

Rescaling to local perceptual color space

- A scaling factor Scaling is derived from the local modulation value m as follows:

idxmod = ( ( m – 32 ) + 1 ) >> 1
idx = Clip3( 0 , 298 , ( idxmod >> 6 ) )
lambda = idxmod – ( idx  << 6 )

iScal = ( invScalmod[ idx ] * ( 64 – lambda ) +  invScalmod[ idx + 1 ] * lambda + 32 ) >> 6

Scaling  = ( preScal * iScal + 8 ) >> 4

where

  - preScal  = 1053 >> ( BDLBD − 8 )

  - invScalmod is a 1D LUT given in the Table 3.

- The samples (L, a, b) are converted to (L’, a’, b’ ) as follows:

L’ = ( L * Scaling + 64 ) >> 7
a’ = ( ( a – ( 1 << ( BDLBD – 1 ) ) * Scaling + 128 ) >> 8
b’ = ( ( b – ( 1 << ( BDLBD – 1 ) ) * Scaling + 128 ) >> 8

Conversion to global perceptual color space

- The samples a’ and b’ are then rescaled as follows:

Cmod = sqrt_int( a’ * a’ + b’ * b’ )

k1C = ( k1 * Cmod + 4096 ) >> 13

k1C_round = ( k1C << 7 )

idx  = Min( 639, ( k1C_round >> 9 ) )

lambda = k1C_round – ( idx  << 9 )

Cconv = ( LUTLab[ idx ] * ( 512 – lambda ) + LUTLab[ idx + 1 ] * lambda + 256 ) >> 9

Cconv = Cconv + 1024 + k1C

a' = ( a’ * Cconv + 256 ) >> 9

b' = ( b’ * Cconv + 256 ) >> 9

where

  - sqrt_int( n ) is an integer-based implementation of the square root operation. A proposed pseudo-code implementation is described below.

  - k1  = 492

  - LUTLab is a 1D LUT given in the Table 4.

Conversion to XYZ color space

- The samples (x, y, z) are derived from (L’, a’, b’ ) as follows:

y = ( 1130 * L’ + 4096 ) >> 13 )

x = ( 262 * a’ + 4096 ) >> 13 ) + L’

z = ( –655 * b’ + 4096 ) >> 13 ) + L’

- Then for s being each of x, y or z, the inverse mapping is achieved as follows:

s = ( ( (s − GLogc0 ) * GLoga0 + 16384 ) >> 15 ) + 2048

s = ( pow2_int_s9[ Max( 0, s ) ] + 8 ) >> 4

s = ( s − GLogb0 + 8 ) >> 3

where

  - GLoga0 = 13187

  - GLogb0 = 1970

  - GLogc0 = 3888

  - pow2_int_s9[ i ] = Round( 16384. * power( 2. , ( 1. / 512. ) * i ) )   for i = 0 ... 511

- The samples x and z are finally modified as follows:

x = x + ( x * EpsilonD65X + 2048 ) >> 12

z = z + ( z * EpsilonD65Z + 2048 ) >> 12

where

  - EpsilonD65X = −203

  - EpsilonD65Z = 365

## Recombination of the reconstructed modulation and LBD pictures

The reconstructed modulation picture and the reconstructed LBD picture are combined to give the output HBD picture that will have to be finally converted to the output format. This last step depends on the output format and is therefore not addressed in this description.

For each sample position (sx=0…Width-1,sy=0…Height-1), m being the sample of the reconstructed modulation picture, and for s being each of the x, y and z samples of the 3 components of the reconstructed LBD picture, s is modified as follows:

- In YCbCr mode:	s =  Clip3( MinHBD, MaxHBD, ( m * s + 16 )  >> 5 ) / ( 1 << 10 )

- Lab mode:		s =  Clip3( 128 * MinHBD, 128 * MaxHBD, ( m * s ) ) / ( 1 << 17 )

where

  - MinHBD = 5

  - MaxHBD = 10240000

Table 3: Derivation of variable InvScal from input Idx

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invScalmod | 1216 | 1053 | 994 | 956 | 929 | 908 | 890 | 875 | 862 | 851 | 840 | 831 | 822 | 814 | 807 |
| Idx | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| invScalmod | 800 | 794 | 788 | 782 | 777 | 771 | 767 | 762 | 757 | 753 | 749 | 745 | 741 | 738 | 734 |
| Idx | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| invScalmod | 731 | 728 | 724 | 721 | 718 | 715 | 713 | 710 | 707 | 705 | 702 | 700 | 697 | 695 | 692 |
| Idx | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| invScalmod | 690 | 688 | 686 | 684 | 682 | 680 | 678 | 676 | 674 | 672 | 670 | 668 | 666 | 665 | 663 |
| Idx | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| invScalmod | 661 | 660 | 658 | 656 | 655 | 653 | 652 | 650 | 649 | 647 | 646 | 644 | 643 | 642 | 640 |
| Idx | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| invScalmod | 639 | 637 | 636 | 635 | 634 | 632 | 631 | 630 | 629 | 627 | 626 | 625 | 624 | 623 | 622 |
| Idx | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| invScalmod | 620 | 619 | 618 | 617 | 616 | 615 | 614 | 613 | 612 | 611 | 610 | 609 | 608 | 607 | 606 |
| Idx | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| invScalmod | 605 | 604 | 603 | 602 | 601 | 600 | 599 | 598 | 598 | 597 | 596 | 595 | 594 | 593 | 592 |
| Idx | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 |
| invScalmod | 592 | 591 | 590 | 589 | 588 | 587 | 587 | 586 | 585 | 584 | 583 | 583 | 582 | 581 | 580 |
| Idx | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| invScalmod | 580 | 579 | 578 | 577 | 577 | 576 | 575 | 575 | 574 | 573 | 572 | 572 | 571 | 570 | 570 |
| Idx | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 |
| invScalmod | 569 | 568 | 568 | 567 | 566 | 566 | 565 | 564 | 564 | 563 | 563 | 562 | 561 | 561 | 560 |
| Idx | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 |
| invScalmod | 559 | 559 | 558 | 558 | 557 | 556 | 556 | 555 | 555 | 554 | 554 | 553 | 552 | 552 | 551 |
| Idx | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 |
| invScalmod | 551 | 550 | 550 | 549 | 549 | 548 | 547 | 547 | 546 | 546 | 545 | 545 | 544 | 544 | 543 |
| Idx | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 | 208 | 209 |
| invScalmod | 543 | 542 | 542 | 541 | 541 | 540 | 540 | 539 | 539 | 538 | 538 | 537 | 537 | 536 | 536 |
| Idx | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 |
| invScalmod | 535 | 535 | 534 | 534 | 533 | 533 | 532 | 532 | 531 | 531 | 531 | 530 | 530 | 529 | 529 |
| Idx | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| invScalmod | 528 | 528 | 527 | 527 | 527 | 526 | 526 | 525 | 525 | 524 | 524 | 524 | 523 | 523 | 522 |
| Idx | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 |
| invScalmod | 522 | 521 | 521 | 521 | 520 | 520 | 519 | 519 | 519 | 518 | 518 | 517 | 517 | 517 | 516 |
| Idx | 255 | 256 | 257 | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 |
| invScalmod | 516 | 515 | 515 | 515 | 514 | 514 | 513 | 513 | 513 | 512 | 512 | 512 | 512 | 512 | 512 |
| Idx | 270 | 271 | 272 | 273 | 274 | 275 | 276 | 277 | 278 | 279 | 280 | 281 | 282 | 283 | 284 |
| invScalmod | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 |
| Idx | 285 | 286 | 287 | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 | 298 | 299 |
| invScalmod | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 |

Table 4: Derivation of variable LUTLab from input Idx

| Idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LUTLab | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 |
| Idx | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| LUTLab | 7 | 7 | 8 | 9 | 9 | 10 | 11 | 11 | 12 | 13 | 14 | 15 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 28 |
| Idx | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| LUTLab | 29 | 30 | 31 | 33 | 34 | 35 | 37 | 38 | 39 | 41 | 42 | 44 | 45 | 47 | 49 | 50 | 52 | 54 | 55 | 57 | 59 | 61 | 63 | 64 | 66 |
| Idx | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 |
| LUTLab | 68 | 70 | 72 | 74 | 76 | 79 | 81 | 83 | 85 | 87 | 90 | 92 | 94 | 97 | 99 | 102 | 104 | 107 | 109 | 112 | 114 | 117 | 120 | 123 | 125 |
| Idx | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 |
| LUTLab | 128 | 131 | 134 | 137 | 140 | 143 | 146 | 149 | 152 | 155 | 159 | 162 | 165 | 168 | 172 | 175 | 179 | 182 | 186 | 189 | 193 | 197 | 200 | 204 | 208 |
| Idx | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| LUTLab | 212 | 216 | 220 | 224 | 228 | 232 | 236 | 240 | 244 | 248 | 253 | 257 | 261 | 266 | 270 | 275 | 279 | 284 | 289 | 293 | 298 | 303 | 308 | 313 | 318 |
| Idx | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 |
| LUTLab | 323 | 328 | 333 | 338 | 344 | 349 | 354 | 360 | 365 | 371 | 376 | 382 | 387 | 393 | 399 | 405 | 411 | 417 | 423 | 429 | 435 | 441 | 447 | 453 | 460 |
| Idx | 175 | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 |
| LUTLab | 466 | 473 | 479 | 486 | 493 | 499 | 506 | 513 | 520 | 527 | 534 | 541 | 548 | 555 | 563 | 570 | 577 | 585 | 592 | 600 | 608 | 615 | 623 | 631 | 639 |
| Idx | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 |
| LUTLab | 647 | 655 | 663 | 672 | 680 | 688 | 697 | 705 | 714 | 723 | 731 | 740 | 749 | 758 | 767 | 776 | 786 | 795 | 804 | 814 | 823 | 833 | 843 | 852 | 862 |
| Idx | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 |  |  |  |  |  |  |  |  |  |  |  |  |
| LUTLab | 872 | 882 | 892 | 902 | 913 | 923 | 933 | 944 | 954 | 965 | 976 | 987 | 998 |  |  |  |  |  |  |  |  |  |  |  |  |

| Idx | Idx | 238 | 239 | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 | 256 | 257 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LUTLab | Lab | 1009 | 1020 | 1031 | 1042 | 1054 | 1065 | 1077 | 1089 | 1100 | 1112 | 1124 | 1136 | 1148 | 1161 | 1173 | 1185 | 1198 | 1210 | 1223 | 1236 |
| Idx | Idx | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 | 270 | 271 | 272 | 273 | 274 | 275 | 276 | 277 |
| LUTLab | Lab | 1249 | 1262 | 1275 | 1288 | 1302 | 1315 | 1329 | 1342 | 1356 | 1370 | 1384 | 1398 | 1412 | 1426 | 1441 | 1455 | 1470 | 1485 | 1500 | 1515 |
| Idx | Idx | 278 | 279 | 280 | 281 | 282 | 283 | 284 | 285 | 286 | 287 | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 |
| LUTLab | Lab | 1530 | 1545 | 1560 | 1576 | 1591 | 1607 | 1623 | 1638 | 1654 | 1671 | 1687 | 1703 | 1720 | 1736 | 1753 | 1770 | 1787 | 1804 | 1821 | 1839 |
| Idx | Idx | 298 | 299 | 300 | 301 | 302 | 303 | 304 | 305 | 306 | 307 | 308 | 309 | 310 | 311 | 312 | 313 | 314 | 315 | 316 | 317 |
| LUTLab | Lab | 1856 | 1874 | 1892 | 1910 | 1928 | 1946 | 1964 | 1983 | 2001 | 2020 | 2039 | 2058 | 2077 | 2096 | 2116 | 2135 | 2155 | 2175 | 2195 | 2215 |
| Idx | Idx | 318 | 319 | 320 | 321 | 322 | 323 | 324 | 325 | 326 | 327 | 328 | 329 | 330 | 331 | 332 | 333 | 334 | 335 | 336 | 337 |
| LUTLab | Lab | 2235 | 2256 | 2276 | 2297 | 2318 | 2339 | 2360 | 2382 | 2403 | 2425 | 2447 | 2469 | 2491 | 2513 | 2536 | 2558 | 2581 | 2604 | 2627 | 2650 |
| Idx | Idx | 338 | 339 | 340 | 341 | 342 | 343 | 344 | 345 | 346 | 347 | 348 | 349 | 350 | 351 | 352 | 353 | 354 | 355 | 356 | 357 |
| LUTLab | Lab | 2674 | 2697 | 2721 | 2745 | 2769 | 2794 | 2818 | 2843 | 2868 | 2893 | 2918 | 2943 | 2969 | 2994 | 3020 | 3046 | 3073 | 3099 | 3126 | 3153 |
| Idx | Idx | 358 | 359 | 360 | 361 | 362 | 363 | 364 | 365 | 366 | 367 | 368 | 369 | 370 | 371 | 372 | 373 | 374 | 375 | 376 | 377 |
| LUTLab | Lab | 3180 | 3207 | 3235 | 3262 | 3290 | 3318 | 3346 | 3375 | 3403 | 3432 | 3461 | 3490 | 3520 | 3549 | 3579 | 3609 | 3640 | 3670 | 3701 | 3732 |
| Idx | Idx | 378 | 379 | 380 | 381 | 382 | 383 | 384 | 385 | 386 | 387 | 388 | 389 | 390 | 391 | 392 | 393 | 394 | 395 | 396 | 397 |
| LUTLab | Lab | 3763 | 3794 | 3826 | 3858 | 3890 | 3922 | 3955 | 3987 | 4020 | 4053 | 4087 | 4120 | 4154 | 4188 | 4223 | 4257 | 4292 | 4327 | 4363 | 4398 |
| Idx | Idx | 398 | 399 | 400 | 401 | 402 | 403 | 404 | 405 | 406 | 407 | 408 | 409 | 410 | 411 | 412 | 413 | 414 | 415 | 416 | 417 |
| LUTLab | Lab | 4434 | 4470 | 4506 | 4543 | 4580 | 4617 | 4654 | 4692 | 4730 | 4768 | 4806 | 4845 | 4884 | 4923 | 4962 | 5002 | 5042 | 5082 | 5123 | 5164 |
| Idx | Idx | 418 | 419 | 420 | 421 | 422 | 423 | 424 | 425 | 426 | 427 | 428 | 429 | 430 | 431 | 432 | 433 | 434 | 435 | 436 | 437 |
| LUTLab | Lab | 5205 | 5246 | 5288 | 5330 | 5372 | 5415 | 5458 | 5501 | 5544 | 5588 | 5632 | 5677 | 5721 | 5766 | 5811 | 5857 | 5903 | 5949 | 5996 | 6043 |
| Idx | Idx | 438 | 439 | 440 | 441 | 442 | 443 | 444 | 445 | 446 | 447 | 448 | 449 | 450 | 451 | 452 | 453 | 454 | 455 | 456 | 457 |
| LUTLab | Lab | 6090 | 6137 | 6185 | 6233 | 6282 | 6331 | 6380 | 6429 | 6479 | 6530 | 6580 | 6631 | 6682 | 6734 | 6786 | 6838 | 6891 | 6944 | 6997 | 7051 |
| Idx | Idx | 458 | 459 | 460 | 461 | 462 | 463 | 464 | 465 | 466 | 467 | 468 | 469 | 470 | 471 | 472 | 473 | 474 | 475 | 476 | 477 |
| LUTLab | Lab | 7105 | 7159 | 7214 | 7270 | 7325 | 7381 | 7438 | 7494 | 7552 | 7609 | 7667 | 7725 | 7784 | 7843 | 7903 | 7963 | 8023 | 8084 | 8145 | 8207 |
| Idx | Idx | 478 | 479 | 480 | 481 | 482 | 483 | 484 | 485 | 486 | 487 | 488 | 489 | 490 | 491 | 492 | 493 | 494 | 495 | 496 | 497 |
| LUTLab | Lab | 8269 | 8331 | 8394 | 8457 | 8521 | 8585 | 8650 | 8715 | 8780 | 8846 | 8913 | 8980 | 9047 | 9115 | 9183 | 9252 | 9321 | 9390 | 9460 | 9531 |
| Idx | Idx | 498 | 499 | 500 | 501 | 502 | 503 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LUTLab | Lab | 9602 | 9674 | 9746 | 9818 | 9891 | 9965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Idx | 504 | 505 | 506 | 507 | 508 | 509 | 510 | 511 | 512 | 513 | 514 | 515 | 516 | 517 | 518 | 519 | 520 | 521 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LUTLab | 10039 | 10113 | 10188 | 10264 | 10340 | 10416 | 10493 | 10571 | 10649 | 10728 | 10807 | 10887 | 10967 | 11048 | 11129 | 11211 | 11294 | 11377 |
| Idx | 522 | 523 | 524 | 525 | 526 | 527 | 528 | 529 | 530 | 531 | 532 | 533 | 534 | 535 | 536 | 537 | 538 | 539 |
| LUTLab | 11460 | 11544 | 11629 | 11715 | 11800 | 11887 | 11974 | 12062 | 12150 | 12239 | 12328 | 12418 | 12509 | 12600 | 12692 | 12785 | 12878 | 12972 |
| Idx | 540 | 541 | 542 | 543 | 544 | 545 | 546 | 547 | 548 | 549 | 550 | 551 | 552 | 553 | 554 | 555 | 556 | 557 |
| LUTLab | 13066 | 13161 | 13257 | 13353 | 13450 | 13548 | 13646 | 13745 | 13845 | 13945 | 14046 | 14148 | 14251 | 14354 | 14458 | 14562 | 14667 | 14773 |
| Idx | 558 | 559 | 560 | 561 | 562 | 563 | 564 | 565 | 566 | 567 | 568 | 569 | 570 | 571 | 572 | 573 | 574 | 575 |
| LUTLab | 14880 | 14987 | 15095 | 15204 | 15314 | 15424 | 15535 | 15647 | 15760 | 15873 | 15987 | 16102 | 16218 | 16334 | 16451 | 16569 | 16688 | 16808 |
| Idx | 576 | 577 | 578 | 579 | 580 | 581 | 582 | 583 | 584 | 585 | 586 | 587 | 588 | 589 | 590 | 591 | 592 | 593 |
| LUTLab | 16928 | 17050 | 17172 | 17295 | 17418 | 17543 | 17668 | 17795 | 17922 | 18050 | 18179 | 18308 | 18439 | 18571 | 18703 | 18836 | 18971 | 19106 |
| Idx | 594 | 595 | 596 | 597 | 598 | 599 | 600 | 601 | 602 | 603 | 604 | 605 | 606 | 607 | 608 | 609 | 610 | 611 |
| LUTLab | 19242 | 19379 | 19517 | 19655 | 19795 | 19936 | 20078 | 20220 | 20364 | 20508 | 20654 | 20800 | 20948 | 21096 | 21246 | 21397 | 21548 | 21701 |
| Idx | 612 | 613 | 614 | 615 | 616 | 617 | 618 | 619 | 620 | 621 | 622 | 623 | 624 | 625 | 626 | 627 | 628 | 629 |
| LUTLab | 21854 | 22009 | 22165 | 22321 | 22479 | 22638 | 22798 | 22959 | 23121 | 23285 | 23449 | 23615 | 23781 | 23949 | 24118 | 24288 | 24459 | 24631 |
| Idx | 630 | 631 | 632 | 633 | 634 | 635 | 636 | 637 | 638 | 639 | 640 |  |  |  |  |  |  |  |
| LUTLab | 24805 | 24980 | 25156 | 25333 | 25511 | 25690 | 25871 | 26053 | 26236 | 26421 | 26606 |  |  |  |  |  |  |  |

### Pseudo-code of  sqrt_int  function

int sqrt_int(int num)

{

int res = 0;

int bit = 1 << 30; // The second-to-top bit is set

// "bit" starts at the highest power of four <= the argument.

while (bit > num)

bit >>= 2;

while (bit != 0) {

if (num >= res + bit) {

num -= res + bit;

res = (res >> 1) + bit;

}

else

res >>= 1;

bit >>= 2;

}

return res;

}