# Proposed syntax and semantics modifications for modulation picture coded using frame packing mechanism

This document provides the proposed syntax and semantics changes to support the modulation picture coded with the frame packing mechanism. The mechanism is claimed to be close to the Frame Packing Arrangement.

F.14.1.X Modulation channel information SEI message syntax

| modulation_channel_info( payloadSize ) { | Descriptor |  |
| --- | --- | --- |
| mod_channel_cancel_flag | u(1) | related to the output signal format (modulated picture) |
| if ( !mod_channel_cancel_flag ) { |  |  |
| mod_chroma_format_idc | ue(v) |  |
| mod_sample_format_idc | ue(v) |  |
| if( mod_sample_format_idc = = 1 ) { |  |  |
| mod_bit_depth_luma_minus8 | u(3) |  |
| mod_bit_depth_chroma_minus8 | u(3) |  |
| } |  |  |
| mod_video_full_range_flag | u(1) |  |
| mod_colour_primaries | u(8) |  |
| mod_transfer_characteristics | u(8) |  |
| mod_matrix_coeffs | u(8) |  |
| mod_lbd_pic_height | ue(v) | related to the modulation picture frame-packing |
| mod_fp_different_views_size_flag | u(1) |  |
| if ( mod_fp_different_views_size_flag ) { |  |  |
| mod_lbd_pic_width | ue(v) |  |
| mod_pic_width | ? |  |
| mod_pic_height | ue(v) |  |
| mod_fp_num_part_minus1 | u(4) |  |
| mod_pic_width_part_offset_x | ue(v) |  |
| } |  |  |
| mod_lbd_format | u(2) | related to the LBD picture processing |
| if( (mod_lbd_format = = 1) ¦ ¦ (mod_lbd_format = = 2) ) { |  |  |
| lbd_glog_a0 | u(15) |  |
| lbd_gslog_b0 | u(14) |  |
| lbd_gslog_c0 | u(10) |  |
| lbd_scaling_factor | u(10) |  |
| lbd_luminance_balance_factor | u(10) |  |
| } |  |  |
| mod_channel_persistence_flag |  |  |
| } |  |  |

### F.14.1.X Modulation channel information SEI message semantics

The modulation channel SEI message provides information for the post-processing recomposing process of the primary picture and the X to generate a reconstructed modulated picture. The recomposing process takes as inputs the decoded modulation picture, and the decoded limited bit-depth (LBD) picture. The decoded modulation picture is processed by an upsampling process (if its width or height are smaller than the width and height of the LBD picture luma array, respectively), followed by a mapping process to generate the mapped modulation picture. The decoded LBD picture is processed by a color resampling and transform process followed by a mapping process to generate the mapped LBD picture. The mapped modulation picture and the mapped LBD picture are then combined to generate the reconstructed modulated picture, which has to be converted finally to the output format.

mod_channel_cancel_flag equal to 1 indicates that the modulation channel information SEI message cancels the persistence of any previous modulation channel information SEI message in output order that applies to the current layer. mod_channel_cancel_flag equal to 0 indicates that modulation channel information follows.

–	A new CVS begins.

–	The bitstream ends.

–	A picture picB in an access unit containing a modulation channel information SEI message is output having PicOrderCnt( picB ) greater than PicOrderCnt( picA ) and mod_channel_cancel_flag in the modulation channel information SEI message is equal to 1, where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

mod_chroma_format_idc specifies the chroma sampling relative to the luma sampling of the reconstructed modulated pictures as specified in clause 6.2. The value of mod_chroma_format_idc shall be in the range of 0 to 3, inclusive.

mod_sample_format_idc specifies the representation format of the samples of the luma and chroma samples of the reconstructed modulated pictures. mod_sample_format_idc equal to 1 indicates that the samples are represented with integer format. The values of mod_sample_format_idc  different from 1 are reserved for future use by ITU-T | ISO/IEC.  Decoders shall ignore all modulation channel information SEI messages that contain a value of mod_sample_format_idc different from 1.

mod_bit_depth_luma_minus8 plus 8 specifies the bit depth used for the luma component of the reconstructed modulated pictures. mod_bit_depth_luma_minus8 shall be in the range of 0 to 8, inclusive.

mod_bit_depth_chroma_minus8 plus 8 specifies the bit depth used for the chroma components of the reconstructed modulated pictures. mod_bit_depth_chroma_minus8 shall be in the range 0 to 8 inclusive.

mod_video_full_range_flag has the same semantics as specified in clause E.2.1 for the video_full_range_flag syntax element, except that mod_video_full_range_flag specifies the colour space of the reconstructed modulated picture, rather than the colour space used for the CVS.

mod_colour_primaries has the same semantics as specified in clause E.2.1 for the colour_primaries syntax element, except that mod_colour_primaries specifies the colour space of the reconstructed modulated picture, rather than the colour space used for the CVS.

mod_transfer_characteristics has the same semantics as specified in clause E.2.1 for the transfer_characteristics syntax element, except that mod_transfer_characteristics specifies the opto-electronic transfer characteristics of the reconstructed modulated picture, rather than the opto-electronic transfer characteristics used for the CVS.

mod_matrix_coeffs has the same semantics as specified in clause E.2.1 for the matrix_coeffs syntax element, except that mod_matrix_coeffs specifies the colour space of the reconstructed modulated picture, rather than the colour space used for the CVS.

mod_fp_different_views_size_flag equal to 1 specifies the decoded LBD picture and the decoded modulation picture have different sizes and a modulation picture resampling not described in this Specification should be invoked. mod_fp_different_views_size_flag equal to 0 specifies the decoded LDR picture and the decoded modulation picture have same size.

mod_lbd_pic_height specifies the height of the LBD picture luma array in units of luma samples.

mod_lbd_pic_width specifies the width of the LBD picture luma array in units of luma samples.

mod_pic_width specifies the width in units of luma samples of each tile constituting the packed modulation picture. mod_pic_width shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

mod_pic_height specifies the height in units of luma samples of each tile constituting the packed modulation picture. mod_pic_height shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

mod_fp_num_part_minus1 plus 1 specifies the number of tiles constituting the packed modulation picture. mod_fp_num_part shall not be equal to 0. mod_fp_num_part_minus1 shall be in the range 0 to 15 inclusive.

mod_pic_width_part_offset_x specifies the horizontal padding in between two consecutive tiles constituting the packed modulation picture.

Figure 1: Example of frame-packing configuration with mod_fp_different_views_size_flag equal to 0.

Figure 2: Example of frame-packing configuration with mod_fp_different_views_size_flag equal to 1.

The derivation of the decoded LDR picture arrays decLDRL of dimension ( WidthY  x  HeightY ), decLDRCb and decLDRCr of dimension ( WidthC  x  HeightC ), and of the decoded modulation picture sample array decModL of dimension ( WidthMod  x  HeightMod ) is performed as follows.

The inputs to this process are the reconstructed sample arrays recSampleY of dimension ( pic_width_in_luma_samples x pic_height_in_luma_samples ) and recSampleCb and recSampleCr of dimension (PicWidthInSamplesC x PicHeightInSamplesC ).

The variables mod_fp_num_part, WidthY, HeightY, WidthC, HeightC, WidthMod and HeightMod are derived as follows:

mod_fp_num_part = mod_fp_num_part_minus1 + 1

–	If mod_fp_different_views_size_flag equal to 0:

WidthY = pic_width_in_luma_samples

HeightY = mod_lbd_pic_height

WidthMod = pic_width_in_luma_samples

HeightMod = mod_lbd_pic_height

–	otherwise:

WidthY = mod_lbd_pic_width

HeightY = mod_lbd_pic_height

WidthMod = mod_pic_width

HeightMod = mod_fp_num_part  * mod_pic_height

The decoded LDR picture sample array decLDRL, decLDRCb, decLDRCr and the decoded modulation picture sample array decModL are derived as follows:

–	The variables row_offsetY and row_offsetC are derived as follows:

row_offsetY = CtbSizeY * Ceil( mod_lbd_pic_height ÷ CtbSizeY )

row_offsetC = offsetY / SubHeightC

decLDRL[x][y] = recSampleY[x][ row_offsetY + y] for x=0,… WidthY, y=0,… HeightY

decLDRCb [x][y] = recSampleCb [x][ row_offsetC + y] for x=0,… WidthC, y=0,… HeightC

decLDRCr [x][y] = recSampleCr [x][ row_offsetC + y] for x=0,… WidthC, y=0,… HeightC

–	If mod_fp_different_views_size_flag equal to 0, the decoded modulation picture sample array decModL is derived as follows:

decModL[x][y] = recSampleY [x][y] for x=0,… WidthMod, y=0,… HeightMod

–	otherwise, the decoded modulation picture sample array decModL is derived as follows:

row_offsetY = CtbSizeY * Ceil( mod_pic_height ÷ CtbSizeY )

row_offsetC = offsetY / SubHeightC

for r=0,…(mod_fp_num_part-1), the variables col_offsetY[r] and col_offsetC[r] are derived as follows:

col_offsetY[r] = r * ( mod_pic_width + mod_pic_width_part_offset_x)

col_offsetC[r]  = col_offsetY[r]  / SubHeightC

decModL[x][r* row_offsetY + y] = recSampleY [col_offsetY[r] + x][y] for x=0,… mod_pic_width, y=0,… mod_pic_height

mod_lbd_format  indicates the representation of the decoded LBD picture as specified in Table F.X after being decoded in accordance with this Specification. Values greater than 2 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore all modulation channel information SEI messages that contain a value of mod_lbd_format greater than 2.

Table F.X

| mod_lbd_format | Meaning |
| --- | --- |
| 0 | Unspecified |
| 1 | Rec. ITU-R BT.709-5 Y’CbCr |
| 2 | Rec. ITU-R BT.2020 Non Constant Luminance Y’CbCr |

NOTE 2 –  When mod_lbd_format points to a Y’CbCr picture (mod_lbd_format is equal to 1 or 2), the decoded LBD picture can be rendered on ‘conventional’ rendering devices else the decoded LBD picture is coded in a specific color space and may not be rendered properly on a ‘conventional’ rendering devices.

Figure X: HBD picture reconstruction process.

NOTE 3 – The reconstructed modulated picture reconstruction process is described in Figure X. The final color space and color format conversion of the reconstructed modulated picture sample arrays recHbdX, recHbdY and recHbdZ into the target output format, specified in the modulation channel information SEI message, is not part of the reconstruction process of the reconstructed modulated picture.

The variable decModL represents the decoded auxiliary picture with the same width and height as the associated primary coded picture of the current layer with AuxId[ nuhLayerId ] equal to AUX_MODULATION. The variable decLbd represents the luma and chroma sample arrays of the primary coded picture.

NOTE 4 – If decModL does not have the same width and height as the luma sample array of the associated decoded primary coded picture decLbdL, a resampling process of decModL not described in this Specification is invoked.

The derivation of the mapped modulation picture mapModL is performed according to the following ordered steps:

1. The variables MinModValue and MaxModValue are derived as follows:

–	If mod_lbd_format is equal to 0, the following applies:

MinModValue = – 4096
MaxModValue = 44916

–	Otherwise ( mod_lbd_format is equal to 1 or mod_lbd_format is equal to 2), the following applies:

MinModValue = 4096
MaxModValue = 27213

2. The variable Norm and the variable mapModL[ x ][ y ], the mapped modulation picture of the ( x, y) sample position, are derived as follows:

for( y = 0; y < pic_height_in_luma_samples; y++ )
	for( x = 0; x < pic_width_in_luma_samples; x++ ) {
		Norm = ( decModL[ x ][ y ] * ( MaxModValue – MinModValue ) + 
				( MinModValue << ModPicBitDepth ) + ( 1 << ( ModPicBitDepth – 1 ) ) ) >> ModPicBitDepth			Norm = LutPow2IntS9[ ( ( Norm + 4096 ) + 2 ) >> 3 ] >> 1
		mapModL[ x ][ y ] = ( Norm + 128 ) >> 8
	}

where LutPow2IntS9[ i ] is derived as follows, for i = 0 .. 511: 
	LutPow2IntS9[ i ] = Round( 16384*2 i  512 )

3. The variable mapModL[ x ][ y ] is processed as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1:

–	If mod_lbd_format is equal to 0, the following applies:

mapModL[ x ][ y ] = Max( 32, mapModL[ x ][ y ] )

–	Otherwise ( mod_lbd_format is equal to 1 or mod_lbd_format is equal to 2), the following applies:

mapModL[ x ][ y ] = Clip3( 128, 6400, mapModL[ x ][ y ] )

lbd_glog_a0 specifies the parameter a0 that is used to control the tone mapping process of the LBD picture samples. The value of lbd_glog_a0 shall be in the range of 0 to ( 215 – 1 ), inclusive.

lbd_glog_b0 specifies the parameter b0 that is used to control the tone mapping process of the LBD picture samples. The value of lbd_glog_b0 shall be in the range of 0 to ( 214 – 1 ), inclusive.

lbd_gslog_c0 specifies the parameter c0 that is used to control the tone mapping process of the LBD picture samples. The value of lbd_glog_c0 shall be in the range of 0 to ( 210 – 1 ), inclusive.

lbd_scaling_factor is used to control the tone mapping process of the LBD picture samples. The value of lbd_scaling_factor shall be in the range of 0 to ( 210 – 1 ), inclusive.

lbd_luminance_balance_factor is used to control the tone mapping process of the LBD picture samples. The value of lbd_luminance_balance_factor shall be in the range of 0 to ( 210 – 1 ), inclusive.

The variable decLbd[ x ][ y ][ c ] represents the sample value at coordinates (x, y) of the colour component c of the decoded primary coded picture decLbd (limited bit-depth picture).

The derivation of the mapped LBD picture mapLbd is performed according to the following ordered steps:

1. –	If chroma_format_idc is not equal to 3, a 4:4:4 chroma upsampling process not described in this Specification is invoked.

2. Depending on the value of mod_lbd_format, the recomposing mode is described as follows:

–	If mod_lbd_format is equal to 0, the inverse mapping process is performed according to the following ordered steps:

1. The variable idx, idxMod, Lambda, Scaling and the variables tmpScal and tmpPerceptual[ x ][ y ][ c ]  are derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1 and c = 0 .. 2:

idxMod = ( ( mapModL[ x ][ y ] – 32 ) + 1 ) >> 1
idx = Clip3( 0, 298, ( idxMod >> 6 ) )
Lambda = idxMod – ( idx << 6 )
Scaling = ( LutScalMod[ idx ] * ( 64 – Lambda ) + invScalMod[ idx + 1 ] * Lambda + 32 ) >> 6
tmpScal = ( 1053 >> ( BitDepthY – 8 ) ) * Scaling + 8 ) >> 4

tmpPerceptual[ x ][ y ][ 0 ] = ( decLbd[ x ][ y ][ 0 ] * tmpScal + 64 ) >> 7
tmpPerceptual[ x ][ y ][ 1 ] = ( ( decLbd[ x ][ y ][ 1 ] – ( 1 << (BitDepthC – 1 ) ) * tmpScal	+ 128 ) >> 8
tmpPerceptual[ x ][ y ][ 2 ] = ( ( decLbd[ x ][ y ][ 2 ] – ( 1 << (BitDepthC – 1 ) ) * tmpScal	+ 128 ) >> 8

where LutScalMod is given in Table 1.

Table 1: Derivation of variable LutScalMod from input idx

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LutScalMod | 1216 | 1053 | 994 | 956 | 929 | 908 | 890 | 875 | 862 | 851 | 840 | 831 | 822 | 814 | 807 |
| idx | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| LutScalMod | 800 | 794 | 788 | 782 | 777 | 771 | 767 | 762 | 757 | 753 | 749 | 745 | 741 | 738 | 734 |
| idx | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 |
| LutScalMod | 731 | 728 | 724 | 721 | 718 | 715 | 713 | 710 | 707 | 705 | 702 | 700 | 697 | 695 | 692 |
| idx | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| LutScalMod | 690 | 688 | 686 | 684 | 682 | 680 | 678 | 676 | 674 | 672 | 670 | 668 | 666 | 665 | 663 |
| idx | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| LutScalMod | 661 | 660 | 658 | 656 | 655 | 653 | 652 | 650 | 649 | 647 | 646 | 644 | 643 | 642 | 640 |
| idx | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| LutScalMod | 639 | 637 | 636 | 635 | 634 | 632 | 631 | 630 | 629 | 627 | 626 | 625 | 624 | 623 | 622 |
| idx | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 | 100 | 101 | 102 | 103 | 104 |
| LutScalMod | 620 | 619 | 618 | 617 | 616 | 615 | 614 | 613 | 612 | 611 | 610 | 609 | 608 | 607 | 606 |
| idx | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| LutScalMod | 605 | 604 | 603 | 602 | 601 | 600 | 599 | 598 | 598 | 597 | 596 | 595 | 594 | 593 | 592 |
| idx | 120 | 121 | 122 | 123 | 124 | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 |
| LutScalMod | 592 | 591 | 590 | 589 | 588 | 587 | 587 | 586 | 585 | 584 | 583 | 583 | 582 | 581 | 580 |
| idx | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| LutScalMod | 580 | 579 | 578 | 577 | 577 | 576 | 575 | 575 | 574 | 573 | 572 | 572 | 571 | 570 | 570 |
| idx | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 |
| LutScalMod | 569 | 568 | 568 | 567 | 566 | 566 | 565 | 564 | 564 | 563 | 563 | 562 | 561 | 561 | 560 |
| idx | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 | 175 | 176 | 177 | 178 | 179 |
| LutScalMod | 559 | 559 | 558 | 558 | 557 | 556 | 556 | 555 | 555 | 554 | 554 | 553 | 552 | 552 | 551 |
| idx | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 |
| LutScalMod | 551 | 550 | 550 | 549 | 549 | 548 | 547 | 547 | 546 | 546 | 545 | 545 | 544 | 544 | 543 |
| idx | 195 | 196 | 197 | 198 | 199 | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 | 208 | 209 |
| LutScalMod | 543 | 542 | 542 | 541 | 541 | 540 | 540 | 539 | 539 | 538 | 538 | 537 | 537 | 536 | 536 |
| idx | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 |
| LutScalMod | 535 | 535 | 534 | 534 | 533 | 533 | 532 | 532 | 531 | 531 | 531 | 530 | 530 | 529 | 529 |
| idx | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 | 238 | 239 |
| LutScalMod | 528 | 528 | 527 | 527 | 527 | 526 | 526 | 525 | 525 | 524 | 524 | 524 | 523 | 523 | 522 |
| idx | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 |
| LutScalMod | 522 | 521 | 521 | 521 | 520 | 520 | 519 | 519 | 519 | 518 | 518 | 517 | 517 | 517 | 516 |
| idx | 255 | 256 | 257 | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 |
| LutScalMod | 516 | 515 | 515 | 515 | 514 | 514 | 513 | 513 | 513 | 512 | 512 | 512 | 512 | 512 | 512 |
| idx | 270 | 271 | 272 | 273 | 274 | 275 | 276 | 277 | 278 | 279 | 280 | 281 | 282 | 283 | 284 |
| LutScalMod | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 |
| idx | 285 | 286 | 287 | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 | 298 | 299 |
| LutScalMod | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 |

2. The variable cMod, k1C, k1Crnd, Lambda, cConv, idx  are derived as follows:

cMod = SqrtInt( 	tmpPerceptual[ x ][ y ][ 1 ]  * tmpPerceptual[ x ][ y ][ 1 ] +
				tmpPerceptual[ x ][ y ][ 2 ]  * tmpPerceptual[ x ][ y ][ 2 ]  )
k1C = ( k1 * cMod + 4096 ) >> 13
k1Crnd = ( k1C << 7 )
idx  = Min( 639, ( k1CRnd >> 9 ) )
Lambda = k1Crnd – ( idx  << 9 )

cConv = ( LutLab[ idx ] * ( 512 – Lambda ) + LutLab[ idx + 1 ] * Lambda + 256 ) >> 9
cConv = cConv + 1024 + k1C

tmpPerceptual[ x ][ y ][ c ]   = ( tmpPerceptual[ x ][ y ][ c ] * cConv + 256 ) >> 9, for c = 1, 2

where SqrtInt is an integer-based root implementation of the square root operation, k1 = 492 and LutLab is given in Table 2.

Table 2: Derivation of variable LutLab from input idx

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LutLab | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 4 | 4 | 5 | 5 | 6 | 6 |
| idx | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| LutLab | 7 | 7 | 8 | 9 | 9 | 10 | 11 | 11 | 12 | 13 | 14 | 15 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 28 |
| idx | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| LutLab | 29 | 30 | 31 | 33 | 34 | 35 | 37 | 38 | 39 | 41 | 42 | 44 | 45 | 47 | 49 | 50 | 52 | 54 | 55 | 57 | 59 | 61 | 63 | 64 | 66 |
| idx | 75 | 76 | 77 | 78 | 79 | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 |
| LutLab | 68 | 70 | 72 | 74 | 76 | 79 | 81 | 83 | 85 | 87 | 90 | 92 | 94 | 97 | 99 | 102 | 104 | 107 | 109 | 112 | 114 | 117 | 120 | 123 | 125 |
| idx | 100 | 101 | 102 | 103 | 104 | 105 | 106 | 107 | 108 | 109 | 110 | 111 | 112 | 113 | 114 | 115 | 116 | 117 | 118 | 119 | 120 | 121 | 122 | 123 | 124 |
| LutLab | 128 | 131 | 134 | 137 | 140 | 143 | 146 | 149 | 152 | 155 | 159 | 162 | 165 | 168 | 172 | 175 | 179 | 182 | 186 | 189 | 193 | 197 | 200 | 204 | 208 |
| idx | 125 | 126 | 127 | 128 | 129 | 130 | 131 | 132 | 133 | 134 | 135 | 136 | 137 | 138 | 139 | 140 | 141 | 142 | 143 | 144 | 145 | 146 | 147 | 148 | 149 |
| LutLab | 212 | 216 | 220 | 224 | 228 | 232 | 236 | 240 | 244 | 248 | 253 | 257 | 261 | 266 | 270 | 275 | 279 | 284 | 289 | 293 | 298 | 303 | 308 | 313 | 318 |
| idx | 150 | 151 | 152 | 153 | 154 | 155 | 156 | 157 | 158 | 159 | 160 | 161 | 162 | 163 | 164 | 165 | 166 | 167 | 168 | 169 | 170 | 171 | 172 | 173 | 174 |
| LutLab | 323 | 328 | 333 | 338 | 344 | 349 | 354 | 360 | 365 | 371 | 376 | 382 | 387 | 393 | 399 | 405 | 411 | 417 | 423 | 429 | 435 | 441 | 447 | 453 | 460 |
| idx | 175 | 176 | 177 | 178 | 179 | 180 | 181 | 182 | 183 | 184 | 185 | 186 | 187 | 188 | 189 | 190 | 191 | 192 | 193 | 194 | 195 | 196 | 197 | 198 | 199 |
| LutLab | 466 | 473 | 479 | 486 | 493 | 499 | 506 | 513 | 520 | 527 | 534 | 541 | 548 | 555 | 563 | 570 | 577 | 585 | 592 | 600 | 608 | 615 | 623 | 631 | 639 |
| idx | 200 | 201 | 202 | 203 | 204 | 205 | 206 | 207 | 208 | 209 | 210 | 211 | 212 | 213 | 214 | 215 | 216 | 217 | 218 | 219 | 220 | 221 | 222 | 223 | 224 |
| LutLab | 647 | 655 | 663 | 672 | 680 | 688 | 697 | 705 | 714 | 723 | 731 | 740 | 749 | 758 | 767 | 776 | 786 | 795 | 804 | 814 | 823 | 833 | 843 | 852 | 862 |
| idx | 225 | 226 | 227 | 228 | 229 | 230 | 231 | 232 | 233 | 234 | 235 | 236 | 237 |  |  |  |  |  |  |  |  |  |  |  |  |
| LutLab | 872 | 882 | 892 | 902 | 913 | 923 | 933 | 944 | 954 | 965 | 976 | 987 | 998 |  |  |  |  |  |  |  |  |  |  |  |  |

| idx | idx | 238 | 239 | 240 | 241 | 242 | 243 | 244 | 245 | 246 | 247 | 248 | 249 | 250 | 251 | 252 | 253 | 254 | 255 | 256 | 257 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LutLab | Lab | 1009 | 1020 | 1031 | 1042 | 1054 | 1065 | 1077 | 1089 | 1100 | 1112 | 1124 | 1136 | 1148 | 1161 | 1173 | 1185 | 1198 | 1210 | 1223 | 1236 |
| idx | idx | 258 | 259 | 260 | 261 | 262 | 263 | 264 | 265 | 266 | 267 | 268 | 269 | 270 | 271 | 272 | 273 | 274 | 275 | 276 | 277 |
| LutLab | Lab | 1249 | 1262 | 1275 | 1288 | 1302 | 1315 | 1329 | 1342 | 1356 | 1370 | 1384 | 1398 | 1412 | 1426 | 1441 | 1455 | 1470 | 1485 | 1500 | 1515 |
| idx | idx | 278 | 279 | 280 | 281 | 282 | 283 | 284 | 285 | 286 | 287 | 288 | 289 | 290 | 291 | 292 | 293 | 294 | 295 | 296 | 297 |
| LutLab | Lab | 1530 | 1545 | 1560 | 1576 | 1591 | 1607 | 1623 | 1638 | 1654 | 1671 | 1687 | 1703 | 1720 | 1736 | 1753 | 1770 | 1787 | 1804 | 1821 | 1839 |
| idx | idx | 298 | 299 | 300 | 301 | 302 | 303 | 304 | 305 | 306 | 307 | 308 | 309 | 310 | 311 | 312 | 313 | 314 | 315 | 316 | 317 |
| LutLab | Lab | 1856 | 1874 | 1892 | 1910 | 1928 | 1946 | 1964 | 1983 | 2001 | 2020 | 2039 | 2058 | 2077 | 2096 | 2116 | 2135 | 2155 | 2175 | 2195 | 2215 |
| idx | idx | 318 | 319 | 320 | 321 | 322 | 323 | 324 | 325 | 326 | 327 | 328 | 329 | 330 | 331 | 332 | 333 | 334 | 335 | 336 | 337 |
| LutLab | Lab | 2235 | 2256 | 2276 | 2297 | 2318 | 2339 | 2360 | 2382 | 2403 | 2425 | 2447 | 2469 | 2491 | 2513 | 2536 | 2558 | 2581 | 2604 | 2627 | 2650 |
| idx | idx | 338 | 339 | 340 | 341 | 342 | 343 | 344 | 345 | 346 | 347 | 348 | 349 | 350 | 351 | 352 | 353 | 354 | 355 | 356 | 357 |
| LutLab | Lab | 2674 | 2697 | 2721 | 2745 | 2769 | 2794 | 2818 | 2843 | 2868 | 2893 | 2918 | 2943 | 2969 | 2994 | 3020 | 3046 | 3073 | 3099 | 3126 | 3153 |
| idx | idx | 358 | 359 | 360 | 361 | 362 | 363 | 364 | 365 | 366 | 367 | 368 | 369 | 370 | 371 | 372 | 373 | 374 | 375 | 376 | 377 |
| LutLab | Lab | 3180 | 3207 | 3235 | 3262 | 3290 | 3318 | 3346 | 3375 | 3403 | 3432 | 3461 | 3490 | 3520 | 3549 | 3579 | 3609 | 3640 | 3670 | 3701 | 3732 |
| idx | idx | 378 | 379 | 380 | 381 | 382 | 383 | 384 | 385 | 386 | 387 | 388 | 389 | 390 | 391 | 392 | 393 | 394 | 395 | 396 | 397 |
| LutLab | Lab | 3763 | 3794 | 3826 | 3858 | 3890 | 3922 | 3955 | 3987 | 4020 | 4053 | 4087 | 4120 | 4154 | 4188 | 4223 | 4257 | 4292 | 4327 | 4363 | 4398 |
| idx | idx | 398 | 399 | 400 | 401 | 402 | 403 | 404 | 405 | 406 | 407 | 408 | 409 | 410 | 411 | 412 | 413 | 414 | 415 | 416 | 417 |
| LutLab | Lab | 4434 | 4470 | 4506 | 4543 | 4580 | 4617 | 4654 | 4692 | 4730 | 4768 | 4806 | 4845 | 4884 | 4923 | 4962 | 5002 | 5042 | 5082 | 5123 | 5164 |
| idx | idx | 418 | 419 | 420 | 421 | 422 | 423 | 424 | 425 | 426 | 427 | 428 | 429 | 430 | 431 | 432 | 433 | 434 | 435 | 436 | 437 |
| LutLab | Lab | 5205 | 5246 | 5288 | 5330 | 5372 | 5415 | 5458 | 5501 | 5544 | 5588 | 5632 | 5677 | 5721 | 5766 | 5811 | 5857 | 5903 | 5949 | 5996 | 6043 |
| idx | idx | 438 | 439 | 440 | 441 | 442 | 443 | 444 | 445 | 446 | 447 | 448 | 449 | 450 | 451 | 452 | 453 | 454 | 455 | 456 | 457 |
| LutLab | Lab | 6090 | 6137 | 6185 | 6233 | 6282 | 6331 | 6380 | 6429 | 6479 | 6530 | 6580 | 6631 | 6682 | 6734 | 6786 | 6838 | 6891 | 6944 | 6997 | 7051 |
| idx | idx | 458 | 459 | 460 | 461 | 462 | 463 | 464 | 465 | 466 | 467 | 468 | 469 | 470 | 471 | 472 | 473 | 474 | 475 | 476 | 477 |
| LutLab | Lab | 7105 | 7159 | 7214 | 7270 | 7325 | 7381 | 7438 | 7494 | 7552 | 7609 | 7667 | 7725 | 7784 | 7843 | 7903 | 7963 | 8023 | 8084 | 8145 | 8207 |
| idx | idx | 478 | 479 | 480 | 481 | 482 | 483 | 484 | 485 | 486 | 487 | 488 | 489 | 490 | 491 | 492 | 493 | 494 | 495 | 496 | 497 |
| LutLab | Lab | 8269 | 8331 | 8394 | 8457 | 8521 | 8585 | 8650 | 8715 | 8780 | 8846 | 8913 | 8980 | 9047 | 9115 | 9183 | 9252 | 9321 | 9390 | 9460 | 9531 |
| idx | idx | 498 | 499 | 500 | 501 | 502 | 503 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LutLab | Lab | 9602 | 9674 | 9746 | 9818 | 9891 | 9965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| idx | 504 | 505 | 506 | 507 | 508 | 509 | 510 | 511 | 512 | 513 | 514 | 515 | 516 | 517 | 518 | 519 | 520 | 521 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LutLab | 10039 | 10113 | 10188 | 10264 | 10340 | 10416 | 10493 | 10571 | 10649 | 10728 | 10807 | 10887 | 10967 | 11048 | 11129 | 11211 | 11294 | 11377 |
| idx | 522 | 523 | 524 | 525 | 526 | 527 | 528 | 529 | 530 | 531 | 532 | 533 | 534 | 535 | 536 | 537 | 538 | 539 |
| LutLab | 11460 | 11544 | 11629 | 11715 | 11800 | 11887 | 11974 | 12062 | 12150 | 12239 | 12328 | 12418 | 12509 | 12600 | 12692 | 12785 | 12878 | 12972 |
| idx | 540 | 541 | 542 | 543 | 544 | 545 | 546 | 547 | 548 | 549 | 550 | 551 | 552 | 553 | 554 | 555 | 556 | 557 |
| LutLab | 13066 | 13161 | 13257 | 13353 | 13450 | 13548 | 13646 | 13745 | 13845 | 13945 | 14046 | 14148 | 14251 | 14354 | 14458 | 14562 | 14667 | 14773 |
| idx | 558 | 559 | 560 | 561 | 562 | 563 | 564 | 565 | 566 | 567 | 568 | 569 | 570 | 571 | 572 | 573 | 574 | 575 |
| LutLab | 14880 | 14987 | 15095 | 15204 | 15314 | 15424 | 15535 | 15647 | 15760 | 15873 | 15987 | 16102 | 16218 | 16334 | 16451 | 16569 | 16688 | 16808 |
| idx | 576 | 577 | 578 | 579 | 580 | 581 | 582 | 583 | 584 | 585 | 586 | 587 | 588 | 589 | 590 | 591 | 592 | 593 |
| LutLab | 16928 | 17050 | 17172 | 17295 | 17418 | 17543 | 17668 | 17795 | 17922 | 18050 | 18179 | 18308 | 18439 | 18571 | 18703 | 18836 | 18971 | 19106 |
| idx | 594 | 595 | 596 | 597 | 598 | 599 | 600 | 601 | 602 | 603 | 604 | 605 | 606 | 607 | 608 | 609 | 610 | 611 |
| LutLab | 19242 | 19379 | 19517 | 19655 | 19795 | 19936 | 20078 | 20220 | 20364 | 20508 | 20654 | 20800 | 20948 | 21096 | 21246 | 21397 | 21548 | 21701 |
| idx | 612 | 613 | 614 | 615 | 616 | 617 | 618 | 619 | 620 | 621 | 622 | 623 | 624 | 625 | 626 | 627 | 628 | 629 |
| LutLab | 21854 | 22009 | 22165 | 22321 | 22479 | 22638 | 22798 | 22959 | 23121 | 23285 | 23449 | 23615 | 23781 | 23949 | 24118 | 24288 | 24459 | 24631 |
| idx | 630 | 631 | 632 | 633 | 634 | 635 | 636 | 637 | 638 | 639 | 640 |  |  |  |  |  |  |  |
| LutLab | 24805 | 24980 | 25156 | 25333 | 25511 | 25690 | 25871 | 26053 | 26236 | 26421 | 26606 |  |  |  |  |  |  |  |

3. The variable tmpMapLbd is derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1:

tmpMapLbd[ x ][ y ][ 1 ] = ( 1130 * tmpPerceptual[ x ][ y ][ 0 ] + 4096 ) >> 13 )
tmpMapLbd[ x ][ y ][ 0 ] = ( 262 * tmpPerceptual[ x ][ y ][ 1 ] + 4096 ) >> 13 ) + tmpPerceptual[ x ][ y ][ 1 ]tmpMapLbd[ x ][ y ][ 2 ] = ( –655 * tmpPerceptual[ x ][ y ][ 2 ] + 4096 ) >> 13 ) + 
					tmpPerceptual[ x ][ y ][ 1 ]

NOTE 4 – tmpMapLbd is expressed in XYZ color space

4. The variable Map is derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1:

Map = ( ( (tmpMapLbd[ x ][ y ][ c ] − LbdGslogC0) * LbdGslogA0 + 16384 ) >> 15 ) + 2048
Map = ( LutPow2IntS9[ Max( 0, Map ) ] + 8 ) >> 4
tmpMapLbd[ x ][ y ][ c ]  = ( Map − LbdGslogB0 + 8 ) >> 3

where:
LbdGslogA0 = 13187
LbdGslogB0 = 1970
LbdGslogC0 = 3888
LutPow2IntS9[ i ] is derived as follows, for i = 0 .. 511:
		LutPow2IntS9[ i ] = Round( 16384*2 i  512 )

5. The variable mapLbd[ x ][ y ][ c ] is derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1:

mapLbd[ x ][ y ][ 0 ] = tmpMapLbd[ x ][ y ][ 0 ] + ( tmpMapLbd[ x ][ y ][ 0 ] * EpsilonD65x + 2048 ) >> 12
mapLbd[ x ][ y ][ 1 ] = tmpMapLbd[ x ][ y ][ 1 ]
mapLbd[ x ][ y ][ 2 ] = tmpMapLbd[ x ][ y ][ 2 ] + ( tmpMapLbd[ x ][ y ][ 2 ] * EpsilonD65z + 2048 ) >> 12

where:
EpsilonD65x = −203
EpsilonD65z = 365

6. The variable recHbd[ x ][ y ][ c ] that represents the reconstructed modulated picture is derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1, for c = 0 .. 2:

recHbd[ x ][ y ][ c ]  = Clip3( MinHbd, MaxHbd, ( mapLbd[ x ][ y ][ c ] * mapModL[ x ][ y ] ) ) / ( 1 << 17 )
where:
MinHbd = 640
MaxHbd = 1310720000

–	Otherwise ( mod_lbd_format is equal to 1 or mod_lbd_format is equal to 2 ),

The inverse mapping process is performed according to the following ordered steps:

1. For x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1:

decLbd[ x ][ y ][ c ] = decLbd[ x ][ y ][ c ] – ( 1 << (BitDepthC – 1) ), c = 1, 2

2. The variable tmpLbd[ x ][ y ][ c ] is derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1:

–	If ( mod_lbd_format is equal to 1 ),

tmpLbd [ x ][ y ][ 0 ] = ( 4096 * decLbd [ x ][ y ][ 0 ] + 5743 * decLbd [ x ][ y ][ 2 ] + 512 ) >> 10
tmpLbd [ x ][ y ][ 1 ] = ( 4096 * decLbd [ x ][ y ][ 0 ] – 1410 * decLbd [ x ][ y ][ 1 ] – 2095 * decLbd[ x ][ y ][ 2 ] + 512 ) >> 10
tmpLbd [ x ][ y ][ 2 ] = ( 4096 * decLbd [ x ][ y ][ 0 ] + 7258 * decLbd [ x ][ y ][ 1 ] + 512 ) >> 10

–	Otherwise ( mod_lbd_format is equal to 2 ),

tmpLbd [ x ][ y ][ 0 ] = ( 4096 * decLbd [ x ][ y ][ 0 ] + 6040 * decLbd [ x ][ y ][ 2 ] + 512 ) >> 10
tmpLbd [ x ][ y ][ 1 ] = ( 4096 * decLbd [ x ][ y ][ 0 ] – 674 * decLbd [ x ][ y ][ 1 ] – 							         2340 * decLbd[ x ][ y ][ 2 ] + 512 ) >> 10
tmpLbd [ x ][ y ][ 2 ] = ( 4096 * decLbd [ x ][ y ][ 0 ] + 7706 * decLbd [ x ][ y ][ 1 ] + 512 ) >> 10

NOTE 5 – This steps corresponds to a YCbCr to RGB color space conversion with 12-bit precision

3. The variable Map and the variable tmpMapLbd[ x ][ y ][ c ] are derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1, for c = 0 .. 2:

Map = ( tmpLbd [ x ][ y ][ c ] * lbd_scaling_factor + 128 ) >> 8 ) –  lbd_gslog_c0
Map = ( Map * lbd_gslog_a0 + 8192 ) >> 14
Map = ( ( ( LutPow2IntS8[  Map + 1024 ] + 8 ) >> 4 ) –  lbd_gslog_b0 + 16 ) >> 5
tmpMapLbd[ x ][ y ][ c ] = ( Map * lbd_luminance_balance_factor + 512 ) >> 10

4. The variable mapLbd[ x ][ y ][ c ] is derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1:

mapLbd[ x ][ y ][ 0 ] = ( 422 * tmpMapLbd[ x ][ y ][ 0 ] + 366 * tmpMapLbd[ x ][ y ][ 1 ] + 185 * 
					tmpMapLbd[ x ][ y ][ 2 ] + 128 )  >> 10
mapLbd[ x ][ y ][ 1 ] = ( 218 * tmpMapLbd[ x ][ y ][ 0 ] + 732 * tmpMapLbd[ x ][ y ][ 1 ] + 74 *  							tmpMapLbd[ x ][ y ][ 2 ] + 128 )  >> 10
mapLbd[ x ][ y ][ 2 ] = ( 20 * tmpMapLbd[ x ][ y ][ 0 ] + 122 * tmpMapLbd[ x ][ y ][ 1 ] + 973 *  
					tmpMapLbd[ x ][ y ][ 2 ] + 128 )  >> 10

where LutPow2IntS8[ i ] is derived as follows, for i = 0 .. 255:
				LutPow2IntS8[ i ] = Round( 16384*2 i  256 )

NOTE 7 – The inverse LBD mapped picture mapLbd is expressed in XYZ color space.
NOTE 8 – In format YCbCr, the decoded LBD picture (decLbd) is displayable on rendering devices compliant with BT.709 or BT.2020 NCL.

5. The variable recHbd[ x ][ y ][ c ] that represents the reconstructed modulated picture is derived as follows, for x = 0 .. pic_width_in_luma_samples – 1, for y = 0 .. pic_height_in_luma_samples – 1, for c = 0 .. 2:

recHbd[ x ][ y ][ c ] = Clip3( MinHbd, MaxHbd, ( mapLbd[ x ][ y ][ c ] * mapModL[ x ][ y ]  + 16 ) >> 5 )
					 / ( 1 << 10 )

where:
MinHbd = 5
MaxHbd = 10240000

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).