| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0140 |
| --- | --- |

| Title: | SCCE4: Subset 3.2 CU based 1D Dictionary with only 2CTU reference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Ying Chen Joel Sole Marta Karczewicz | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com cheny@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal presents a pixel based 1D dictionary as a follow-up proposal for JCTVC-Q0149, with some simplifications and clean-ups. The string matching is done by allowing reference pixels within the CTU and the left CTU. Hash based fast search is enabled at the encoder. The simulation results report that the proposed method achieves 11.7% and 10.5% BD-rate savings for 1080P RGB and YUV respectively in full frame intra BC condition, while 14.4% and 11.9% BD-rate savings for 1080P RGB and YUV in the 2CTU intra BC condition.

# Introduction

In the Valencia meeting, SCCE4 [1] was established to study different sample matching methods, including JCTVC-Q0149 [2]. In JCTVC-Q0149, string matching is performed by referring to any previous decoded pixels. Therefore, it has a search range similar to full frame Intra Block Copy (BC). Both encoder and decoder maintain a previously coded pixel buffer (namely dictionary) for prediction and reconstruction.

# Proposed CU based 1D dictionary

The proposed 1D dictionary method has several features summarized as follows:

- The method utilizes the same reconstruction buffer as intra BC, with up to 2CTU search range, and therefore, there is no additional buffer to store the previously coded pixels.

- The reference search area is constrained to be 2CTU : left CTU (if applicable) and current CTU

- The reference string is signaled through 2D motion vector.

- When the current string starts a new line, the reference string also starts a new line. In other words, the current string and reference string have the same shape, as shown in Fig. 1.

- The previously coded pixels within the current CU are allowed to be chosen as reference pixels.

- 1D dictionary is enabled in a CU basis. If a CU is coded with 1D dictionary, horizontal raster scan applies for pixels within this CU. Note that it is processed CU by CU.

Fig. 1. CU based 1D dictionary with reference string and current string

# Simulation results

The proposed scheme is implemented on top of SCM1.0 reference software and tested using the common test condition defined in [3]. Table 1 and Table 2 demonstrate the coding performance under full frame intra BC and 2CTU intra BC test conditions.

Table 1. Results in Full frame Intra BC                Table 2. Results in 2CTU Intra BC

# References

[1] Y. Chen and J. Xu, “HEVC Screen Content Coding Core Experiment 4 (SCCE4): String matching ofr sample coding”, JCT-VC Document, JCTVC-Q1124, Valencia, Spain, March 2014.

[2] F. Zou, Y. Chen, C. Pang, J. Sole and M. Karczewicz, “Pixel-based 1D dictionary coding”, JCT-VC Document, JCTVC-Q0149, Valencia, Spain, March 2014.

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-Q1015, March 2014

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).