| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0142 |
| --- | --- |

| Title: | SCCE3 Test A.4: color table signaling with merge, sharing and DPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Zhan Ma, Wei Wang, Haoping Yu 2330 Central Expy, Santa Clara CA 95050 United States | Tel: Email: | +1 408 330 5142 zhan.ma@huawei.com, haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution reports the results of SCCE3 Test A.4 on color table signaling with merge, sharing and DPCM. It is reported that the proposed method provides bitrate reduction of 16.5% and 10.9% against SCM-1.0 anchor, for coding of 1080p text & graphics content with 2 CTU and full frame Intra Block Copy search ranges, in all intra configuration, respectively. The gains for lossy coding in these categories are -12.6% and -9.2%.

# Introduction

JCTVC-Q0034/JCTVC-Q0167 [2, 3] were proposed to derive the color table using packed (interleaved) components together. It includes color table merge from neighboring blocks as implicit signaling method. It further contains the inter table sharing and intra table DPCM as explicit signaling mechanism. In this contribution, the color table generation and coding method in JCTVC-Q0034/JCTVC-Q0167 is implemented on top of SCCE3 palette anchor (SCM-1.0+JCTVC-Q0094+bug-fix of JCTVC-Q0047) to further demonstrate the coding efficiency.

# Proposed methods

## Color table derivation

- For each CU, it first creates the color histogram using 3 color components together, i.e., packed G, B, R or packed Y, Cb, Cr, according to its frequency of occurrence in descending order;

- A hash based method is used to group adjacent colors;

  - For each packed color, a hash value is formed by concatenating M MSB of each color component, where M is QP-dependent. The entire color space is then separated into (256/2^M)^3 partitions;

  - Starting from the most frequently occurred color, it is selected as a representation color, while all the others lie in the same partition and the neighboring partitions are grouped together;

  - This approach continues until N colors are chosen. In current solution, N <= 128;

- For these N chosen colors, they are re-ordered by sorting them in a ascending order based on the value of packed colors, i.e., (G<<16)+(B<<8)+(R) or (Y<<16)+(Cb<<8)+(Cr).

## Reference color table generation

For each CTU, if there exists a sub-CU within current CTU that is coded by color palette mode, then the largest sub-CU’s color table is selected as the reference color table of current CTU. If multiple CUs share the same size color table, then the color table of the last CU’s is selected as reference.

If current CTU does not contain any sub-CU that is coded using palette mode, then its left CTU’s color table, if available, is selected as reference. Reference table is initialized row by row for the first CTUs in each row.

## Color table Merge

A CU coded with color table merge mode could reuse the color table from previously coded CUs. An additional flag is signaled to indicate whether the reference color table from the above or left CTU is used for current CU.

## Color table Sharing and DPCM

Inter-table sharing and Intra-table DPCM are used for color table coding. For the Inter-table sharing, the left CTU’s color table is selected as the reference table. Instead of signaling the color itself, it tries to find the matched colors in reference table and signal the difference between actual matched color index and predicted index.  Here, predicted index is derived using the previously encoded/decoded color.   For those colors which couldn’t find the matched ones in reference table, they are encoded predicatively using previously encoded color components.

## Residual quantization and coding

The residuals are quantized and coded using the same method as in HEVC.

# Experimental results

Simulations are performed under the common test condition [1]. Full test results are provided in the attached Excel spreadsheets. Table 1 and Table 2 summarize the All-Intra coding results with respect to SCM-1.0 anchor with 2CTU Intra Block Copy search ranges.

Table 1: Intra lossless coding results with respect to SCM-1.0 for 2CTU Intra Block Copy

Table 2: Intra lossy coding results with respect to SCM-1.0 for 2CTU Intra Block Copy

# Recommendation

It is recommend that the committee adopts this tool as the basis for further development of color table encoding of color palette mode.

# Reference

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu, “Description of screen content coding technology proposal by Huawei Technologies (USA)”, JCTVC-Q0034, Valencia, Spain, April 2014

- W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu, “AHG8: String match in coding of screen content”, JCTVC-Q0176, Valencia, Spain, April 2014.

# Patent rights declaration(s)

Huawei Technologies (USA) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).