Draft Text specification for SCCE3 Test A.4

This text specification is on top of JCTVC-Q0034_v4.

## Known issues in the text:

- Chroma sampling formats other than 4:4:4 are not supported.

## Syntax

7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for( i = 0; i < 1; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| sps_extension_7bits | u(7) |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| palette_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !intra_bc_flag[ x0 ][ y0 ] ) |  |
| pred_mode_flag | ae(v) |
| if( palette_enabled_flag && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) { |  |
| color_table_size | ae(v) |
| color_table(x0, y0) |  |
| } |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] && palette_mode_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| color_table( x0, y0) { | Descriptor |
| --- | --- |
| color_table_merge_flag | ae(v) |
| if( color_table_merge_flag ) { |  |
| color_table_merge_dir | ae(v) |
| }else { |  |
| color_table_idx_ref[ MAX_COLOR_TABLE_SIZE] = { -1, …, -1 } |  |
| prev_pred_color_table_idx = -1 |  |
| for( ch = 0; ch < 3; ch++ ) { |  |
| num_bits_for_max_color_diff | ae(v) |
| color_table_share_enabled = 1 |  |
| for( i = 0; i < color_table_size; i++ ) { |  |
| if( ch = = 0 && color_table_share_enabled ! = 0 ) { |  |
| color_table_share_flag | ae(v) |
| if( color_table_share_flag ) { |  |
| color_table_share_idx_diff | ae(v) |
| pred_color_table_idx = |  |
| color_table_idx_ref[ i ] = pred_color_table_idx + color_table_share_idx_diff |  |
| } |  |
| if( pred_color_table_idx > = ref_color_table_size ) |  |
| color_table_share_enabled = 0 |  |
| prev_pred_color_table_idx = pred_color_table_idx |  |
| } |  |
| if( color_table_idx_ref[ i ] < 0 ) { |  |
| if( i = = 0 ) { |  |
| color_value | u(8) |
| } else { |  |
| abs_color_diff | ae(v) |
| if( abs_color_diff ! = 0 |  |
| ¦ ¦ ( ch = = 1 && color_table[ i ][ 0 ] ! = color_table[ i – 1 ][ 0 ] ¦ ¦ |  |
| ch = = 2 && (color_table[ i ][ 0 ] ! = color_table[ i – 1 ][ 0 ] ¦ ¦ |  |
| color_table[ i ][ 1 ] = = color_table[ i – 1 ][ 1 ] ) ) |  |
| ¦ ¦ ( color_table[ i – 1 ][ ch ] – abs_color_diff > = 0 |  |
| && color_table[ i – 1 ][ ch ] + abs_color_diff < = 255 ) ) |  |
| color_diff_sign | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.3.2 Sequence parameter set RBSP semantics

sps_extension_flag[ 0 ] equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, palette_enabled_flag, implicit_rdpcm_enabled_flag, explicit_rdpcm_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, high_precision_offsets_enabled_flag, fast_rice_adaptation_enabled_flag, and cabac_bypass_alignment_enabled_flag are present in the SPS RBSP syntax structure. sps_extension_flag[ 0 ] equal to 0 specifies that these syntax elements are not present.

palette_enabled_flag equal to 1 specifies that the palette mode may be used for intra blocks. palette_enabled_flag equal to 0 specifies that the palette mode is not applied. When not present, the value of palette_enabled_flag is inferred to be equal to 0.

## Semantics

palette_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded using the palette mode. palette_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded using the palette mode.

When palette_mode_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

color_table_size[x0][y0]: specifies the number of colors in color table

color_table_merge_flag[x0][y0]: equal to 1 specifies that current coding unit uses the color table derived from neighbor blocks. color_table_merge_flag[x0][y0] equal to 0 specifies that current coding unit uses the color table derived from itself.

color_table_merge_dir[x0][y0]: equal to 1 specifies that current coding unit uses the color table derived from its left neighbor block, while color_table_merge_dir[x0][y0] equal to 0 specifies that current coding unit uses the color table derived from its upper neighbor block.

num_bits_for_max_color_diff[x0][y0][n]: specifies the number of bits required to represent the maximum color difference of color component n in color table.

color_table_share_flag[x0][y0][n][cIdx]: equal to 1 specifies that current color entry of current color table uses the color for sharing from its reference table. color_table_share_flag[x0][y0][n][cIdx] equal to 0 specifies that current color entry is not shared from the reference table.

color_table_share_idx_diff[x0][y0][n][cIdx]: specifies the color index difference for shared color entry.

color_value[x0][y0][n][0]: specifies the value of the first color entry of current color table.

abs_color_diff[x0][y0][n][cIdx]: specifies the absolute of color difference for successive entries in current color table.

color_diff_sign[x0][y0][n][cIdx]: specifies the sign of color difference for successive entries in current color table.