| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0142 |
| --- | --- |

| Title: | SCCE3 Test A.4: color table signaling with merge, sharing and DPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Zhan Ma, Wei Wang, Haoping Yu 2330 Central Expy, Santa Clara CA 95050 United States | Tel: Email: | +1 408 330 5142 zhan.ma@huawei.com, haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution reports results of SCCE3 Test A.4 on color table signaling with merge, sharing and DPCM. It is reported that the proposed method provides bitrate reduction of 16.5% and 10.9% against SCM-1.0 anchor, for lossless coding of 1080p text & graphics content with 2 CTU and full Intra Block Copy search ranges, in all intra configuration, respectively. The BD-Rate gains for lossy coding in these categories are -12.6% and -9.2%.

# Introduction

JCTVC-Q0034/JCTVC-Q0167 proposed to derive the color table using packed (interleaved) components together. It including color table merge from neighboring blocks as implicit signaling method. It further contains the inter table sharing and intra table DPCM as explicit signaling mechanism. In this contribution, the color table generation and coding method in JCTVC-Q0034/JCTVC-Q0167 is implemented on top of SCCE3 palette anchor (SCM-1.0+JCTVC-Q0094+bug-fix of JCTVC-Q0047).

# Proposed methods

## Reference color table generation

For each CTU, if there exist a sub-CU within current CTU that is coded by palette mode, then the largest sub-CU’s color table is selected as the reference color table of current CTU. If multiple CUs have the same size table, then color table of the last CU is selected as reference.

If current CTU does not contain any sub-CU that is coded using palette mode, then its left CTU’s color table, is selected as reference, except the first CTU of each CTU row.

# Experimental results

Full test results are provided in the attached Excel spreadsheets. Table 1 and Table 2 summarize the All-Intra coding results with respect to SCM-1.0 anchor with 2CTU Intra Block Copy search ranges.

Table 1: Intra lossless coding results with respect to SCM-1.0 for 2CTU Intra Block Copy

Table 2: Intra lossy coding results with respect to SCM-1.0 for 2CTU Intra Block Copy

# Patent rights declaration(s)

Huawei Technologies (USA) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).