Draft Text specification for SCCE3 Test C.3

This text specification is on top of JCTVC-Q0034_v4/JCTVC-Q0176_v2.

## Known issues in the text:

- Chroma sampling formats other than 4:4:4 are not supported.

## Syntax

7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| … |  |
| … |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| sps_hybrid_color_index_search_eabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| …. |  |
| … |  |
| } |  |

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| color_table_idx_flag | ae(v) |
| if( color_table_idx_flag ) { |  |
| color_table_size | ae(v) |
| color_table(x0, y0) |  |
| if ( color_table_size > 1 ) { |  |
| nCbS = 1 << log2CbSize |  |
| color_index_map(x0, y0, nCbS, nCbS) |  |
| } |  |
| } else { |  |
| … |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] && color_table_idx_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |

| color_table( x0, y0) { | Descriptor |
| --- | --- |
| color_table_merge_flag | ae(v) |
| if( color_table_merge_flag ) { |  |
| color_table_merge_dir | ae(v) |
| }else { |  |
| color_table_idx_ref[ MAX_COLOR_TABLE_SIZE] = { -1, …, -1 } |  |
| prev_pred_color_table_idx = -1 |  |
| for( ch = 0; ch < 3; ch++ ) { |  |
| num_bits_for_max_color_diff | ae(v) |
| color_table_share_enabled = 1 |  |
| for( i = 0; i < color_table_size; i++ ) { |  |
| if( ch = = 0 && color_table_share_enabled ! = 0 ) { |  |
| color_table_share_flag | ae(v) |
| if( color_table_share_flag ) { |  |
| color_table_share_idx_diff | ae(v) |
| pred_color_table_idx = |  |
| color_table_idx_ref[ i ] = pred_color_table_idx + color_table_share_idx_diff |  |
| } |  |
| if( pred_color_table_idx > = ref_color_table_size ) |  |
| color_table_share_enabled = 0 |  |
| prev_pred_color_table_idx = pred_color_table_idx |  |
| } |  |
| if( color_table_idx_ref[ i ] < 0 ) { |  |
| if( i = = 0 ) { |  |
| color_value | u(8) |
| } else { |  |
| abs_color_diff | ae(v) |
| if( abs_color_diff ! = 0 |  |
| ¦ ¦ ( ch = = 1 && color_table[ i ][ 0 ] ! = color_table[ i – 1 ][ 0 ] ¦ ¦ |  |
| ch = = 2 && (color_table[ i ][ 0 ] ! = color_table[ i – 1 ][ 0 ] ¦ ¦ |  |
| color_table[ i ][ 1 ] = = color_table[ i – 1 ][ 1 ] ) ) |  |
| ¦ ¦ ( color_table[ i – 1 ][ ch ] – abs_color_diff > = 0 |  |
| && color_table[ i – 1 ][ ch ] + abs_color_diff < = 255 ) ) |  |
| color_diff_sign | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| color_index_map(x0, y0, nCbS, nCbS ) { | Descriptor |
| --- | --- |
| color_idx_map_scan_dir | ae(v) |
| if( sps_hybrid_color_index_search_eabled_flag && nCbS = = MAX_CU_SIZE ) |  |
| hybrid_color_index_search_eabled_flag = 1 |  |
| else |  |
| hybrid_color_index_search_eabled_flag = 0 |  |
| for( i = 0; i < nCbS * nCbS ) { |  |
| idx_match _flag | ae(v) |
| if( idx_match_flag ) |  |
| unmatched_idx | ae(v) |
| else { |  |
| if( hybrid_color_index_search_eabled_flag ) |  |
| 2d_block_search_flag | ae(v) |
| if( 2d_block_search_flag ) { |  |
| 2d_lcu_index | ae(v) |
| 2d_distance | ae(v) |
| 2d_length | ae(v) |
| } else { |  |
| sig_dist_flag | ae(v) |
| if( sig_dist_flag ) |  |
| dist_equal_one_flag | ae(v) |
| else |  |
| distance_minus2 | ae(v) |
| sig_len_ flag | ae(v) |
| if ( sig_len_ flag ) { |  |
| length_minus1 | ae(v) |
| i += length_minus1 + 1; |  |
| } else { |  |
| length_minus_cuWidth | ae(v) |
| i += length_minus_cuWidth + cuWidth; |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.3.2 Sequence parameter set RBSP semantics

sps_hybrid_color_index_search_eabled_flag equal to 1 specifies that hybrid_color_index_search_eabled_flag is present in the SPS RBSP syntax structure. sps_hybrid_color_index_search_eabled_flag equal to 0 specifies hybrid_color_index_search_eabled_flag is not present.

## Semantics

color_table_size[x0][y0]: specifies the number of colors in color table

color_table_merge_flag[x0][y0]: equal to 1 specifies that current coding unit uses the color table derived from neighbor blocks. color_table_merge_flag[x0][y0] equal to 0 specifies that current coding unit uses the color table derived from itself.

color_table_merge_dir[x0][y0]: equal to 1 specifies that current coding unit uses the color table derived from its left neighbor block, while color_table_merge_dir[x0][y0] equal to 0 specifies that current coding unit uses the color table derived from its upper neighbor block.

num_bits_for_max_color_diff[x0][y0][n]: specifies the number of bits required to represent the maximum color difference of color component n in color table.

color_table_share_flag[x0][y0][n][cIdx]: equal to 1 specifies that current color entry of current color table uses the color for sharing from its reference table. color_table_share_flag[x0][y0][n][cIdx] equal to 0 specifies that current color entry is not shared from the reference table.

color_table_share_idx_diff[x0][y0][n][cIdx]: specifies the color index difference for shared color entry.

color_value[x0][y0][n][0]: specifies the value of the first color entry of current color table.

abs_color_diff[x0][y0][n][cIdx]: specifies the absolute of color difference for successive entries in current color table.

color_diff_sign[x0][y0][n][cIdx]: specifies the sign of color difference for successive entries in current color table.

color_idx_map_scan_dir[x0][y0]: equal to 1 specifies that the index map of current coding unit uses the horizontal scanning; color_idx_map_scan_dir[x0][y0] equal to 0 specifies that the index map of current coding unit uses the vertical scanning;

idx_match _flag[x0][y0][cIdx]: equal to 1 specifies that current index could find its match from reference buffer; idx_match _flag[x0][y0][cIdx]: equal to 0 specifies that current index could not find its match from reference buffer;

unmatched_idx[x0][y0][cIdx]: specifies the value of current index which could not find its match from reference buffer. After parsing unmatched_idx [x0][y0][cIdx], cIdx = cIdx + 1;

hybrid_color_index_search_eabled_flag: equal to 1 specifies that current coding unit is using hybrid 1D and 2D search mode. hybrid_color_index_search_eabled_flag equal to 0 specifies that current coding unit uses only 1D search method.

2d_block_search_flag[x0][y0]: equal to 1 specifies that the type of current matched pair is a 2D block match.  2d_block_search_flag[x0][y0] equal to 0 specifies that the type of current matched pair is a 1D string match.

2d_lcu_index[x0][y0][cIdx]: specifies the index of referenced CTU where 2D match was found. It is the nth CTU to the left side of current CTU. n=0 means that 2D match was found within current CTU.

2d_distance[x0][y0][cIdx]: specifies the distance of current matched pair.  It is the raster scan index of top-left corner of the matched reference block within the reference CTU.

2d_length[x0][y0][cIdx]: specifies the length of current matched 2D pair.

The variable 2D width and height are derived as follows:

2DWidth[x0][y0][cIdx] = (2d_length[x0][y0][cIdx]%(1+minw-MIN_2D_WIDTH))+MIN_2D_WIDTH;

2DHeight[x0][y0][cIdx] = (2d_length[x0][y0][cIdx]/(1+minw-MIN_2D_WIDTH))+MIN_2D_HEIGHT;

sig_dist_flag[x0][y0][cIdx]: equal to 1 specifies that the distance of current matched pair is significant distance; sig_dist_flag[x0][y0][cIdx] equal to 0 specifies that the distance of current matched pair is not significant distance;

dist_equal_one_flag[x0][y0][cIdx]: equal 1 specifies that the distance equals to 1; dist_equal_one_flag[x0][y0][cIdx] equal 0 specifies that the distance equals to the width of current coding unit.

distance_minus2[x0][y0][cIdx]: specifies the distance minus 2 of current matched pair.

The variable distance is derived as follows:

distance = distance_minus2[x0][y0][cIdx] + 2;

sig_len_flag[x0][y0][cIdx]: equal to 1 specifies that the length of current matched pair is significant length; sig_len_flag[x0][y0][cIdx] equal to 0 specifies that the length of current matched pair is not significant length;

length_minus1[x0][y0][cIdx]: specifies the length minus 1 of current matched pair.

The variable length is derived as follows:

Length = length_minus [x0][y0][cIdx] + 1;

cIdx   = cIdx + length;

length_minus_cuWidth[x0][y0][cIdx]:: specifies the length minus the width of current coding unit for current matched pair.

The variable length is derived as follows:

Length = length_minus_cuWidth[x0][y0][cIdx] + cuWidth;

cIdx   = cIdx + length;