| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0144 |
| --- | --- |

| Title: | SCCE3 Test C.3: Combined Color Table and Index Map Coding of A.4 and B.8 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhan Ma, Wei Wang, Meng Xu, Haoping Yu 2330 Central Expy, Santa Clara CA 95050 United States | Tel: Email: | +1 408 330 5142 zhan.ma@huawei.com, haoping.yu@huawei.com |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution reports results of SCCE3 Test C.3 on combined color table and index map coding of SCCE3 Test A.4 and B.8. It is reported that the proposed method provides bitrate reduction of 29.2% and 18.3% against SCM-1.0 anchor, for lossless coding of 1080p text & graphics content with 2 CTU and full frame Intra Block Copy search ranges, in all intra configuration, respectively. The BD-rate gains for lossy coding in these categories are -29.7% and -19.3%.

# Introduction

JCTVC-Q0034/JCTVC-Q0167 [2. 3] proposed to code the color table using packed (interleaved) components together, and code the index map using string match method. In this contribution, the color table derivation, merge, sharing, and DPCM, together with hybrid 1-D and 2-D string search and coding methods in JCTVC-Q0034/JCTVC-Q0167 are implemented on top of SCCE3 palette anchor (SCM-1.0+JCTVC-Q0094+bug-fix of JCTVC-Q0047).

# Proposed methods

## Color table processing

### Color table derivation

- For each CU, it first creates the color histogram using 3 color components together, i.e., packed G, B, R or packed Y, Cb, Cr, according to its frequency of occurrence in descending order;

- A hash based method is used to merge adjacent colors;

  - For each packed color, a hash value is formed by concatenating M MSB of each color component, where M is QP-dependent. The entire color space is then separated into (256/2^M)^3 partitions;

  - Starting from the most frequently occurred color, it is selected as a representation color, while all the others lie in the same partition and the neighboring partitions are grouped together;

  - This approach continues until N colors are chosen. In current solution, N <= 128;

- For these N chosen colors, they are re-ordered by sorting them in a ascending order based on the value of packed colors, i.e., (G<<16)+(B<<8)+(R) or (Y<<16)+(Cb<<8)+(Cr).

### Reference color table generation

For each CTU, if there exists a sub-CU within current CTU that is coded by palette mode, then the largest sub-CU’s color table is selected as the reference color table of current CTU. If multiple CU have the largest size, then the last CU’s color table is selected as reference.

If current CTU does not contain any sub-CU that is coded using palette mode, then its left CTU’s color table, if available, is selected as reference. Reference table is initialized row by row for the first CTUs in each row.

### Color table merge

A CU coded with color table merge mode could reuse the color table from previously coded CUs. An additional flag is signaled to indicate whether the reference color table from the above or left CTU is used for current CU.

### Color table coding

Inter-table sharing and Intra-table DPCM are used for color table coding. For the Inter-table sharing, the left CTU’s color table is selected as the reference table. Instead of signaling the color itself, it tries to find the matched colors in reference table and signal the difference between actual matched color index and predicted index.  Here, predicted index is derived using the previously encoded/decoded color.   For those colors which couldn’t find the matched ones in reference table, they are encoded predicatively using previously encoded color components.

## Index map processing

### 1-D string match

- Both horizontal and vertical index map scanning is used

- For each scanning pattern, 1-D string search is applied from the first index (the most top left) to the last index (the most bottom right) to derive the matched pairs.   Previously processed indices will be used as the reference in the successive index search.

- If current index couldn’t find its match from previously processed indices, it is encoded as the unmatched pair by sending a flag (signaling no-match) and the index value itself, i.e., (unmatched_flag=1, index).

- If current index finds one or more matches from the reference buffer, the search continues for the next pixel, and so on. The matched pairs are encoded as by sending a flag (signaling matched string), the distance (offset between the current position and reference position) and the length of successive matched indices, i.e., (unmatched_flag =0, distance, length).

  - If there are multiple matches found for the current index, the pair providing the best R-D performance should be chosen. (To reduce the complexity, the current implementation uses a heuristic manner to select the pair with the longest matched length).

To improve the performance, additional one-line buffer for horizontal scan and one-column buffer for vertical scan are provided for the index map processing of each CU, hence, the first index in current CU could possibly find its match from the line buffer instead of encoding it directly as unmatched index.  Since these upper and left line buffers are already preserved to store the reconstructed pixels for intra prediction, this proposal reuses these two buffers without introducing additional buffers.  To use it for index search, all pixels in the line buffers are converted to the index by using the color table for the current CU.

### Hybrid 1-D string match and 2-D block match

In addition to using 1-D string search to derive the matched pixels, 2-D block search is also added to derive the matched blocks.  Previously processed indices will be used as the reference for the successive index search.  Multiple CTUs, e.g. up to 3 CTUs, to the left of current CTU will also be used as the reference area for 2-D block search.

- Both 1-D and 2-D search are performed for the current index. For the 1-D search result, search engine records matched length and corresponding distance or location.  For the 2-D search result, search engine records matched block width and height, corresponding reference LCU index and the location of reference index in the LCU.

- If the current index couldn’t find its match from previously processed indices, it is encoded as the unmatched index by encoding a flag indicating unmatched pixel and the index value itself.

- If the current index chooses a 1-D matched string as its search result, the encoding is done by sending a flag indicating matched pixel,  the match type flag (1-D), the distance (offset between current position and matched position) and the length of successive matched indices.

  - If there are multiple 1-D matches for current index, the pair providing the best R-D performance should be chosen. (To save the complexity, current implementation uses a heuristic manner to select the pair with the longest matched length).

- If the current index chooses a 2-D match as its search result, the encoding is done by sending a flag indicating the matched pixel, the match type flag (2-D), the reference LCU index, the distance (the address of matched position in reference LCU) and the length (width/height) of successive matched block.

  - If there are multiple 2-D matches for current index, the pair providing the best R-D performance should be chosen. (To save the complexity, current implementation uses a heuristic manner to select the pair with the biggest matched area).

- A Coded Map is introduced in the coding process.  Whenever a match is found, the area covered by 1-D length or 2-D block is marked in the Coded Map. After the coding of the current index is finished, the coding process scans through the coded map to find the first un-coded location as the next index to process.

- Also in the current 1-D/2-D string search, if any of the location in the current string has already been coded in previous 2-D search, the match result for this location is always treated as a match even though the index value between the current location and reference location may be different.

- Note the flag mentioned above for unmatched and matched signaling may be just a one-bit flag used in all cases.

In current SCCE3 test, hybrid 1-D and 2-D search is enabled on CTU level only.

## Residual quantization and coding

The residuals are quantized and coded using the same method as in HEVC.

# Experimental results

Simulations are performed under the common test condition [1]. Full test results are provided in the attached Excel spreadsheets. Table 1 and Table 2 summarize the All-Intra coding results with respect to SCM-1.0 anchor with 2CTU Intra Block Copy search ranges. Left 3 CTUs are used for hybrid 1-D and 2-D string search.

Text spec is provided as another separated document as attached.

Table 1: Intra lossless coding results with respect to SCM-1.0 for 2CTU Intra Block Copy

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 29.2% | 28.4% | 14.3% | 44.5% |
| RGB, text & graphics with motion,720p | 16.5% | 17.0% | 1.4% | 31.1% |
| RGB, mixed content, 1440p | 4.8% | 4.8% | 4.7% | 4.9% |
| RGB, mixed content, 1080p | 3.9% | 3.9% | 3.9% | 3.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 35.4% | 34.2% | 14.5% | 51.2% |
| YUV, text & graphics with motion,720p | 16.5% | 18.5% | 1.5% | 39.2% |
| YUV, mixed content, 1440p | 2.6% | 3.0% | 0.8% | 5.3% |
| YUV, mixed content, 1080p | 3.0% | 3.0% | 3.0% | 3.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 129% |  |  |  |
| Dec Time[%] | 109% |  |  |  |

Table 2: Intra lossy coding results with respect to SCM-1.0 for 2CTU Intra Block Copy

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -29.7% | -30.3% | -30.1% |
| RGB, text & graphics with motion,720p | -19.2% | -18.3% | -18.5% |
| RGB, mixed content, 1440p | -5.9% | -5.2% | -5.3% |
| RGB, mixed content, 1080p | -8.7% | -8.8% | -8.5% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -29.0% | -32.0% | -31.2% |
| YUV, text & graphics with motion,720p | -15.4% | -19.6% | -21.6% |
| YUV, mixed content, 1440p | -6.3% | -11.3% | -11.1% |
| YUV, mixed content, 1080p | -9.1% | -16.0% | -15.8% |
| YUV, Animation, 720p | 0.1% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 118% |  |  |
| Dec Time[%] | 117% |  |  |

# Recommendation

It is recommended to adopt this tool for further color palette coding investigation.

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu, “Description of screen content coding technology proposal by Huawei Technologies (USA)”, JCTVC-Q0034, Valencia, Spain, April 2014

- W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu, “AHG8: String match in coding of screen content”, JCTVC-Q0176, Valencia, Spain, April 2014.

# Patent rights declaration(s)

Huawei Technologies (USA) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).