| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0145 |
| --- | --- |

| Title: | SCCE4: Result of Test 4.3.4 for string search for sample coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang , Meng Xu, Zhan Ma , Haoping Yu 2330 Central Expy, Santa Clara, CA 95050 | Tel: Email: | +1 408 330 5142 zhan.ma@huawei.com |
| Source: | Huawei R&D USA |  |  |

_____________________________

# Abstract

It is proposed to perform the 1-D and 2-D string search on pixels by modifying the technologies applied in JCTVC-Q0034/JCTVC-Q0176 [1, 2] for index map coding. It is reported that the proposed method provides bitrate reduction of 9.6% against SCM-1.0 anchor, for lossless coding of 1080p text & graphics content with full frame Intra Block Copy enabled, in all intra configuration, respectively. Meanwhile, the BD-rate gain for lossy coding is -11.4% for the same category using same anchor settings.

# Introduction

JCTVC-Q0034/JCTVC-Q0167 were proposed to encode the color table using packed (interleaved) components together, and code the index map using string match method. In JCTVC-R0144 (SCCE3 Test C.3), the color table derivation, merge, sharing, and DPCM, together with hybrid 1-D and 2-D string search and coding methods in JCTVC-Q0034/JCTVC-Q0167 are implemented on top of SCCE3 palette anchor (SCM-1.0+JCTVC-Q0094+bug-fix of JCTVC-Q0047).  This contribution adopted the coding procedure in JCTVC-R0144 with modifications described to perform the string search in pixel domain and without signaling the color palette or color table.

# Proposed methods

This contribution adopted the coding procedure in JCTVC-R0144 with modifications described below:

- Full CU recursive compatible;

- Skip writing color table to bit-stream except that current CU has only one color

- When coding of unmatched index, instead of writing its RGB value to bit-stream, an adaptive table for unmatched data is created and adp_idx_found_flag is defined.

adp_idx_found_flag is set to 0 to indicate that the pixel value represented by unmatched index is not found in adaptive color table. This flag and the RGB value are written to bit-stream, and the RGB value is added to adaptive table.

adp_idx_found_flag is set to 1 to indicate that the pixel value represented by unmatched index is found in adaptive color table. This flag and the adaptive table index are written to bit-stream.

- 1-D and 2-D string search is conducted upon the reconstructed pixels to the left of current CTU. It uses 3 left CTUs in this proposal.  Residuals between matched string and original string are encoded using HEVC residual engine without change.

# Experimental Results

HM-14.0+RExt-7.0+SCC-1.0 default coding structure is used as anchor, where full frame IBC is enabled. The common test condition specified in [1] is followed. The summarized experimental results are shown in Table 1~6 respectively. More detailed experimental results could be found in the attached spreadsheets. For the simulation performed here, it uses the 3 left CTUs for pixel string search. This search can be extended to improve the performance significantly.

Table 1 Experimental result for lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 9.6% | 9.9% | 4.8% | 14.0% |
| RGB, text & graphics with motion,720p | 10.1% | 7.7% | 0.2% | 21.1% |
| RGB, mixed content, 1440p | 0.8% | 0.8% | 0.5% | 1.1% |
| RGB, mixed content, 1080p | 0.6% | 0.6% | 0.6% | 0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 11.9% | 12.1% | 3.5% | 17.4% |
| YUV, text & graphics with motion,720p | 8.6% | 7.3% | 0.1% | 17.9% |
| YUV, mixed content, 1440p | 0.5% | 0.5% | 0.1% | 1.0% |
| YUV, mixed content, 1080p | 0.5% | 0.5% | 0.5% | 0.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 112% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.9% | 5.0% | 2.7% | 9.3% |
| RGB, text & graphics with motion,720p | 1.4% | 3.5% | 0.1% | 8.1% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.7% | 6.3% | 1.9% | 12.0% |
| YUV, text & graphics with motion,720p | 1.2% | 3.8% | 0.1% | 7.9% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 151% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 2.4% | 3.3% | 1.9% | 5.6% |
| RGB, text & graphics with motion,720p | 0.9% | 2.5% | 0.1% | 5.7% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 2.0% | 4.3% | 1.6% | 7.6% |
| YUV, text & graphics with motion,720p | 0.7% | 2.6% | 0.1% | 5.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 168% |  |  |  |

Table 2 Experimental result for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -11.4% | -11.7% | -11.5% |
| RGB, text & graphics with motion,720p | -6.4% | -5.6% | -5.9% |
| RGB, mixed content, 1440p | -1.7% | -1.4% | -1.3% |
| RGB, mixed content, 1080p | -3.6% | -3.8% | -3.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -11.7% | -13.4% | -12.2% |
| YUV, text & graphics with motion,720p | -5.3% | -7.5% | -9.2% |
| YUV, mixed content, 1440p | -1.2% | -3.4% | -3.3% |
| YUV, mixed content, 1080p | -3.4% | -6.7% | -6.9% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 121% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.9% | -7.3% | -7.3% |
| RGB, text & graphics with motion,720p | -6.5% | -5.3% | -5.8% |
| RGB, mixed content, 1440p | -2.2% | -1.6% | -1.6% |
| RGB, mixed content, 1080p | -4.1% | -4.1% | -3.6% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -7.1% | -9.8% | -9.0% |
| YUV, text & graphics with motion,720p | -5.5% | -7.7% | -9.3% |
| YUV, mixed content, 1440p | -1.5% | -5.2% | -4.8% |
| YUV, mixed content, 1080p | -3.6% | -8.3% | -8.4% |
| YUV, Animation, 720p | 0.1% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 177% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.2% | -4.6% | -4.5% |
| RGB, text & graphics with motion,720p | -4.6% | -3.9% | -4.1% |
| RGB, mixed content, 1440p | -2.1% | -1.8% | -1.6% |
| RGB, mixed content, 1080p | -3.2% | -3.3% | -3.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.7% | -6.0% | -5.5% |
| YUV, text & graphics with motion,720p | -3.4% | -5.5% | -6.6% |
| YUV, mixed content, 1440p | -1.3% | -5.1% | -4.9% |
| YUV, mixed content, 1080p | -1.7% | -7.1% | -5.5% |
| YUV, Animation, 720p | 0.0% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 204% |  |  |

Additional results with respect to the 2CTU IBC are also provided in the attached excel.

# Recommendation

It is recommended to adopt this contribution for the further development of string search technologies.

# References

- JCTVC-Q1015, “Common conditions for screen content coding tests,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- JCTVC-R0144, “SCCE3 Test C.3: Combined Color Table and Index Map Coding of A.4 and B.8” Sapporo, JP, 30 Jun. – 9 Jul. 2014.

- Z. Ma, W. Wang, M. Xu, X. Wang, H. Yu, “Description of screen content coding technology proposal by Huawei Technologies (USA)”, JCTVC-Q0034, Valencia, Spain, April 2014

- W. Wang, Z. Ma, M. Xu, X. Wang, H. Yu, “AHG8: String match in coding of screen content”, JCTVC-Q0176, Valencia, Spain, April 2014.

# Patent rights declaration(s)

Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text Modifications

The modifications is marked as yellow in separated documents.