| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0147_proposed text |
| --- | --- |

| Title: | SCCE5 Test 3.2.1: In-loop color-space transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Jianle Chen Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA Xiaoyu Xiu Yuwen He Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121 | Tel: Email: | + 1-858-651-6660 lizhang@qti.qualcomm.com + 1-858-210-4830 xiaoyu.xiu@interdigital.com |
| Source: | Qualcomm Inc., InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document provides the proposed text. All the changes on top of JCTVC-P1005_v4 are highlighted in green or red strikethrough.

# Syntax and semantics

#### 7.3.2.2	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| … |  |
| if( sps_extension_flag[ 0 ] ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| implicit_rdpcm_enabled_flag | u(1) |
| explicit_rdpcm_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| high_precision_offsets_enabled_flag | u(1) |
| fast_rice_adaptation_enabled_flag | u(1) |
| cabac_bypass_alignment_enabled_flag | u(1) |
| residual_color_space_conversion_enabled_flag | u(1) |
| } |  |
| if( sps_extension_7bits ) |  |
| … |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| } |  |
| } else { |  |
| … |  |
| } |  |
| } else { |  |
| ... |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_color_space_conversion_enabled_flag&& (CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] == 4)) |  |
| cu_residual_csc_flag | ae(v) |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

residual_color_space_conversion_enabled_flag equal to 1 specifies that the residual color space conversion process may be invoked in the decoding process. residual_color_space_conversion_enabled_flag equal to 0 specifies that the residual color space conversion process is not applied. When not present, the value of residual_color_space_conversion_enabled_flag is inferred to be equal to 0. When this flag is equal to 1, chroma_format_idc shall be equal to 3.

cu_residual_csc_flag equal to 1 specifies the residual samples of the current coding unit are represented in the converted color space. cu_residual_csc_flag equal to 0 specifies that the residual samples of the current coding unit are not represented in the converted color space.  When this flag is equal to 1 and one coding unit is intra coded, chroma modes of all prediction units within current coding unit (i.e., intra_chroma_pred_mode) shall be equal to 4.

When cu_residual_csc_flag is not present, it is inferred to be equal to 0.

# Decoding process

### 8.4.1	General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

A variable nCbS is set equal to 1  <<  log2CbSize.

Depending on the values of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- When intra_bc_flag[ xCb ][ yCb ] is equal to 1, the derivation process for block vector components in intra block copying prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and variable log2CbSize as inputs, and the output being bvIntra.

- If cu_residual_csc_flag[ xCb ][ yCb ] is equal to 1, the following applies:

–	For the variable cIdx proceeding over the values 0..2, the following ordered steps apply:

  - Set variable comp equal to (!cIdx ? L : (cIdx = =1 ? Cb : Cr).

  - The general decoding process for intra blocks as specified in subclause  is invoked with the location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable cIdx, and variable controlPara equal to 1 as inputs, and the output is the residual sample array resSamplescomp.

–	The residual modification process for residual blocks using color space conversion as specified in subclause 8.6.7 is invoked with the variable blkSize set equal to nCbS, the (nCbS)x(nCbS) array rY set equal to resSamplesL, the (nCbS)x(nCbS) array rCb set equal to resSamplesCb, and the (nCbS)x(nCbS) array rCr set equal to resSamplesCr as inputs, and the output are modified versions of the (nCbS)x(nCbS) arrays resSamplesL, resSamplesCb and resSamplesCr.

- The general decoding process for intra blocks as specified in subclause  is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, and the variable cIdx set equal to 0, and variable controlPara equal to ( cu_residual_csc_flag[ xCb ][ yCb ] ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- If cu_residual_csc_flag[ xCb ][ yCb ] is equal to 1, the following applies:

- For the variable cIdx proceeding over the values 0..2, the following ordered steps apply:

  - Set variable comp equal to (!cIdx ? L : (cIdx = =1 ? Cb : Cr).

  - The general decoding process for intra blocks as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to 0, the variable cIdx, and variable controlPara set equal to 1 as inputs, and the output is the residual sample array resSamplescomp.

- Set the variable nSubCbS equal to ( nCbS  >> 1 ) and the residual modification process for residual blocks using color space conversion as specified in subclause 8.6.7 is invoked with the variable blkSize set equal to nSubCbS, the (nSubCbS)x(nSubCbS) array rY set equal to resSamplesL, the (nSubCbS)x(nSubCbS) array rCb set equal to resSamplesCb, and the (nSubCbS)x(nSubCbS) array rCr set equal to resSamplesCr as inputs, and the outputs are modified versions of the (nSubCbS)x(nSubCbS) arrays resSamplesL, resSamplesCb and resSamplesCr.

- The general decoding process for intra blocks as specified in subclause  is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to 0, and the variable cIdx set equal to 0 , and variable controlPara set equal to ( cu_residual_csc_flag[ xCb ][ yCb ] ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, and the variable cIdx set equal to 1, and variable controlPara set equal to ( cu_residual_csc_flag[ xCb ][ yCb ] ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, and the variable cIdx set equal to 2 , and variable controlPara set equal to ( cu_residual_csc_flag[ xCb ][ yCb ] ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, and the variable cIdx set equal to 1 , and variable controlPara set equal to ( cu_residual_csc_flag[ xCb ][ yCb ] ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause  is invoked with the chroma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, and the variable cIdx set equal to 2, and variable controlPara set equal to ( cu_residual_csc_flag[ xCb ][ yCb ] ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

#### 8.4.4.1	General decoding process for intra blocks

Inputs to this process are:

–	a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current transform block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable predModeIntraBc specifying the intra block copying mode,

–	a variable bvIntra specifying the intra block copying vector,

–	a variable cIdx specifying the colour component of the current block.

–	a variable controlPara specifying the applicable processes.

Output of this process is a modified reconstructed picture before deblocking filtering when controlPara is unequal to 1, or residual sample array when controlPara is equal to 1.

The luma sample location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC )	(8-4)

The variable splitFlag is derived as follows:

–	If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ].

–	Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

- cIdx is greater than 0

- split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ] is equal to 1

- log2TrafoSize is greater than 2

–	Otherwise, splitFlag is set equal to 0.

Depending on the value of splitFlag, the following applies:

–	If splitFlag is equal to 1, the following ordered steps apply:

- The variables xTb1 and yTb1 are derived as follows:

  - If either cIdx is equal to 0 or ChromaArrayType is not equal to 2, the following applies:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - Otherwise (ChromaArrayType is equal to 2 and cIdx is greater than 0), the following applies:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 2  <<  ( log2TrafoSize − 1 ) ).

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (splitFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( cIdx > 0  &&  ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- The variable nTbS is set equal to 1  <<  log2TrafoSize.

- The variable yTbOffset is set equal to blkIdx * nTbS.

- The variable yTbOffsetY is set equal to yTbOffset * SubHeightC.

- When controlPara is unequal to 2, T the variable residualDpcm is derived as follows:

  - If all of the following conditions are true, residualDpcm is set equal to 1.

    - implicit_rdpcm_enabled_flag is equal to 1.

    - either transform_skip_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ] is equal to 1, or cu_transquant_bypass_flag is equal to 1.

    - either predModeIntra is equal to 10, or predModeIntra is equal to 26.

  - Otherwise, residualDpcm is set equal to explicit_rdpcm_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ].

- When controlPara is unequal to 1, Ddepending upon the value of predModeIntraBc, the following applies:

–	When predModeIntraBc is equal to 0, the general intra sample prediction process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraBc is equal to 1), the intra block copying process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the variable trafoDepth, the variable bvIntra, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

- When controlPara is unequal to 2, Tthe scaling and transformation process as specified in subclause  is invoked with the luma location ( xTbY, yTbY + yTbOffsetY ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

- When controlPara is unequal to 2 and residualDpcm is equal to 1, depending upon the value of predModeIntraBc, the following applies:

–	When predModeIntraBc is equal to 0, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to predModeIntra / 26, the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

–	Otherwise, (predModeIntraBc is equal to 1), the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to explicit_rdpcm_dir_flag[ xTbY ][ yTbY + yTbOffsetY ][ cIdx ], the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- When controlPara is unequal to 2 and cross_component_prediction_enabled_flag is equal to 1, ChromaArrayType is equal to 3, and cIdx is not equal to 0, the residual modification process for transform blocks using cross-component prediction as specified in subclause Error: Reference source not found is invoked with the current luma transform block location ( xTbY, yTbY ), the variable nTbS, the variable cIdx, the (nTbS)x(nTbS) array rY set equal to the corresponding luma residual sample array resSamples of the current transform block, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- When controlPara is unequal to 1, Tthe picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

#### 8.5.4	Decoding process for the residual signal of coding units coded in inter prediction mode

#### 8.5.4.1	General

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array resSamplesL of luma residual samples, where nCbSL is derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array resSamplesCb of chroma residual samples for the component Cb, where nCbSwC and nCbShC are derived as specified below,

–	when ChromaArrayType is not equal to 0, an (nCbSwC)x(nCbShC) array resSamplesCr of chroma residual samples for the component Cr, where nCbSwC and nCbShC are derived as specified below.

…

Depending on the value of rqt_root_cbf, the following applies:

–	If rqt_root_cbf is equal to 0 or skip_flag[ xCb ][ yCb ] is equal to 1, all samples of the (nCbSL)x(nCbSL) array resSamplesL and when ChromaArrayType is not equal to 0, all samples of the two (nCbSwC)x(nCbShC) arrays resSamplesCb and resSamplesCr are set equal to 0.

–	Otherwise (rqt_root_cbf is equal to 1), the following ordered steps apply:

- The decoding process for luma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable nCbS set equal to nCbSL, and the (nCbSL)x(nCbSL) array resSamplesL as inputs, and the output is a modified version of the (nCbSL)x(nCbSL) array resSamplesL.

- When ChromaArrayType is not equal to 0, the decoding process for chroma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 1, the variable nCbSw set equal to nCbSwC, the variable nCbSh set equal to nCbShC, and the (nCbSwC)x(nCbShC) array resSamplesCb as inputs, and the output is a modified version of the (nCbSwC)x(nCbShC) array resSamplesCb.

- When ChromaArrayType is not equal to 0, the decoding process for chroma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 2, the variable nCbSw set equal to nCbSwC, the variable nCbSh set equal to nCbShC, and the (nCbSwC)x(nCbShC) array resSamplesCr as inputs, and the output is a modified version of the (nCbSwC)x(nCbShC) array resSamplesCr.

- When cu_residual_csc_flag[ xCb ][ yCb ] is equal to 1, the residual modification process for residual blocks using color space conversion as specified in subclause 8.6.7 is invoked with the variable blkSize set equal to nCbSL, the (nCbSL)x(nCbSL) array rY set equal to resSamplesL, the (nCbSL)x(nCbSL) array rCb set equal to resSamplesCb, and the (nCbSL)x(nCbSL) array rCr set equal to resSamplesCr as inputs, and the modified arrays resSamplesL, resSamplesCb and resSamplesCr as outputs.

### 8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y + (cu_residual_csc_flag [ xTbY ][ yTbY ] ? -5 : 0)	(8-5)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_csc_flag [ xTbY ][ yTbY ] ? -5 : 0)	(8-6)

–	Otherwise (cIdx is equal to 2),

qP = Qp′C + (cu_residual_csc_flag [ xTbY ][ yTbY ] ? -3 : 0)	(8-7)

### 8.6.7	Residual modification process for transform blocks using color space conversion

This process is only invoked when ChromaArrayType is equal to 3.

Inputs to this process are:

–	a variable blkSize specifying the block size,

–	an (blkSize)x(blkSize) array of luma residual samples rY with elements rY[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCb with elements rCb[ x ][ y ],

–	an (blkSize)x(blkSize) array of chroma residual samples rCr with elements rCr[ x ][ y ].

Outputs of this process are:

–	an modified (blkSize)x(blkSize) array rY of luma residual samples,

–	an modified (blkSize)x(blkSize) array rCb of chroma residual samples,

–	an modified (blkSize)x(blkSize) array rCr of chroma residual samples.

The (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr are modified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize – 1 are modified as follows:

tmp = rY[ x ][ y ] – ( rCb[ x ][ y ]  >>  1 )
rY[ x ][ y ] = tmp + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp – (  rCr[ x ][ y ]  >>  1 )
rCr[ x ][ y ] = rCb[ x ][ y ] + rCr[ x ][ y ]

- Otherwise (cu_transquant_bypass_flag is equal to 0), the (blkSize)x(blkSize) arrays of residual samples rY, rCb and rCr with  x = 0..blkSize − 1, y = 0..blkSize – 1 are modified as follows:

tmp = rY[ x ][ y ] - rCb[ x ][ y ] 
rY[ x ][ y ] = rY[ x ][ y ] + rCb[ x ][ y ]
rCb[ x ][ y ] = tmp – rCr[ x ][ y ]
rCr[ x ][ y ] = tmp + rCr[ x ][ y ]

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 9..11 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
|  | cu_residual_csc_flag | Table 9-XX | 0 | 1 | 2 |

Table 9-XX – Values of initValue for ctxIdx of cu_residual_csc_flag

| Initialization variable | ctxIdx of cu_residual_csc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 154 | 154 | 154 |

Table 9-2 – Syntax elements and associated binarizations

| Syntax structure | Syntax element |  | Binarization |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Process |  | Input parameters |  |  |  |  |  |
| coding_unit( ) | cu_transquant_bypass_flag |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | cu_skip_flag |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | intra_bc_flag |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | pred_mode_flag |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | part_mode |  | Error: Reference source not found |  | ( xCb, yCb ) = ( x0, y0), log2CbSize |  |  |  |  |  |
|  | pcm_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | prev_intra_luma_pred_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | mpm_idx[ ][ ] |  | TR |  | cMax = 2, cRiceParam = 0 |  |  |  |  |  |
|  | rem_intra_luma_pred_mode[ ][ ] |  | FL |  | cMax = 31 |  |  |  |  |  |
|  | intra_chroma_pred_mode[ ][ ] |  | Error: Reference source not found |  | - |  |  |  |  |  |
|  | rqt_root_cbf |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | cu_residual_csc_flag |  | FL |  | cMax = 1 |  |  |  |  |  |
| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |  |  |  |  |
| Syntax element |  | binIdx |  |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 | 3 | 4 | >= 5 |  |
| … |  |  |  |  |  |  |  |  |  |  |
| end_of_slice_segment_flag |  | terminate |  | na |  | na | na | na | na |  |
| rqt_root_cbf |  | 0 |  | na |  | na | na | na | na |  |
| cu_residual_csc_flag |  | 0 |  | na |  | na | na | na | na |  |
| merge_flag[ ][ ] |  | 0 |  | na |  | na | na | na | na |  |