| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0148 |
| --- | --- |

| Title: | SCE1: Crosscheck of test 5.1 results on improved weighted prediction for color gamut scalability (JCTVC-R0239) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking results for JCTVC-R0239. The simulation results reportedly matched those provided by the proponents. It is remarked that using more than 8 regions for inter-layer weighted prediction violates the HEVC version 1 constraint on the maximum number of weighting parameters to be signaled per slice.

# Introduction

In SCE1, using inter-layer weighted prediction (WP) to mimic inter-layer color prediction for color gamut scalability (CGS) is studied. JCTVC-R0239 proposes more accurate estimation of WP parameters by considering luma and chroma values, and also using more ILR pictures in the reference list of the enhancement layer. Different weight and offset values are signaled for each of those pictures. In JCTVC-R0239, up to 32 regions with different weighting are tested. In this crosscheck, the case of 8 regions is tested.

In HEVC version 1, there is a constraint in section 7.4.7.3:

The variable sumWeightL0Flags is derived to be equal to the sum of luma_weight_l0_flag[ i ] + 2 * chroma_weight_l0_flag[ i ], for i = 0..num_ref_idx_l0_active_minus1.

When slice_type is equal to B, the variable sumWeightL1Flags is derived to be equal to the sum of luma_weight_l1_flag[ i ] + 2 * chroma_weight_l1_flag[ i ], for i = 0..num_ref_idx_l1_active_minus1.

It is a requirement of bitstream conformance that, when slice_type is equal to P, sumWeightL0Flags shall be less than or equal to 24, and when slice_type is equal to B, the sum of sumWeightL0Flags and sumWeightL1Flags shall be less than or equal to 24.

It is remarked that it is not an encoder only method when more than 8 regions are used in inter-layer weighted prediction.

# Experimental results

We received the source code from the proponents, implemented in SHM-6.0, and did a code study to verify that the proposed method was implemented as described. We ran simulations according to the SCE 1 description Error: Reference source not found. The results matched what were provided by the proponents and are summarized as follows (timing information is not reliable).

From the results above, it can be observed that inter-layer weighted prediction with 8 regions leads to significant performance loss when compared to SHM-6 with CGS.

On 6/24, the proponent released an update of the code to include the case where all the pictures in RA configuration have the same number of additional ILRs. The update does not affect the performance of AI cases. The related results are summarized as follows (timing information is not reliable). Some improvement is observed.

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-R0239. The implemented algorithm is in line with the proponent’s description, and the simulation results also match those provided by the proponents.

The inter-layer weighted prediction with 8 regions leads to significant performance loss when compared to SHM-6 with CGS on. Moreover, it is not encoder only method when more than 8 regions are used in inter-layer weighted prediction.

# References

- Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li, “HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability”, JCTVC-Q1101, 27 March – 4 April 2014, Valencia, ES.