| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0149_r1 |
| --- | --- |

| Title: | SCE1: Crosscheck of test 5.2 results on color gamut scalability by matrix mapping (JCTVC-R0163) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the crosschecking results for JCTVC-R0163. The simulation results reportedly matched those provided by the proponents. It is remarked that coding loss is observed for sequence TableCar. It is also remarked that the method does not work properly when CGS LUT is restricted to the same bit depth (12 bits) as to SHM-6.

# Introduction

In JCTVC-R0163, non-uniform chroma partitioning and entropy coding method of CGS LUT coefficient are proposed on top of fixed 8x2x2 partitioning.

Different from uniform paritioning, the partition boundary for chroma components are signaled to decoder for each CGS LUT. To determine the optimal partition boundaries for chroma components, two encoder methods are used.

For CGS LUT coefficient coding, each residue is split into three parts: quotient, remainder and sign. Each part is signaled separately.

# Experimental results

We received the source code from the proponents, implemented in SHM-6.0, and did a code study to verify that the proposed method was implemented as described. We ran simulations according to the SCE 1 description Error: Reference source not found. The results matched what were provided by the proponents and are summarized as follows

## Results

The results below were obtained when using the quantized mean values of Cb and Cr components as the chroma partition boundaries. On average, coding improvement was observed over SHM-6 with CGS on. However, for sequence TableCar, up to 1.3% coding loss was noticed in the case of RA-1x.

## Results of advanced encoder optimization

The proponent also provided results with advanced encoder optimization with which the chroma partition boundaries are searched with a range of +/- 1 from the quantized mean values of Cb and Cr components.

Compared to the results in section 2.1, the advanced encoder optimization provides better coding performance. However for sequence TableCar, 0.3%-0.4% coding loss was still observed for AI-1x and RA-1x.

## Results of advanced encoder optimization when LUT coefficients are restricted to 12-bit

It is observed that the proposed method uses higher bit accuracy for CGS LUT coefficient than SHM-6. For fair comparison, in this section the bit-depth of CGS LUT of the proposed method is restricted to 12 bits which is the same as that in SHM-6. It can be observed from the results below that the proposed method does not work properly with 12-bit CGS LUT when base layer is 8-bit.

# Conclusion

In this contribution, we have presented the results of our cross-check of JCTVC-R0163. The implemented algorithm is in line with the proponent’s description, and the simulation results also match those provided by the proponents.

It is remarked that coding loss is observed for sequence TableCar. It is also remarked that the method does not work properly when CGS LUT is restricted to the same bit depth (12 bits) as to SHM-6.

# References

- Duenas, P. Andrivon, E. Alshina, Y. Ye, K. Ugur, X. Li, “HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability”, JCTVC-Q1101, 27 March – 4 April 2014, Valencia, ES.