| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0150 |
| --- | --- |

| Title: | On CGS Signaling Improvement and Constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Jianle Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1 858 658 3923 lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal includes five aspects to optimize and fix the design of colour gamut scalability in SHVC.

- Signal reference layer id for CGS LUT to indicate the reference layer which the specified LUT is applied to.

- Restrict the dynamic range of CGS LUT coefficient to 12 bits in CGS LUT.

- Modify the CGS LUT chroma input and output bit depth signalling to the same way as the existing chroma bit depth signalling mechanism.

- Restrict the input and output bit-depth of CGS module such that the input bit-depth is equal to that of the reference layer and the output bit-depth is not smaller than that of the reference layer or larger than that of the current layer.

- Add a constraint that when CGS is enabled, the weighted prediction for the associated inter-layer reference picture shall be disabled.

# Signaling Reference Layer id for CGS LUT

In current SHVC working draft JCTVC-Q1008, CGS LUT table is signaled in PPS without identifying which reference layer the CGS LUT is applied to. In a multi-layer scenario, one layer may have more than one reference layers. In such cases, it is necessary to signal reference layer id for each CGS LUT.

Proposed spec text:

| colour_mapping_table( ) { | Descriptor |
| --- | --- |
| cm_ref_layer_id | ue(v) |
| cm_octant_depth | u(2) |
| cm_y_part_num_log2 | u(2) |
| cm_input_luma_bit_depth_minus8 | u(3) |
| cm_input_chroma_bit_depth_delta | se(v) |
| cm_output_luma_bit_depth_minus8 | u(3) |
| cm_output_chroma_bit_depth_delta | se(v) |
| cm_res_quant_bits | u(2) |
| colour_mapping_octants( 0, 0, 0, 0, 1 << cm_octant_depth ) |  |
| } |  |

cm_ref_layer_id specifies the nuh_layer_id value of the associated reference layer picture for which the colour mapping table is specified. The value of cm_ref_layer_id shall be less than the nuh_layer_id of any picture that refers to this PPS.

# Restriction on bit-depth of CGS LUT coefficients and input

During SCE study on CGS, bit-depth of CGS LUT was investigated. As shown in JCTVC-Q0021, CGS LUT with 12-bit precision provided as good tradeoff between coding efficiency and complexity. Therefore, it is proposed to add a bitstream conforming constraint such that CGS LUT coefficients shall not have a bit-depth higher than 12 bits.

Proposed spec text:

The value of LutY[ yIdx ][ uIdx ][ vIdx ][ vertex ], LutU[ yIdx ][ uIdx ][ vIdx ][ vertex ], LutV[ yIdx ][ uIdx ][ vIdx ][ vertex ] shall be in the range of −211 to 211 – 1, inclusively.

# Syntax name change on chroma input and output bit-depth

It is proposed to to signal the chroma input and output depth in the same way as the existing chroma bit depth signalling mechanism.

Proposed spec text:

| colour_mapping_table( ) { | Descriptor |
| --- | --- |
| cm_octant_depth | u(2) |
| cm_y_part_num_log2 | u(2) |
| cm_input_luma_bit_depth_minus8 | u(3) |
| cm_input_chroma_bit_depth_deltaminus8 | u(3)se(v) |
| cm_output_luma_bit_depth_minus8 | u(3) |
| cm_output_chroma_bit_depth_deltaminus8 | u(3)se(v) |
| cm_res_quant_bits | u(2) |
| colour_mapping_octants( 0, 0, 0, 0, 1 << cm_octant_depth ) |  |
| } |  |

cm_input_luma_bit_depth_minus8 specifies the sample bit depth of the input luma component of the colour mapping process. The variable CMInputBitDepthY is derived as follows:

CMInputBitDepthY = 8 + cm_input_luma_bit_depth_minus8	(F-1)

cm_input_chroma_bit_depth_deltaminus8 specifies the sample bit depth of the input chroma components of the colour mapping process. The variable CMInputBitDepthC is derived as follows:

CMInputBitDepthC = CMInputBitDepthY8 + cm_input_chroma_bit_depth_deltaminus8	(F-2)

cm_output_luma_bit_depth_minus8 specifies the sample bit depth of the output luma component of the colour mapping process. The variable CMOutputBitDepthY is derived as follows:

CMOutputBitDepthY = 8 + cm_output_luma_bit_depth_minus8	(F-3)

cm_output_chroma_bit_depth_deltaminus8 specifies the sample bit depth of the output chroma components of the colour mapping process. The variable CMOutputBitDepthC is derived as follows:

CMOutputBitDepthC = CMOutputBitDepthY8 + cm_output_chroma_bit_depth_deltaminus8	(F-4)

# Restriction on input and output bit-depth of CGS module

In the current SHVC draft 6 (JCTVC-Q1008), input and output bit-depth information of CGS module is signaled. To avoid potential problem, it is proposed to add restriction such that the input bit-depth shall be equal to that of CGS reference layer and the output bit-depth shall not be smaller than that of the reference layer or larger than that of the current layer.

Proposed spec text:

The value of CMInputBitDepthY shall be equal to the bit depth for the luma component of the reference layer picture and the value of CMInputBitDepthC shall be equal to the bit depth for the chroma component of of the reference layer picture.

The value of CMOutputBitDepthY shall be in the range of the bit depth for the luma component of the reference layer picture to the bit depth for the luma component of any picture that refers to PPS, inclusively. The value of CMOutputBitDepthC shall be in the range of the bit depth for the chroma component of the reference layer picture to the bit depth for the chroma component of any picture that refers to PPS, inclusively.

# Constraint on no inter-layer WP when CGS is enabled

During SCE study on CGS, inter-layer weighted prediction (WP) was used to achieve inter-layer color prediction. Since asymmetric 3D LUT based CGS shows much higher coding efficiency than inter-layer WP, the former was adopted. It is asserted that inter-layer WP does not bring additional benefit on top of 3D LUT based CGS but leads to additional complexity. Therefore, it is proposed to have a bitstream conforming constraint that when CGS is enabled the weighted prediction for the associated inter-layer reference picture shall be disabled.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).