| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0157_v2 |
| --- | --- |

| Title: | Miscellaneous questions and clarifications for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

Miscellaneous questions and issues in need of clarification for SHVC are raised for consideration.

- What profile_idc value should be used for the Monochrome 8 profile?

- Should cm_octant depth in the colour mapping table be reduced from 2 bits to 1?

- Constraint needed to restrict that DependencyId and ViewOrderIdx must increase with increasing values of nuh_layer_id

- Should a limit be imposed on how many CGS 3D LUT tables must be stored?

The v2 version of this document provides a revision to the 3rd proposal based upon a suggestion made during BoG discussion, modifying the constraint on the values of DependencyId and ViewOrderIdx for equal values of AuxId, rather than just for zero values of AuxId.

# Issues

## What profile_idc should be used for the Monochrome 8 profile?

Should the RExt profile_idc be used, or a separate profile_idc value allocated?

## Should cm_octant depth in the colour mapping table be reduced from 2 bits to 1?

Currently cm_octang_depth uses 2 bits.  Its value is constrained to be in the range of 0..1, so a single bit flag could be used instead with no difference in current behavior, but future extensibility may be affected.

## Constraint needed to restrict that DependencyId and ViewOrderIdx must increase with increasing values of nuh_layer_id

In the current specification, there is a constraint missing to impose the design intent restricting that values of DependencyId and ViewId must increase with increasing values of nuh_layer_id.

Proposed language to modify in the Scalable Main, Scalable Main 10 profile constraints:

When AuxId[ i ] and AuxId[ j ] are both equal to 0 and i is not equal to j  i is greater than j, DependencyId[ i ] shall not be equal to be greater than DependencyId[ j ]

Proposed language to modify in the Stereo Main profile constraints:

When AuxId[ i ] and AuxId[ j ] are both equal to 0 and i is greater than j, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx [ j ]

Alternative constraints were suggested during discussion during the BoG presentation, as follows:

Proposed language to modify in the Scalable Main, Scalable Main 10 profile constraints:

When AuxId[ i ] equal to AuxId[ j ] and i is not equal to j  i is greater than j, DependencyId[ i ] shall not be equal to be greater than DependencyId[ j ]

Proposed language to modify in the Stereo Main profile constraints:

When AuxId[ i ] equal to AuxId[ j ] and i is greater than j, ViewOrderIdx[ i ] shall be greater than ViewOrderIdx [ j ]

## Should a limit be imposed on how many CGS 3D LUT tables must be stored?

The colour mapping 3D LUT is signaled in the PPS.  Each 3D LUT may require up to 384*12 bits.  There may be up to 64 picture parameter sets.  In the worst case, a significant amount of 3D LUT storage is required, 64 * 384 * 12 bits.

It is proposed to allow only 8 picture parameter sets containing 3D LUTs, in order to limit the storage requirement.

Proposed language:

colour_mapping_enabled_flag equal to 1 specifies the colour mapping process is applied to the inter-layer reference pictures for the coded pictures referring to the PPS. colour_mapping_enabled_flag equal to 0 specifies that the colour mapping process is not applied to the inter-layer reference pictures for the coded pictures referring to the PPS. When not present, colour_mapping_enabled_flag is inferred to be equal to 0.

When pps_pic_parameter_set_id greater than or equal to 8, colour_mapping_enabled_flag shall be equal to 0.

# Patent rights declaration(s)

Vidyo, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.