| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0158 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3 – 9 July 2014 | Document: JCT3V-I0039 |
| --- | --- |

| Title: | Modifications to Scalable Main profile and general and profile-specific level limits |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill@vidyo.com |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

Modifications are proposed to the Scalable Main profile, and to the Scalable Main profile-specific level limits.  Modifications are also proposed to the general level limits for the layered extensions.

The proposed Scalable Main profile includes the following modified constraints:

- Total number of layers in the Output Layer Set (OLS) shall be less than or equal to 4

- Total number of layers in a chain of dependencies shall be less than or equal to 3, expressed by limiting the number of direct or indirect reference layers for the current layer to 2

The constraints of the general and profile-specific level limits are unchanged from the design intent of JCTVC-Q1008, but have some editorial improvements to express the limits:

- Level constraints apply individually to each layer, including picture size, sample rate, bit rate

- Level constraints apply collectively to the OLS, with 2x values compared to the single layer constraints for picture size, sample rate, bit rate

# Discussion

The definition of Scalable Main profile in JCTVC-Q1008 allows a chain of dependent layers with up to 8 layers, and has no explicit limit on the total number of layers, beyond an inherent limit in the number of layers which can be signaled, which is 63.

Further restriction will constrain complexity while addressing many use cases of interest.  In AVC’s SVC profiles, there was a limit of 3 layers with dependencies.  A similar limit of 3 layers of in a chain of dependencies is proposed to also be used for Scalable Main profile.

A limit of 4 total layers in an OLS conforming to Scalable Main profile is also proposed.

New language is also provided to express level limits that are general to the layered extensions and Scalable main profile-specific, without changing the design intent from JCTVC-Q1008.  Level limits are applied on a per layer basis, and on a per OLS basis.  The motivation for the per layer limits is to ensure that no one layer in a multi-layer decode is more difficult to decode than a version 1 single layer decoder with the same value of level_idc would allow.  Some editorial improvement is still required to express the per layer bitrate limits.

# Proposed text

Bitstreams containing OLSs conforming to the Scalable Main or Scalable Main 10 profiles shall obey the following constraints on a derived sub-bitstream for each of the OLSs indicated to conform to the Scalable Main or Scalable Main 10 profile, respectively, with layerSetIdx being the layer set for the OLS, and the sub-bitstream being derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with tIdTarget equal to 6 and with layerIdListTarget containing the nuh_layer_id values of the layers that are included in the layer set with the index layerSetIdx and are primary picture layers.

Bitstreams containing OLSs conforming to the Scalable Main or Scalable Main 10 profiles shall also obey the following constraints on the base layer bitstream derived by invoking the sub-bitstream extraction process as specified in subclause Error: Reference source not found with tIdTarget equal to 6 and with layerIdListTarget containing only one nuh_layer_id value that is equal to 0 as inputs.

The base layer bitstream derived from bitstreams conforming to the Scalable Main profile shall obey the following constraints:

–	The base layer bitstream shall obey all constraints of the Main profile specified in subclause A.3.2.

–	SPSs of the base layer bitstream shall have general_profile_idc equal to 1 or general_profile_compatibility_flag[ 1 ] equal to 1.

The base layer bitstream derived from bitstreams conforming to the Scalable Main 10 profile shall obey the following constraints:

–	The base layer bitstream shall obey all constraints of the Main 10 profile specified in subclause A.3.3.

–	SPSs of the base layer bitstream shall have general_profile_idc equal to 1 or 2, or general_profile_compatibility_flag[ 1 ] or general_profile_compatibility_flag[ 2 ] equal to 1.

The derived sub-bitstream for an OLS conforming to the Scalable Main or Scalable Main 10 profiles shall obey the following constraints:

–	All active SPSs for the sub-bitstream shall have chroma_format_idc equal to 1 only.

–	CtbLog2SizeY derived from any active SPS for the sub-bitstream shall be in the range of 4 to 6, inclusive.

–	The variables NumResampling, NumSampleResampling, and NumMotionResampling shall be less than or equal to 1 for each decoded picture with nuh_layer_id included in layerIdListTarget that was used to derive the sub-bitstream.

–	ScalabilityId[ j ][ smIdx ] shall be equal to 0 for any smIdx value not equal to 2 and for any value of j such that layer_id_in_nuh[ j ] is among layerIdListTarget that was used to derive the sub-bitstream.

–	For a layer with layer id iNuhLId equal to any of nuh_layer_id included in layerIdListTarget that was used to derive the sub-bitstream, the value of NumRefLayers[ iNuhLId ], which specifies the total number of direct and indirect dependent layers and is derived as in F.7.4.3.1, shall be less than or equal to 2.

–	NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] shall be less than or equal to 4

–	All active SPSs shall have sps_extension_type_flag[ i ] equal to 0 only for i equal to 0, and in the range of 2 to 6, inclusive.

–	All active PPSs shall have pps_extension_type_flag[ i ] equal to 0 only for i in the range of 0 to 6, inclusive.

–	DependencyId[ i ] shall not be equal to DependencyId[ j ] for any values of i and j among layerIdListTarget that was used to derive the sub-bitstream such that i is not equal to j.

The derived sub-bitstream for an OLS conforming to the Scalable Main profile shall obey the following constraints:

–	All active SPSs for the sub-bitstream shall have bit_depth_luma_minus8 equal to 0 only.

–	All active SPSs for the sub-bitstream shall have bit_depth_chroma_minus8 equal to 0 only.

–	All active PPSs for the sub-bitstream shall have colour_mapping_enabled_flag equal to 0 only.

The derived sub-bitstream for an OLS conforming to the Scalable Main 10 profile shall obey the following constraints:

–	All active SPSs for the sub-bitstream shall have bit_depth_luma_minus8 in the range of 0 to 2, inclusive.

–	All active SPSs for the sub-bitstream shall have bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

      - General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 is considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this subclause, subclause G.11.2.2, and subclause G.11.3:

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0 (i.e. the 0-th access unit).

- Let the variable fR be set equal to 1  300.

- Let the variable olsIdx be the index of the OLS

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- For each OLS, let the variable olsSizeInSamplesY be derived as follows:

olsSizeInSamplesY = 0		(H-1)
for( i = 0; i < NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ]; i++ )
	if( layer_id_included_flag[ OlsIdxToLsIdx[ olsIdx ]  ][  i ] )
		olsSizeInSamplesY  +=  pic_width_vps_in_luma_samples * pic_height_vps_in_luma_samples

where pic_width_vps_in_luma_samples and pic_height_vps_in_luma_samples are found, for each value of i, in the vps_rep_format_idx[ LayerIdxInVps[ LayerSetLayerIdList[ OlsIdxToLsIdx[ olsIdx ] ][ i ] ] ]-th rep_format( ) syntax structure in the VPS.

Each OLS conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found, where "access unit" is used to denote the set of pictures in the layers contained in the OLS and with the same output time, along with their associated non-VCL NAL units, and the CPB is understood to be the BPB:

- For each layer with nuh_layer_id equal to currLayerId in the OLS, PicSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Error: Reference source not found

- For each layer with nuh_layer_id equal to currLayerId in the OLS, the value of pic_width_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- For each layer with nuh_layer_id equal to currLayerId in the OLS, the value of pic_height_vps_in_luma_samples of the vps_rep_format_idx[ LayerIdxInVps[ currLayerId ] ]-th rep_format( ) syntax structure in the VPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- For each layer with nuh_layer_id equal to currLayerId in the OLS, the value of max_vps_dec_pic_buffering_minus1[ olsIdx ][ LayerIdxInVps[ currLayerId ] ][ HighestTid ] shall be less than or equal to MaxDpbSize as derived by Equation A-2, with PicSizeInSamplesY being replaced with olsSizeInSamplesY, for the tier and level of the OLS.

- For level 5 and higher levels, the value of CtbSizeY for each layer in the OLS shall be equal to 32 or 64.

- The value of NumPocTotalCurr shall be less than or equal to 8.

- When decoding each coded picture in the OLS, the value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Error: Reference source not found for the tier and level of the OLS.

- For the VCL HRD parameters of the OLS, CpbSize[ i ] shall be less than or equal to 1000 * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxCPB is specified in Error: Reference source not found for the tier and level of the OLS in units of 1000 bits.

- For the NAL HRD parameters of the OLS, CpbSize[ i ] shall be less than or equal to 1100 * MaxCPB for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where CpbSize[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxCPB is specified in Error: Reference source not found for the tier and level of the OLS in units of 1100 bits.

- The value of olsSizeInSamplesY shall be less than or equal to 2 * MaxLumaPs, where MaxLumaPs is specified in Error: Reference source not found for the tier and level of the OLS.

Error: Reference source not found specifies the limits for each level of each tier.

A tier and level to which a OLS conforms are indicated by the syntax elements general_tier_flag and general_level_idc as follows:

–	general_tier_flag equal to 0 indicates conformance to the Main tier, and general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Error: Reference source not found. general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Error: Reference source not found marked with "-").

–	general_level_idc shall be set equal to a value of 30 times the level number specified in Error: Reference source not found.

      - Profile-specific level limits for the Scalable Main and Scalable Main 10 profiles

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

Each OLS conforming to the Scalable Main or Scalable Main 10 profiles at a specified tier and level shall obey the following constraints for each conformance test as specified in Annex Error: Reference source not found, where "access unit" is used to denote the set of pictures in the layers contained in the OLS and with the same output time, along with their associated non-VCL NAL units, and the CPB is understood to be the BPB:

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( PicSizeInSamplesY  MaxLumaSr, fR ), where PicSizeInSamplesY is the value of PicSizeInSamplesY for access unit n − 1 and MaxLumaSr is the value specified in Error: Reference source not found that applies to access unit n – 1 for the tier and level of the OLS.

- The difference between consecutive output times in different access units, as specified in subclause Error: Reference source not found, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( olsSizeInSamplesY  MaxLumaSr, fR ), where olsSizeInSamplesY is the value of olsSizeInSamplesY of access unit n and MaxLumaSr is the value specified in Error: Reference source not found for access unit n for the tier and level of the OLS, provided that access unit n is an access unit that has a picture that is output and is not the last of such access units.

- The removal time of access unit 0 shall satisfy the constraint that the number of slice segments in picture 0 is less than or equal to Min( MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * PicSizeInSamplesY / MaxLumaPs, MaxSliceSegmentsPerPicture ), for the value of PicSizeInSamplesY of picture 0, where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Error: Reference source not found and Error: Reference source not found, respectively, that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in picture n is less than or equal to Min( MaxSliceSegmentsPerPicture * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Error: Reference source not found and Error: Reference source not found, respectively, that apply to picture n.

- [Ed.: An expression is needed to constrain the per layer VCL bitrate.

- [Ed.: An expression is needed to constrain the per layer bitrate. ]

- The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to 1.5 * ( Max( PicSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of PicSizeInSamplesY of picture 0, where MaxLumaSr and MinCr are the values specified in Error: Reference source not found that apply to picture 0.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to 1.5 * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and MinCr are the values specified in Error: Reference source not found that apply to picture n.

- The removal time of access unit 0 shall satisfy the constraint that the number of tiles in picture 0 is less than or equal to Min( MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs, MaxTileCols * MaxTileRows ), for the value of PicSizeInSamplesY of picture 0, where MaxTileCols and MaxTileRows are the values specified in Error: Reference source not found that apply to picture 0.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in picture n is less than or equal to Min( MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Error: Reference source not found that apply to picture n.

- The nominal removal time of access unit n (with n greater than 0) from the CPB, as specified in subclause Error: Reference source not found, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( olsSizeInSamplesY  (2* MaxLumaSr), fR ), where olsSizeInSamplesY is the value of olsSizeInSamplesY for access unit n − 1 and MaxLumaSr is the value specified in Error: Reference source not found that applies to access unit n – 1 for the tier and level of the OLS.

- The difference between consecutive output times in different access units, as specified in subclause Error: Reference source not found, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( olsSizeInSamplesY  (2 * MaxLumaSr), fR ), where olsSizeInSamplesY is the value of olsSizeInSamplesY of access unit n and MaxLumaSr is the value specified in Error: Reference source not found for access unit n for the tier and level of the OLS, provided that access unit n is an access unit that has a picture that is output and is not the last of such access units.

- The removal time of access unit 0 shall satisfy the constraint that the number of coded slice segments in access unit 0 is less than or equal to Min( Max NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ], MaxSliceSegmentsPerPicture * 2 * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSliceSegmentsPerPicture * olsSizeInSamplesY / MaxLumaPs ), MaxSliceSegmentsPerPicture ), for the value of olsSizeInSamplesY of access unit 0, where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Error: Reference source not found and Error: Reference source not found, respectively, for the tier and level of the OLS.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slice segments in picture n is less than or equal to Min( (Max( NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ] MaxSliceSegmentsPerPicture * 2 * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSliceSegmentsPerPicture ), where MaxSliceSegmentsPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Error: Reference source not found and Error: Reference source not found, respectively, that apply to access unit n for the tier and level of the OLS.

- For the VCL HRD parameters for the OLS, BitRate[ i ] shall be less than or equal to CpbBrVclFactor * 2 * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Error: Reference source not found in units of CpbBrVclFactor bits/s, where CpbBrVclFactor is specified in Error: Reference source not found for the tier and level of the OLS.

- For the NAL HRD parameters for the OLS, BitRate[ i ] shall be less than or equal to CpbBrNalFactor * 2 * MaxBR for at least one value of i in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where BitRate[ i ] is specified in subclause Error: Reference source not found based on parameters selected as specified in subclause Error: Reference source not found and MaxBR is specified in Error: Reference source not found in units of CpbBrNalFactor bits/s, where CpbBrNallFactor is specified in Error: Reference source not found for the tier and level of the OLS.

- The sum of the NumBytesInNalUnit variables for access unit 0 shall be less than or equal to FormatCapabilityFactor * ( Max( olsSizeInSamplesY, fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr for the value of olsSizeInSamplesY of access unit 0, where MaxLumaSr and MinCr are the values specified in Error: Reference source not found and FormatCapabilityFactor is the value specified in Error: Reference source not found that apply to access unit 0 for the tier and level of the OLS.

- The sum of the NumBytesInNalUnit variables for access unit n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and MinCr are the values specified in Error: Reference source not found and FormatCapabilityFactor is the value specified in Error: Reference source not found that apply to access unit n for the tier and level of the OLS.

- The removal time of access unit 0 shall satisfy the constraint that the number of tiles in coded pictures in access unit 0 is less than or equal to Min( Max( NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ], 2 * MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + 2 * MaxTileCols * MaxTileRows * PicSizeInSamplesY / MaxLumaPs ), 2 * MaxTileCols * MaxTileRows ), for the value of olsSizeInSamplesY of access unit 0, where MaxTileCols and MaxTileRows are the values specified in Error: Reference source not found that apply to access unit 0 for the tier and level of the OLS.

- The difference between consecutive CPB removal times of access units n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in coded pictures in access unit n is less than or equal to Min( Max( NumLayersInIdList[ OlsIdxToLsIdx[ olsIdx ] ], 2 * MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), 2 * MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Error: Reference source not found that apply to access unit  n for the tier and level of the OLS.

# Patent rights declaration(s)

Vidyo, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.