| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0160 |
| --- | --- |

| Title: | Crosscheck of JCTVC-R0058 SCCE3 Test D.1: Single color intra mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139, U.S.A | Tel: Email: | +1-617-621-7500 xzhang@merl.com cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document reports results from a crosscheck of JCTVC-R0058, SCCE3 Test D.1: Single color intra mode for screen content coding. The source code provided by the proponent was briefly checked and appeared to be consistent with the description in JCTVC-Q0093. The modifications to the text specification appear to be consistent with the proposed method. The BD-rate performance and encoding run times are consistent with the results from the proposal. Decoding times reported for the crosscheck may vary due to differences in computing platforms.

# Introduction

SCCE3 Test D1 tests a technique named Single Color Mode, which was proposed in JCTVC-Q0093 [1]. Single Color Mode predicts and reconstructs a CU as a smooth area with single color value. A flag at slice-level is sent to indicate enabling or disabling the tool in the current slice. A flag at CU-level is sent to indicate the usage of single color mode for the current CU. If the current CU uses single color mode, then a candidate index is sent to indicate the selected single color value from a candidate lists.

# Experimental Results

The solution is built using GCC 4.8.2. The SCCE3 test conditions from JCTVC-Q1123 [3] were used for these tests. In the software implementation of JCTVC-R0058 SCCE3 Test D1 [2], the added tool is enabled by the following macro in the software:

- #define  MTK_SINGLE_SAMPLE_MODE        1

In the reflector of SCCE3, participants agreed to add a bug-fix change to prevent memory leak when PLT mode is on. This bug-fix modification is added to both the anchor and the tested method. There is no performance change with or without this bug-fix for both the bug and the tested tool.

## Lossy coding results

### SCM-1.0 common test condition with intraBC using full frame search

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.8% | -0.8% | -0.8% | -0.2% | -0.2% | -0.3% | 0.0% | 0.0% | 0.1% |
| RGB, text & graphics with motion,720p | -1.3% | -1.4% | -1.4% | -0.9% | -0.8% | -0.8% | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.6% | -0.6% | -0.6% | 0.0% | 0.0% | 0.0% | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% | -0.3% | -0.4% | -0.4% | 0.1% | 0.3% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% | -0.2% | -0.3% | -0.4% | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | -1.4% | -1.5% | -1.6% | -0.9% | -0.9% | -0.9% | -0.3% | -0.3% | -0.7% |
| YUV, mixed content, 1440p | -0.6% | -0.7% | -0.7% | 0.1% | -0.3% | -0.4% | -0.1% | -0.4% | -0.6% |
| YUV, mixed content, 1080p | -0.6% | -0.6% | -0.6% | -0.3% | -0.5% | -0.4% | 0.1% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% | 0.1% | -0.2% | 0.2% | 0.0% | -0.3% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.1% | 0.0% | -0.4% | -0.3% |
| Enc Time[%] | 99% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  | 101% |  |  |

### SCM-1.0 common test condition with intraBC using full frame search + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.6% | -0.2% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| RGB, text & graphics with motion,720p | -1.1% | -1.1% | -1.1% | -0.6% | -0.6% | -0.6% | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | -0.6% | -0.5% | -0.6% | 0.2% | 0.2% | 0.1% | -0.1% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.5% | -0.3% | -0.3% | -0.2% | 0.0% | 0.1% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.5% | -0.2% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -1.1% | -1.3% | -1.2% | -0.7% | -0.8% | -0.8% | -0.3% | -0.6% | -0.6% |
| YUV, mixed content, 1440p | -0.6% | -0.5% | -0.4% | 0.1% | 0.0% | -0.2% | -0.2% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.4% | -0.3% | -0.4% | -0.2% | -0.1% | -0.2% | 0.2% | -0.9% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% | 0.1% | -0.1% | 0.1% | -0.1% | -0.4% | -0.5% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 99% |  |  | 101% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  | 100% |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.0% | -1.0% | -0.3% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | -1.4% | -1.4% | -1.4% | -0.9% | -0.9% | -0.9% | -0.3% | -0.3% | -0.1% |
| RGB, mixed content, 1440p | -0.7% | -0.6% | -0.7% | -0.1% | 0.0% | -0.1% | -0.2% | -0.3% | -0.1% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% | -0.4% | -0.4% | -0.4% | 0.0% | 0.0% | -0.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.9% | -0.9% | -0.9% | -0.4% | -0.4% | -0.3% | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | -1.4% | -1.5% | -1.6% | -0.9% | -1.1% | -1.0% | -0.4% | -0.5% | -0.8% |
| YUV, mixed content, 1440p | -0.6% | -0.8% | -0.8% | 0.1% | -0.3% | -0.1% | -0.3% | -0.5% | -0.3% |
| YUV, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.4% | -0.3% | -0.6% | -0.3% | 0.5% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% | 0.1% | 0.0% | 0.1% | 0.0% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 101% |  |  | 101% |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU) + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% | -0.2% | -0.1% | -0.1% | 0.1% | 0.1% | 0.1% |
| RGB, text & graphics with motion,720p | -0.9% | -0.9% | -0.9% | -0.5% | -0.5% | -0.5% | 0.0% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.6% | -0.5% | -0.6% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.5% | -0.3% | -0.3% | -0.3% | 0.2% | 0.0% | 0.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.6% | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | -1.0% | -1.1% | -1.0% | -0.6% | -0.6% | -0.6% | -0.2% | -0.4% | -0.2% |
| YUV, mixed content, 1440p | -0.6% | -0.6% | -0.5% | 0.0% | -0.1% | -0.1% | -0.2% | -0.4% | -0.5% |
| YUV, mixed content, 1080p | -0.5% | -0.5% | -0.5% | -0.3% | -0.4% | -0.4% | 0.0% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.2% | 0.1% | -0.1% | -0.2% | 0.1% | -0.2% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  | 100% |  |  |

## Lossless coding results

### 2.2.1 SCM-1.0 common test condition with intraBC using full frame search

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.3% |  | 0.3% |  | 0.1% |  | 0.4% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.3% |  | 0.3% |  | 0.1% |  | 0.7% |  | 0.0% |  | 0.2% |  | -0.1% |  | 0.6% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| RGB, mixed content, 1440p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.4% |  | 0.3% |  | 0.1% |  | 0.5% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion,720p |  |  | 0.3% |  | 0.4% |  | 0.1% |  | 1.0% |  | 0.1% |  | 0.3% |  | 0.0% |  | 0.8% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.2% |
| YUV, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 100% |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 94% |  |  |  |  |  |  |  | 100% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |

### SCM-1.0 common test condition with intraBC using full frame search + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.3% |  | 0.2% |  | 0.0% |  | 0.3% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.5% |  | 0.0% |  | 0.1% |  | -0.1% |  | 0.4% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.3% |  | 0.3% |  | 0.1% |  | 0.4% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion,720p |  |  | 0.2% |  | 0.3% |  | 0.1% |  | 0.7% |  | 0.0% |  | 0.2% |  | 0.0% |  | 0.6% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| YUV, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 97% |  |  |  |  |  |  |  | 102% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 98% |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU)

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.4% |  | 0.4% |  | 0.2% |  | 0.6% |  | 0.0% |  | 0.1% |  | -0.1% |  | 0.4% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.3% |  | 0.4% |  | 0.1% |  | 1.0% |  | 0.1% |  | 0.2% |  | 0.0% |  | 0.8% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.1% |
| RGB, mixed content, 1440p |  |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.4% |  | 0.4% |  | 0.2% |  | 0.6% |  | 0.0% |  | 0.2% |  | 0.0% |  | 0.4% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| YUV, text & graphics with motion,720p |  |  | 0.3% |  | 0.5% |  | 0.1% |  | 1.2% |  | 0.1% |  | 0.3% |  | 0.0% |  | 0.9% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.2% |
| YUV, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 100% |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |  | 105% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 95% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |  | 102% |  |  |  |  |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU) + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.2% |  | 0.2% |  | 0.1% |  | 0.3% |  | -0.1% |  | 0.0% |  | -0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.3% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1440p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.2% |  | 0.2% |  | 0.0% |  | 0.4% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| YUV, text & graphics with motion,720p |  |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.3% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |
| YUV, mixed content, 1440p |  |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 99% |  |  |  |  |  |  |  | 102% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 99% |  |  |  |  |  |  |  | 100% |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |

# Conclusions

It is confirmed that all the rate-distortion data generated for the cross-check match the results released by the proponents in SCCE3. The source code and the text specification seem to match the description in JCTVC-Q0093.

# References

- Y.-W Chen, Y.-C Sun, Y.-W Huang, S. Lei, “Single color intra mode for screen content coding,” JCTVC-Q0093, Valencia, ES, 27 March – 4 April, 2014.

- Y.-W. Chen, Y.-C. Sun, Y.-W. Huang, S. Lei, “SCCE3 Test D.1: Single color intra mode for screen content coding,” JCTVC-R0058, Sapporo, JP, 30 June – 9 July, 2014.

- Y.-W Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” JCTVC-Q1123, Valencia, ES, 27 March – 4 April, 2014.