| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0161 |
| --- | --- |

| Title: | Non-SCCE3: Simplified palette size coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao, Seung-Hwan Kim, Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 jzhao@sharplabs.com kimse@sharplabs.com misrak@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp |  |  |

_____________________________

# Abstract

This contribution proposes to change the binarization of palette_num_signalled_entries to fixed length code. The proposed method is simpler than the existing Truncated Unary (TR) code, and reduces the worst case number of coded bins from 32 bins to 5 bins in the existing palette coding structure. It has been also observed that the proposed method provides coding performance benefit of  0.1% and 0.2% for  lossy coding and lossless coding, respectively.

# Introduction

In JCTVC palette coding working text, the palette size consists of number of entries reused from previous palette table “numPredPreviousPalette” and number of signaled entries “palette_num_signalled_entries”.

| palette_coding_component( x0, y0, CbWidth, CbHeight, NumComp ) { | Descriptor |
| --- | --- |
| compOffset = ( NumComp = = 3 ) ? 0 : ( NumComp – 1 ) |  |
| nCbS = ( 1 << log2CbSize ) |  |
| numPredPreviousPalette = 0 |  |
| for( i = 0; i < previousPaletteSize; i++ ) { |  |
| previous_palette_entry_flag[ i ] | ae(v) |
| if ( previous_palette_entry_flag[ i ] ) { |  |
| for ( cIdx = compOffset; cIdx < NumComp + compOffset; cIdx++ ) |  |
| palette_entries[ cIdx ][ numPredPreviousPalette ] = previousPaletteEntries[ cIdx ][ i ] |  |
| numPredPreviousPalette++ |  |
| } |  |
| } |  |
| if( numPredPreviousPalette < max_palette_size) |  |
| palette_num_signalled_entries | ae(v) |

palette_num_signalled_entries specifies the number of entries in the palette that are explicitly signalled for the current coding unit.

As shown in Table 9-34 in the draft text specification for palette coding, the binarization of “palette_num_signalled_entries” is using truncated unary (TR) code with cMax=max_pallete_size and cRiceRaram=0. In worst case, palette_num_signalled_entries is the allowed maximum palette size, currently 31,and coding palette_num_signalled_entries takes 32 bins.

Table 9-1 – Syntax elements and associated binarizations

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_num_signalled_entries | TR | cMax = max_palette_size, cRiceParam = 0 |

# Proposed

## In this test, instead of TR binarization, we propose fixed length binarization for “palette_num_signalled_entries.”. Considering the maximum palette size in current palette structre is 31, proposed FLC only requires 5 bins instead of the 32 bins when using TR code in the worst case.

## In addition, in the aspect of complexity, fixed length code is simpler, removing the conditional checking required for TR code. Experiments show that with fixed length code, there is no performance loss but small coding gain.

No change to the syntax table and semantics. The only change is changing the binarization to use fixed length code FL instead of TR.

Table 9-2 – Syntax elements and associated binarizations

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_num_signalled_entries | FL | cMax = 31 |

# Coding efficiency results

The experimental conditions are performed following the screen content coding test conditions [2] and SCCE3 description [3] .

Full results are provided in the attached Excel sheets. Table 1 and Table 2 summarize lossy coding results with respect to the SCCE3 palette anchor for full frame Intra Block Copy and 2CTU search ranges, respectively. Table 3 and Table 4 summarize lossless coding results with respect to the SCCE3 palette anchor for full Intra Block Copy and 2CTU search ranges, respectively.

On average there are about 0.1% rate reduction for lossy cases, and up to up to 0.2% rate reduction for lossless cases.

Table 1: Results with respect to the SCCE3 palette anchor for full frame Intra Block Copy (Lossy)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & grap. with mot,1080p | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & grap. with mot,720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot, 1080p | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |
| YUV, text & grap. with mot.,720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | -0.1% | -0.3% | -1.6% | -1.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 2: Results with respect to the SCCE3 palette anchor for 2 CTU Intra Block Copy search (Lossy)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & grap. with mot,1080p | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |
| RGB, text & grap. with mot.,720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot, 1080p | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| YUV, text & grap. with mot.,720p | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.2% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.5% |
| YUV, mixed content, 1080p | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.9% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 3: Results with respect to the SCCE3 palette anchor for full frame Intra Block Copy (Lossless)

|  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate Savings |  |  |  |  |  |  |  | Bit-rate Savings |  |  |  | Bit-rate Savings |  |  |  |
|  | Total |  | Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Min |  |  | Max |  | Total | Ave. | Min | Max | Total | Ave. | Min | Max |
| RGB, T & G. with mot.,1080p | 0.2% | 0.2% |  |  | 0.1% | 0.4% |  |  | 0.1% | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, T & G. w/mot.,720p | 0.1% | 0.1% |  |  | 0.0% | 0.3% |  |  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera cap., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV T & G. w/mot., 1080p | 0.2% | 0.2% |  |  | 0.1% | 0.3% |  |  | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, T & G. w/mot.,720p | 0.1% | 0.1% |  |  | 0.0% | 0.3% |  |  | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 4: Results with respect to the SCCE3 palette anchor for 2CTU Intra Block Copy (Lossless)

|  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate Savings |  |  |  |  |  |  |  | Bit-rate Savings |  |  |  | Bit-rate Savings |  |  |  |
|  | Total |  | Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Min |  |  | Max |  | Total | Ave. | Min | Max | Total | Ave. | Min | Max |
| RGB, T & G. with mot.,1080p | 0.2% | 0.2% |  |  | 0.0% | 0.3% |  |  | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, T & G. w/mot.,720p | 0.2% | 0.1% |  |  | 0.0% | 0.3% |  |  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.1% | 0.1% |  |  | 0.1% | 0.1% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.1% | 0.1% |  |  | 0.1% | 0.1% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera cap., 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV T & G. w/mot., 1080p | 0.2% | 0.2% |  |  | 0.1% | 0.3% |  |  | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| YUV, T & G. w/mot.,720p | 0.1% | 0.1% |  |  | 0.0% | 0.3% |  |  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% |  |  | 0.0% | 0.1% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.1% | 0.1% |  |  | 0.1% | 0.1% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  |  | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

# Conclusion

In conclusion, we propose to change the binarization of “palette_num_signalled_entries” to FLC instead of the current TR code. This does not change the current syntax and semantics. It only changes the binarization table. This FLC binarization is not only simple but reduces the worst case coded bins significantly (from 32 bins to 5 bins). It also provides coding performance benefit for both lossy and lossless. Therefore we propose to have it adopt into Screen content coding draft specifications.

# Reference

[1] “Text specification for palette mode used as a basis in screen content coding core experiment 3 (SCCE3)”. Distributed on SCCE3 mailing list, 6/17/2014

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-Q1015, Apr. 2014.

[3] Y.W Huang, R. Onno,R. Joshi, R. Cohen, X. Xiu, and Z. Ma, “Description of Screen Content Core Experiment 3 (SCCE3): Palette mode”. JCTVC-Q1123,  Apr. 2014.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).