| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0163 |
| --- | --- |

| Title: | SCE1: Test 5.2 results on color gamut scalability by matrix mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
|  | Kiran Misra, Seung-Hwan Kim, Andrew Segall 5705 NW Pacific Rim Blvd Camas WA 98607 USA |  | +1 360 817 8451 misrak@sharplabs.com kimse@sharplabs.com asegall@sharplabs.com |
| Source: | Sony Electronics Inc.and SHARP Laboratories of America |  |  |

_____________________________

# Abstract

This contribution reports the results of test 5.2 of SCE1 based on JCTVC-Q0129 and JCTVC-Q0141. In this test for color gamut scalability, the YCbCr space of the base layer is divided into 8x2x2 regions where the Y is divided uniformly and the Cb and Cr are divided non-uniformly. The chroma non-uniform partition is signaled in the bitstream. And, for each region, the matrix mapping are signaled as in JCTVC-Q0141. The proposed scheme has lesser operations for color mapping than SHM-6.0, and the signaling data structure of the colour mapping is changed from recursive to linear. The proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {−0.7%, −1.6%, −0.1%}, and {−0.6%, −1.0%, −0.5%}, {−0.5%, −1.2%, 0.1%}, and (−0.2%, −0.6%, −0.2%} for AI-1x, AI-2x, RA-1x and RA-2x, respectively. With encoder optimization, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {−1.0%, −1.9%, −0.5%}, and {−0.9%, −1.0%, −0.9%}, {−0.9%, −1.4%, −0.2%}, and (−0.3%, −0.4%, −0.3%} for AI-1x, AI-2x, RA-1x and RA-2x, respectively.

# Introduction

In this proposal, a combination of JCTVC-Q0129 [3] and JCTVC-Q0141 [5], which are both based on the matrix based mapping in JCTVC-Q0072 [2], is investigated. In particular, the color mapping in SHM6 is replaced by matrix mapping generalized from JCTVC-Q0129, and the coding of the mapping parameters in SHM6.0 is replaced by the method in JCTC-Q0141. As in SHM-6.0, the color gamut scalability is conducted before up-sampling and the luma and chroma phases are aligned for luma and chroma matrix mapping.

# Color mapping at decoder

The input to this process is a (y,u,v) triplet in base layer color gamut. The output of the process is the triplet (Y,U,V) in enhancement layer color gamut.

First, following frame level varibles are derived based on signaled parameters as follows.

yShift2Idx = InputBitDepth – 3

uThresh = (uThreshIdx + 16) << (InputBitDepthC - 5)

vThresh = (vThreshIdx + 16) << (InputBitDepthC - 5)

nMappingShift = 10

nMappingOffset = 1 << (nMappingShift - 1)

## (y, u, v) triplet adjustment with phase alignment

As in SHM-6.0, the phase aligned triplet (y’, u’, v’) instead of (y,u,v) is used for luma component color gamut mapping.

## Intermediate variables

With input (y’, u’, v’), intermediate variables are derived as follows.

yIdx = y’ >> yShift2Idx

uIdx = u’ >= uThresh ? 1 : 0

vIdx = v’ >= vThresh ? 1 : 0

## Color mapping

Y = ((y * lutY[yIdx][uIdx][vIdx][0] + u’ * lutY[yIdx][uIdx][vIdx][1] + v’ * lutY[yIdx][uIdx][vIdx][2]

+ nMappingOffset) >> nMappingShift) + lutY[yIdx][uIdx][vIdx][3]

U = ((y’ * lutU[yIdx][uIdx][vIdx][0] + u * lutU[yIdx][uIdx][vIdx][1] + v * lutU[yIdx][uIdx][vIdx][2]

+ nMappingOffset) >> nMappingShift) + lutU[yIdx][uIdx][vIdx][3]

V = ((y’ * lutV[yIdx][uIdx][vIdx][0] + u * lutV[yIdx][uIdx][vIdx][1] + v * lutV[yIdx][uIdx][vIdx][2]

+ nMappingOffset) >> nMappingShift) + lutV[yIdx][uIdx][vIdx][3]

# Signaling of color prediction coefficients

In this test, the method from JCTVC-Q0141 is applied to signal the color prediction coefficients lutY[yIdx][uIdx][vIdx][pIdx], lutU[yIdx][uIdx][vIdx][pIdx], lutV[yIdx][uIdx][vIdx][pIdx]. In particular, the color prediction coefficients are decomposed into two parts: absolute value and sign information. The absolute value is signalled as Quotient and Remainder based on a predetermined divisor (1<<7). The sign information is signaled separately as a flag and only if the absolute value is non-zero.

The value of the color prediction coefficients are derived as follows:

Coefficient = (Quotient << 7) + Remainder

Coefficient = (Sign==0) ? Coefficient : −Coefficient

Note that no prediction technique is applied between coefficients.

# Description of CGS information derivation at the encoder side

In general, CGS information is derived at encoder side for each picture similar to SHM-6.0.

Firstly, the YUV space of the base layer is divided into 8x32x32 non-uniform regions.

Secondly, the average value of Cb and Cr of the current picture, uMean and vMean, respectively, are computed from the 8x32x32 regions, and the thresholds are derived as the followings:

uvShift2Idx = InputBitDepthC – 5

uThresh = (uMean >> uvShift2Idx) << uvShift2Idx

vThresh = (vMean >> uvShift2Idx) << vvShift2Idx

Then the 8x32x32 uniform regions are merged into 8x2x2 regions according to uThresh and vThresh.

As in SHM-6.0, the CGS LUT for the current picture is derived by minimizing the SSE between the base layer construction and the down-sampled enhancement input picture. Subsequently, the distortion resulted by the CGS LUT estimated based on the current model is compared to the distortion resulted based the model signaled in the previous PPS. If the estimated distortion is smaller than that by CGS LUT signaled in the previous PPS, the CGS LUT is updated in PPS.

# Simulation results

The encoder described in Section 4 is used to generate the results. The proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {−0.7%, −1.6%, −0.1%}, and {−0.6%, −1.0%, −0.5%}, {−0.5%, −1.2%, 0.1%}, and (−0.2%, −0.6%, −0.2%} for AI-1x, AI-2x, RA-1x and RA-2x, respectively.

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -0.7% | -1.6% | -0.1% | -0.6% | -1.0% | -0.5% |
| Overall (Test vs Ref) | -0.7% | -1.6% | -0.1% | -0.6% | -1.0% | -0.5% |
| Overall (Test vs single layer) | 17.9% | 8.8% | -9.1% | 10.2% | 10.3% | 3.2% |
| Overall (Ref vs single layer) | 18.7% | 10.4% | -9.0% | 10.9% | 11.4% | 3.7% |
| EL only (Test vs Ref) | -6.3% | -7.3% | -5.5% | -1.5% | -1.8% | -1.3% |
| Overall (Test EL+BL vs single EL+BL) | -36.0% | -40.7% | -50.0% | -26.6% | -26.8% | -31.3% |
| Overall (Ref EL+BL vs single EL+BL) | -35.5% | -39.9% | -50.0% | -26.1% | -26.2% | -31.0% |
| Enc Time[%] | 98.1% |  |  | 102.5% |  |  |
| Dec Time[%] | 89.2% |  |  | 102.4% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -0.5% | -1.2% | 0.1% | -0.2% | -0.6% | -0.2% |
| Overall (Test vs Ref) | -0.5% | -1.2% | 0.1% | -0.2% | -0.6% | -0.2% |
| Overall (Test vs single layer) | 28.5% | 21.6% | -0.3% | 21.0% | 21.7% | 12.2% |
| Overall (Ref vs single layer) | 29.1% | 22.9% | -0.5% | 21.3% | 22.5% | 12.4% |
| EL only (Test vs Ref) | -3.0% | -3.8% | -2.3% | -0.5% | -1.0% | -0.5% |
| Overall (Test EL+BL vs single EL+BL) | -29.2% | -32.6% | -44.0% | -18.6% | -17.9% | -23.8% |
| Overall (Ref EL+BL vs single EL+BL) | -28.9% | -31.9% | -44.1% | -18.4% | -17.5% | -23.7% |
| Enc Time[%] | 99.8% |  |  | 99.6% |  |  |
| Dec Time[%] | 93.9% |  |  | 98.8% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Supplementary results

In Section 5 and Section 6, the threshold (uThresd, vThresh) are quantized from the mean value of the Cb and Cr. To improve the encoder, the threshold was searched with a range of +/- 1 from the uThresd and vThresh in Section 5 to minimize the prediction error. The results are reported here.

The encoder optimization reportedly achieves average {Y, U, V} BD rate gain of {−1.0%, −1.9%, −0.5%}, and {−0.9%, −1.0%, −0.9%}, {−0.9%, −1.4%, −0.2%}, and (−0.3%, −0.4%, −0.3%} for AI-1x, AI-2x, RA-1x and RA-2x, respectively.

|  | AI HEVC 1x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -1.0% | -1.9% | -0.5% | -0.9% | -1.0% | -0.9% |
| Overall (Test vs Ref) | -1.0% | -1.9% | -0.5% | -0.9% | -1.0% | -0.9% |
| Overall (Test vs single layer) | 17.5% | 8.5% | -9.5% | 10.0% | 10.3% | 2.8% |
| Overall (Ref vs single layer) | 18.7% | 10.4% | -9.0% | 10.9% | 11.4% | 3.7% |
| EL only (Test vs Ref) | -8.7% | -9.7% | -8.1% | -2.0% | -2.1% | -1.9% |
| Overall (Test EL+BL vs single EL+BL) | -36.2% | -40.9% | -50.2% | -26.8% | -26.9% | -31.6% |
| Overall (Ref EL+BL vs single EL+BL) | -35.5% | -39.9% | -50.0% | -26.1% | -26.2% | -31.0% |
| Enc Time[%] | 97.3% |  |  | 98.8% |  |  |
| Dec Time[%] | 87.6% |  |  | 97.0% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 1x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -0.9% | -1.4% | -0.2% | -0.3% | -0.4% | -0.3% |
| Overall (Test vs Ref) | -0.9% | -1.4% | -0.2% | -0.3% | -0.4% | -0.3% |
| Overall (Test vs single layer) | 28.0% | 21.3% | -0.6% | 20.9% | 21.9% | 12.0% |
| Overall (Ref vs single layer) | 29.1% | 22.9% | -0.5% | 21.3% | 22.5% | 12.4% |
| EL only (Test vs Ref) | -4.7% | -5.3% | -3.8% | -0.7% | -0.8% | -0.7% |
| Overall (Test EL+BL vs single EL+BL) | -29.5% | -32.8% | -44.2% | -18.6% | -17.8% | -23.9% |
| Overall (Ref EL+BL vs single EL+BL) | -28.9% | -31.9% | -44.1% | -18.4% | -17.5% | -23.7% |
| Enc Time[%] | 104.3% |  |  | 99.8% |  |  |
| Dec Time[%] | 100.3% |  |  | 98.7% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# References

- A. Duenas, P. Andrivon, Y. Ye, E. Alshina, X. Li, K. Ugur, C. Auyeung, S.H. Kim, “Description of HEVC Scalable Extensions Core Experiment SCE1: Color Gamut Scalability,” JCTVC-Q1101, 17th Meeting, Valencia, ES, 27 March – 4 April 2014.

- A. Aminlou, K. Ugur, M. M. Hannuksela, “SCE1: Color gamut scalability using gain-offset models,” JCTVC-Q0072, 17th Meeting, Valencia, ES, 27 March – 4 April 2014.

- C. Auyeung, K. Sato, “Non-SCE1: Reduction of the number of color space regions in SCE1 test 1.1,”, JCTVC-Q0129, 17th Meeting, Valencia, ES, 27 March – 4 April 2014.

- X. Li, J. Chen, M. Karczewicz, Y. He, Y. Ye, J. Dong, P. Bordes, P. Andrivon, E. François, F. Hiron “SCE1.2: Color gamut scalability with asymmetric 3D LUT,”, JCTVC-Q0048, 17th Meeting, Valencia, ES, 27 March – 4 April 2014.

- K. Misra, S.H. Kim, A. Segall, “Non-SCE1: Coding of color gamut prediction coefficients,”, JCTVC-Q0141, 17th Meeting, Valencia, ES, 27 March – 4 April 2014.

- J. Chen, J. Boyce, Y. Ye, M.M. Hannuksela, G.J. Sullivan, Y.K. Wang, “Preliminary version of High efficiency video coding (HEVC) scalable extension Draft 6,” JCTVC-Q1008_v2, 17th Meeting, Valencia, ES, 27 March – 4 April 2014.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).