Text specification for JCTVC-R0174

# Two additional contexts:

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … |  |  |  |  |  |  |
| palette_run | 0,1,2,3 (subclause 9.3.4.2.8) | 4,5 (subclause 9.3.4.2.8) | 6,7 (subclause 9.3.4.2.8) | bypass | bypass | bypass |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | Na | na |

##### 9.3.4.2.8 Derivation process of ctxInc for the syntax element palette_run

Inputs to this process are palette_index and palette_run_type_flag.

Output of this process is the variable ctxInc.

| binIdx | palette_run_type_flag | palette_index | ctxInc |
| --- | --- | --- | --- |
| 0 | 1 |  | 0 |
|  | 0 | 0 | 1 |
|  |  | 1,2 | 2 |
|  |  | greater than 2 | 3 |
| 1 | 1 |  | 4 |
|  | 0 |  | 5 |
| 2 | 1 |  | 6 |
|  | 0 |  | 7 |

# Using exponential Golomb code of order 2 for coding remaining runs

9.3.3.11 Binarization process for palette_run

Input to this process is a request for a binarization for the syntax element palette_run.

Output of this process is the binarization of palette_run according to the following table.

| palette_run | codeword |
| --- | --- |
| 0 | ‘0’ |
| 1 | ‘10’ |
| 2 | ‘110’ |
| >2 | Prefix=‘111’, Suffix=EG2 |