| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0176_r1 |
| --- | --- |

| Title: | Non-SCCE3: Combination of subtest C.4 with new palette tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Wei Pu, Vadim Seregin, Marta Karczewicz, Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1 858 658 4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In subtest C.4 of screen content coding experiment 3 (SCCE3), a combination of proposed tools from subtests A.7, A.8, B.12 and B.13 was tested. This document reports the combination of subtest C.4 with other new palette tools related to contexts for coding index runs and run signalling for binary palette prediction vector and encoder bugfixes. It is reported that BD-rate improvements up to 12.5% and 14.9% is achieved with respect to SCCE3 palette base under full-frame intra block copying and two CTU intra block copying, respectively. When palette predictor size of 128 is used, BD-rate improvements up to 12.9% and 15.3%, respectively, is reported.

# Introduction

In subtest C.4 of screen content coding experiment 3 (SCCE3) (JCTVC-Q1123), a combination of tools from subtests A.7, A.8, B.12 and B.13 was tested. After the CE software was released, the encoder was improved combining some aspects of the encoder from subtest C.2 (JCTVC-R0086). This will be referred to as C.4 with improved encoder. In this document we present results for combination of C.4 with improved encoder and other non-SCCE3 palette tools to show that further performance gains are possible. The tools are:

- Encoder bugfixes listed in the SCCE3 report (JCTVC-RXXXX)

- Additional run contexts for coding ‘index’ runs (JCTVC-R0174)

- Run coding of binary palette prediction vector (JCTVC-R0228)

# Simulation results

The proposed methods are implemented on top of the SCCE3 palette software based on SCM 1.0 and simulated under common test conditions (JCTVC-Q1015) are applied. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. Table 1 shows the BD-rate performance for the proposed combination with respect to the SCCE3 base palette anchor for all intra configuration (full-frame intra block copying).

Table 1: BD-rate performance for the proposed combination for all intra configuration. Anchor is SCCE3 base palette (left: full-frame intra block copying, right: two CTU intra block copying)

Table 2 shows the BD-rate performance for the proposed combination for all intra configuration when palette predictor size of 128 is used. The anchor is SCCE3 base palette anchor (full-frame intra block copying).

Table 2: BD-rate performance for the proposed combination with palette predictor size of 128 for all intra configuration. Anchor is SCCE3 base palette (left: full-frame intra block copying, right: two CTU intra block copying)

# Discussion

Simulation results for a combination of subtest C.4 with encoder improvements, encoder bugfixes, additional contexts for coding of index runs, and run length signalling for binary palette predictor vector are presented. It is reported that for all intra configuration, BD-rate improvements up to 12.5% and 14.9% is achieved with respect to SCCE3 palette base for full-frame intra block copying and two CTU intra block copying, respectively. When palette predictor size of 128 is used, BD-rate improvements up to 12.9% and 15.3%, respectively, is achieved.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).