| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0178 |
| --- | --- |

| Title: | Non-SCCE5: On index signaling of cross component prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Marta Karczewicz Jianle Chen Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

A method for signaling the scaling parameters in cross-component prediction is proposed. The method keeps the current syntax and parsing process of HEVC Range Extensions unchanged, while the mapping from the decoded index to the actual scaling parameter is modified. It is reported that the proposed method provides BD rate reductions of -0.6% and -0.4% for lossy coding of 1080p mixed content with full-frame and 2 CTU Intra Block Copy search ranges, respectively.

# Background

Cross-component prediction (CCP) is employed in HEVC Range Extensions. In CCP, a prediction is performed between luma residual signal and chroma residual signal. The predictor is generated using simple operations like one multiplication (signaled scaling parameter denoted by α) and one bit-shift operation. Specifically, the chroma residual signal is reconstructed using the luma residual signal at the decoder side as:

rC'(x,y)=ΔrC'(x,y)+α×rL'(x,y)>>3	 				(1)

where rC'denotes the reconstructed residual of the chroma component after cross-color compensation,
rL' denotes the reconstructed residual sample of the luma component, and 
ΔrC'denotes the decoded residual signal before cross-color compensation.

The scaling parameter α is allowed to be chosen from −8, −4, −2, −1, 0, 1, 2, 4, 8, inclusively.

The scaling parameter is represented by two parts: one is the sign flag and the other is the absolute value of α, abs(α). Before binarization, a mapping process, as defined in Table 1, is firstly invoked to map abs(α) to M(α). Then M(α) is binarized using truncated unary code and coded using CABAC. At the decoder side, after CABAC decoding, M(α) is inversely mapped back to abs(α).

Table 1. α Mapping Table.

| abs(α) | M(α) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 4 | 3 |
| 8 | 4 |

Such a mapping function was designed to match the probability distribution of abs(α) for YCbCr coding. However, the statistics may be different for RGB coding and therefore, higher coding gains are expected by changing the mapping function.

# Proposed method

One flag in sequence parameter set is added to indicate the usage of the new mapping function. When this flag is equal to 1, the new mapping function, as defined in the following table, is used.

Table 2. α Mapping Table.

| abs(α) | M(α) |
| --- | --- |
| 0 | 0 |
| 1 | 4 |
| 2 | 3 |
| 4 | 2 |
| 8 | 1 |

In addition, the initialization value of the context for signaling the sign flag is set to 79 when the new mapping table is applied.

With the proposed method, the current syntax table and parsing process of HEVC Range Extensions, i.e., binarization process, are reused.

# Simulation results

The method implemented on top of SCM1.0. The experiments are performed using the common test conditions defined for screen content coding [1].Table 3 and  report the coding gain of the proposal for the lossy and lossless coding, respectively, under SCC test conditions with full frame IntraBC search range. For lossy coding, the proposed solution achieves up to 0.6% BD rate reduction for Low delay B case. For lossless coding, the performance is almost unchanged.

Table 3. Coding gains of proposal compared to anchor (Lossy, SCM1.0 with full-frame IntraBC)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | 0.0% | -0.2% | -0.1% | -0.2% | -0.3% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.6% | 0.2% | -0.4% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 98% |  |  | 97% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  | 96% |  |  |

Table 4. Coding gains of proposal compared to anchor (Lossless, SCM1.0 with full-frame IntraBC)

|  | All Intra |  | Random Access |  | Low Delay B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  | 98% |  | 98% |  |
| Dec Time[%] | 99% |  | 96% |  | 98% |  |

and Table 6 report the coding gain of the proposal for the lossy and lossless coding, respectively, under SCC test conditions with local IntraBC search range. The proposed solution achieves up to 0.4% BD rate reduction for Low delay B case. For lossless coding, the performance is almost unchanged.

Table 5. Coding gains of proposal compared to anchor (SCM1.0 with local IntraBC)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U |  | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.3% | -0.2% | -0.2% |  |
| RGB, text & graphics with motion,720p | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.3% |  |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% | -0.1% |  |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% | 0.0% | -0.3% |  |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% |  |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% |  |
| Enc Time[%] | 98% |  |  | 96% |  |  | 99% |  |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 98% |  |  |  |

Table 6. Coding gains of proposal compared to anchor (Lossless, SCM1.0 with local-frame IntraBC)

|  | All Intra |  | Random Access |  | Low Delay B |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  | 98% |  | 98% |  |
| Dec Time[%] | 99% |  | 96% |  | 98% |  |

# References

- W. Pu, W.-S. Kim, J. Chen, J. Sole, M. Karczewicz, “RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4,” JCTVC-O0202, Geneva, CH, 25 Oct. – 1 Nov. 2013.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, ES, 27 March – 4 April 2014.

# Patent rights declaration(s)

Qualcomm Incorporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).