| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0179 |
| --- | --- |

| Title: | Non-SCE1: 3D LUT with larger sizes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Ye, Yuwen He 9710 Scranton Rd, Suite 250 San Diego, CA 92130 | Tel: Email: | +1-858-210-4803/4819 yan.ye@interdigital.com yuwen.he@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

3D LUT based Color Gamut Scalability was adopted into SHVC draft 6 at the 17th JCT-VC meeting. Although the current design limits the maximum size of 3D LUT to 8x2x2, it is reported that larger 3D LUT sizes can provide further performance benefits. Using SHM6.0 without any changes, and with 8x2x2 3D LUT as the anchors, 8x8x8 3D LUT and 8x4x4 3D LUT are reported to provide overall average {Y, U, V} BD-rate reduction of  {-1.2%, -2.2%, -2.6%}  and {-0.5%, -0.6%, -1.0%}, respectively.  With improved encoder design, the performance gain of 8x8x8 3D LUT and 8x4x4 3D LUT can be increased to {-1.6%, -3.3%, -4.0%} and {-0.7%, -1.1%, -1.5%}, respectively. The importance of allowing larger 3D LUT size will become more evident as the sample bit depth increases to 12-bit. It is therefore suggested for the JCT-VC to discuss an appropriate maximum 3D LUT size, such that SHVC can provide maximal benefits to the market requiring Color Gamut Scalability with reasonable cost.

# Introduction

3D LUT based Color Gamut Scalability was adopted into SHVC at the 17th JCT-VC meeting. Based on the performance and complexity evidence available at the last meeting, the 3D LUT size was limited to be at most 8x2x2. In SHVC draft 6 [1], two syntax elements are used to specify the 3D LUT size, as shown in the following syntax table:

| colour_mapping_table( ) { | Descriptor |
| --- | --- |
| cm_octant_depth | u(2) |
| cm_y_part_num_log2 | u(2) |
| cm_input_luma_bit_depth_minus8 | u(3) |
| cm_input_chroma_bit_depth_delta | se(v) |
| cm_output_luma_bit_depth_minus8 | u(3) |
| cm_output_chroma_bit_depth_delta | se(v) |
| cm_res_quant_bits | u(2) |
| colour_mapping_octants( 0, 0, 0, 0, 1 << cm_octant_depth ) |  |
| } |  |

cm_octant_depth specifies the maximal split depth of the colour mapping table. In bitstreams conforming to this version of this Specification, the value of cm_octant_depth shall be in the range of 0 to 1, inclusive. Other values for cm_octant_depth are reserved for future use by ITU-T | ISO/IEC.

The variable COctantNum is derived as follows.

COctantNum = 1 << cm_octant_depth		(F-1)

cm_y_part_num_log2 specifies the number of partitions of the smallest colour mapping table octant for the luma component. The variables YOctantNum and YPartNum are derived as follows. In bitstreams conforming to this version of this Specification, the value of cm_y_part_num_log2 shall be in the range of 0 to 2, inclusive. Other value for cm_y_part_num_log2 is reserved for future use by ITU-T | ISO/IEC.

The variables YOctantNum and YPartNum are derived as follows.

YOctantNum = 1 << ( cm_octant_depth + cm_y_part_num_log2 )	(F-2)

YPartNum = 1 << cm_y_part_num_log2		(F-3)

Therefore, although the syntax element lengths allow larger 3D LUT sizes, bitstream conformance requirements specify the largest 3D LUT size is limited to 8x2x2 with cm_octant_depth being 1 and cm_y_part_num_log2 being 2. In fact, with the current language used to specify the bitstream conformance constraints, not all table sizes smaller than 8x2x2 are allowed. Specifically, table size 8x1x1 is not allowed, and only the following table sizes are allowed: 8x2x2, 4x2x2, 4x1x1, 2x2x2, 2x1x1, 1x1x1.

Given that there is no additional complexity associated with using 8x1x1, in this contribution, it is first proposed to reword the bitstream conformance constraint to allow all table sizes no bigger the maximum specified, which can achieved with the following text (changes in highlight):

cm_y_part_num_log2 specifies the number of partitions of the smallest colour mapping table octant for the luma component. The variables YOctantNum and YPartNum are derived as follows. In bitstreams conforming to this version of this Specification, the value of (cm_y_part_num_log2 + cm_octant_depth) shall be in the range of 0 to 3, inclusive. Other value for cm_y_part_num_log2 is reserved for future use by ITU-T | ISO/IEC.

Further, potential performance benefits of using larger 3D LUT tables are evaluated. It is reported that additional performance gain can be achieved by allowing larger 3D LUT sizes. Detailed performance data for 8x8x8 and 8x4x4 LUT sizes are reported in Section 2, including two sets of data generated with 1) SHM6.0 software with no change, and 2) SHM6.0+encoder only changes. Given that the performance benefits of larger 3D LUT sizes are expected to increase as the sample bit depth increases, it is suggested for the group to discuss an appropriate 3D LUT table size limit.

# CGS performance of 8x8x8 and 8x4x4 3D LUT

## Encoder side optimization

Currently in SHM-6.0 [2], 3D LUT size is selected by considering the relative signaling overhead (relatively to the coded picture sizes) using pre-selected thresholds. Specifically, the 3D LUT size is decreased when the signaling cost of 3D LUT is over 3% of the previously coded picture, and the 3D LUT size is increased when its signaling cost is below 0.5% of the previously coded picture. Such method to determine the 3D LUT size is not optimal in terms of CGS coding efficiency.

In this contribution, rate distortion optimization is applied to select the appropriate 3D LUT s with optimal table size, using the following cost calculation.

JCost(s)=w⋅D(s)+λ⋅bits(s)

where a fixed weight “w” is applied to the distortion D(s) of 3D LUT size “s”. The value of “w” depends on the EL picture’s slice type and temporal level. The rate distortion based 3D LUT size selection is applied only when deciding whether to send updated 3D LUT. Two additional conditions are also applied:

- PPS update of 3D LUT is disabled for temporal level 2 and 3.

- PPS update of 3D LUT is disabled for RASL pictures.

## 8x4x4 3D LUT performance

Table 1. Using 8x4x4 3D LUT with SHM6.0 compared to SHM6.0 CGS anchors (8x2x2 3D LUT)

Table 2. Using 8x4x4 3D LUT with SHM6.0+encoder optimization compared to SHM6.0 CGS anchors (8x2x2 3D LUT)

## 8x8x8 3D LUT performance

Table 3 Using 8x8x8 3D LUT with SHM6.0 compared to SHM6.0 CGS anchors (8x2x2 3D LUT)

Table 4 Using 8x8x8 3D LUT with SHM6.0 + encoder optimization compared to SHM6.0 CGS anchors (8x2x2 3D LUT)

# Conclusions

This proposal evaluated CGS performance with larger 3D LUT size. It is suggested for the JCT-VC to consider an appropriate 3D LUT size limitation.

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, “High efficiency video coding (HEVC) scalable extension Draft 6”, JCTVC-Q1008, Apr. 2014.

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Test Model 6 (SHM 6)”, JCTVC-Q1007, Apr. 2014.