| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0180 |
| --- | --- |

| Title: | Non-SCCE5: Deblocking filter for in-loop color-space transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Jianle Chen Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the SCCE5 Test 3.2.1 contribution, the blocks coded with color transform enabled are de-quantized using a modified quantization parameter (QP) which is different from blocks coded with normal modes. However, when determining the luma/chroma edges in the deblocking filter process, all blocks use the same QPs without distinguishing it is coded with color transform enabled or not. In this contribution, it is proposed to align the quantization parameters used in deblocking filter process to be the same as those used in de-quantization process. It is reported that the proposed method provides BD rate reduction of 1.5% and 2.0% for lossy coding of 1080p mixed content and animation with 2 CTU Intra Block Copy search range, respectively.

# Introduction

In the SCCE5 Test 3.2.1 proposal [1], color-space transform is introduced to further reduce the redundancy among three color component. For lossy coding, the non-normalized YCoCg transform is employed and delta quantization parameters of (-5, -3, -5) are used for the three color components, respectively. That is, when the color-space transform is applied, the quantization parameter is set equal to (QP - 5, QP - 3, QP - 5) for the three components, respectively, where QP is the 'normal' QP value for the coding unit (CU). However, in the deblocking process, the ‘normal’ QP values are used for all CUs without considering the usage of color-space transform.

# Proposed method

During the luma/chroma edge detection process in deblocking filter, the same quantization parameter as used in de-quantization process is utilized, i.e., (QP - 5). In addition, the color-space transform enabling flag for some CUs may be skipped, e.g., the rqt_root_cbf is equal to 0 for inter and intraBC modes. In this case, these CUs are treated as color transform enabled. That is, (QP - 5) is used.

# Simulation results

The method implemented on top of JCTVC-R0147. The experiments are performed using the common test conditions defined for screen content coding [2]. The anchor is color transform method in JCTVC-R0147 without non-normative coding tools enabled. Since the proposed method has no effect on lossless coding, only the performance of lossy coding is reported.Table 1 reports the coding gain of the proposal under SCC test conditions with local IntraBC search range. For the lossy coding, the proposed solution achieves up to 2.0% BD rate reduction for Low delay B case.

Table 1. BD rate reduction compared to JCTVC-R0147 wo/ non-normative tool (2CTU IntraBC)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -1.0% | -0.9% | -0.4% | -1.4% | -1.3% | -0.6% | -1.3% | -1.2% |
| RGB, text & graphics with motion,720p | -0.3% | -1.2% | -1.4% | -0.5% | -1.6% | -1.8% | -1.0% | -1.2% | -1.4% |
| RGB, mixed content, 1440p | -0.4% | -1.2% | -1.2% | -0.7% | -1.8% | -1.8% | -1.1% | -1.6% | -1.5% |
| RGB, mixed content, 1080p | -0.2% | -1.1% | -1.0% | -0.5% | -1.8% | -1.7% | -1.5% | -1.7% | -1.9% |
| RGB, Animation, 720p | -0.6% | -1.5% | -1.5% | -1.0% | -1.3% | -1.2% | -2.0% | -0.3% | -0.3% |
| RGB, camera captured, 1080p | 0.0% | -0.4% | 0.0% | 0.7% | -0.2% | -0.1% | 0.3% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.2% | -0.3% | -0.1% | -0.5% | -0.5% | -0.5% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.3% | -0.6% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.8% | -1.0% | -1.0% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% | -0.5% | 0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% | 0.4% | 0.0% | 0.1% | 0.3% | -0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 97% |  |  |

Table 2 reports the coding gain of the joint solution of this proposal with the whole proposal of JCTVC-R0147 under SCC test conditions with local IntraBC search range. For the lossy coding, the proposed solution achieves up to 2.0% BD rate reduction for Low delay B case..

Table 2. BD rate reduction of JCTVC-R0180 and JCTVC-R0147 compared to SCM1.0 (2CTU IntraBC)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -10.7% | -5.1% | -5.0% | -10.5% | -4.3% | -4.3% | -9.4% | -2.8% | -2.5% |
| RGB, text & graphics with motion,720p | -17.4% | -5.5% | -8.9% | -20.1% | -6.5% | -11.4% | -21.2% | -4.8% | -9.9% |
| RGB, mixed content, 1440p | -20.2% | -8.0% | -8.0% | -27.2% | -12.1% | -11.7% | -28.9% | -9.6% | -8.4% |
| RGB, mixed content, 1080p | -15.7% | -4.3% | -5.5% | -19.7% | -6.7% | -7.6% | -21.7% | -3.1% | -3.7% |
| RGB, Animation, 720p | -25.6% | -18.8% | -16.1% | -26.3% | -17.0% | -12.6% | -25.8% | -11.7% | -5.4% |
| RGB, camera captured, 1080p | -25.2% | -5.4% | -9.9% | -27.4% | -5.5% | -14.0% | -25.9% | -1.7% | -12.1% |
| YUV, text & graphics with motion, 1080p | -1.6% | -1.4% | -1.9% | -1.5% | -1.8% | -2.1% | -2.3% | -2.4% | -2.4% |
| YUV, text & graphics with motion,720p | -0.9% | -1.2% | -1.8% | -1.8% | -1.9% | -3.1% | -2.5% | -2.8% | -4.0% |
| YUV, mixed content, 1440p | 0.0% | -1.5% | -1.4% | 0.1% | -1.1% | -1.1% | 0.1% | -1.3% | -1.1% |
| YUV, mixed content, 1080p | -0.1% | -0.7% | -0.8% | -0.1% | -0.9% | -1.0% | -1.1% | -1.7% | -2.3% |
| YUV, Animation, 720p | 0.1% | -6.9% | -4.1% | -0.2% | -7.0% | -3.6% | -0.7% | -5.7% | -3.6% |
| YUV, camera captured, 1080p | 0.2% | -0.1% | 0.0% | 0.9% | 0.1% | 0.3% | 0.6% | -0.2% | -0.1% |
| Enc Time[%] | 143% |  |  | 112% |  |  | 111% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |

# References

- L. Zhang, J. Chen, J. Sole, M. Karczewicz, X. Xiu, Y. He, Y. Ye, “SCCE5 Test 3.2.1: In-loop color-space transform,” JCTVC-R0147, Sapporo, JP, 30 June – 9 July 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, ES, 27 March – 4 April 2014.

# Patent rights declaration(s)

Qualcomm Incorporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).