| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0180_proposed text |
| --- | --- |

| Title: | Non-SCCE5: Deblocking filter for in-loop color-space transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Jianle Chen Joel Sole Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document provides the proposed text. The proposed changes of JCTVC-R0147 on top of JCTVC-P1005_v4 are highlighted in green. And the additional changes from this proposal are highlighted in cyan or red strikethrough.

# Decoding process

### 8.4.1	General decoding process for coding units coded in intra prediction mode

…

Depending on the values of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- A variable bModified is set equal to (intra_chroma_pred_mode[ xCb ][ yCb ] = = 4 && cu_residual_csc_flag[ xCb ][ yCb ]).

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- When intra_bc_flag[ xCb ][ yCb ] is equal to 1, the derivation process for block vector components in intra block copying prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) and variable log2CbSize as inputs, and the output being bvIntra.

- If bModified cu_residual_csc_flag[ xCb ][ yCb ] is equal to 1, the following applies:

–	For the variable cIdx proceeding over the values 0..2, the following ordered steps apply:

  - Set variable comp equal to (!cIdx ? L : (cIdx = =1 ? Cb : Cr).

  - The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable cIdx, and variable controlPara equal to 1 as inputs, and the output is the residual sample array resSamplescomp.

–	The residual modification process for residual blocks using color space conversion as specified in subclause 8.6.7 is invoked with the variable blkSize set equal to nCbS, the (nCbS)x(nCbS) array rY set equal to resSamplesL, the (nCbS)x(nCbS) array rCb set equal to resSamplesCb, and the (nCbS)x(nCbS) array rCr set equal to resSamplesCr as inputs, and the output are modified versions of the (nCbS)x(nCbS) arrays resSamplesL, resSamplesCb and resSamplesCr.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, and the variable cIdx set equal to 0, and variable controlPara equal to ( cu_residual_csc_flag[ xCb ][ yCb ]bModified ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- If cu_residual_csc_flag[ xCb ][ yCb ]bModified is equal to 1, the following applies:

- For the variable cIdx proceeding over the values 0..2, the following ordered steps apply:

  - Set variable comp equal to (!cIdx ? L : (cIdx = =1 ? Cb : Cr).

  - The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to 0, the variable cIdx, and variable controlPara set equal to 1 as inputs, and the output is the residual sample array resSamplescomp.

- Set the variable nSubCbS equal to ( nCbS  >> 1 ) and the residual modification process for residual blocks using color space conversion as specified in subclause 8.6.7 is invoked with the variable blkSize set equal to nSubCbS, the (nSubCbS)x(nSubCbS) array rY set equal to resSamplesL, the (nSubCbS)x(nSubCbS) array rCb set equal to resSamplesCb, and the (nSubCbS)x(nSubCbS) array rCr set equal to resSamplesCr as inputs, and the outputs are modified versions of the (nSubCbS)x(nSubCbS) arrays resSamplesL, resSamplesCb and resSamplesCr.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to 0, the variable cIdx set equal to 0 , and variable controlPara set equal to (cu_residual_csc_flag[ xCb ][ yCb ]bModified ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

…

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable cIdx set equal to 1, and variable controlPara set equal to (cu_residual_csc_flag[ xCb ][ yCb ]bModified ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra, the variable cIdx set equal to 2 , and variable controlPara set equal to (cu_residual_csc_flag[ xCb ][ yCb ]bModified ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, the variable cIdx set equal to 1 , and variable controlPara set equal to (cu_residual_csc_flag[ xCb ][ yCb ]bModified ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to 0, and the variable cIdx set equal to 2, and variable controlPara set equal to (cu_residual_csc_flag[ xCb ][ yCb ]bModified ? 2 : 3 ) as inputs, and the output is a modified reconstructed picture before deblocking filtering.

#### 8.5.4	Decoding process for the residual signal of coding units coded in inter prediction mode

#### 8.5.4.1	General

…

Depending on the value of rqt_root_cbf, the following applies:

–	If rqt_root_cbf is equal to 0 or skip_flag[ xCb ][ yCb ] is equal to 1, all samples of the (nCbSL)x(nCbSL) array resSamplesL and when ChromaArrayType is not equal to 0, all samples of the two (nCbSwC)x(nCbShC) arrays resSamplesCb and resSamplesCr are set equal to 0.

–	Otherwise (rqt_root_cbf is equal to 1), the following ordered steps apply:

- The decoding process for luma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable nCbS set equal to nCbSL, and the (nCbSL)x(nCbSL) array resSamplesL as inputs, and the output is a modified version of the (nCbSL)x(nCbSL) array resSamplesL.

- When ChromaArrayType is not equal to 0, the decoding process for chroma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 1, the variable nCbSw set equal to nCbSwC, the variable nCbSh set equal to nCbShC, and the (nCbSwC)x(nCbShC) array resSamplesCb as inputs, and the output is a modified version of the (nCbSwC)x(nCbShC) array resSamplesCb.

- When ChromaArrayType is not equal to 0, the decoding process for chroma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 2, the variable nCbSw set equal to nCbSwC, the variable nCbSh set equal to nCbShC, and the (nCbSwC)x(nCbShC) array resSamplesCr as inputs, and the output is a modified version of the (nCbSwC)x(nCbShC) array resSamplesCr.

- When cu_residual_csc_flag[ xCb ][ yCb ]bModified is equal to 1, the residual modification process for residual blocks using color space conversion as specified in subclause 8.6.7 is invoked with the variable blkSize set equal to nCbSL, the (nCbSL)x(nCbSL) array rY set equal to resSamplesL, the (nCbSL)x(nCbSL) array rCb set equal to resSamplesCb, and the (nCbSL)x(nCbSL) array rCr set equal to resSamplesCr as inputs, and the modified arrays resSamplesL, resSamplesCb and resSamplesCr as outputs.

### 8.6.1	Derivation process for quantization parameters

Input to this process is a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

In this process, the variable QpY, the luma quantization parameter Qp’Y, and the chroma quantization parameters Qp′Cb and Qp′Cr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current quantization group relative to the top-left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to xCb − ( xCb & ( ( 1  <<  Log2MinCuQpDeltaSize) − 1 ) ) and yCb − ( yCb & ( ( 1  <<  Log2MinCuQpDeltaSize) − 1 ) ), respectively. The luma size of a quantization group, Log2MinCuQpDeltaSize, determines the luma size of the smallest area inside a coding tree block that shares the same qPY_PRED.

The predicted luma quantization parameter qPY_PRED is derived by the following ordered steps:

…

The variable bModified is defined as (color_transform_flag[ xCb ][ yCb ]&& (CuPredMode[ xCb ][ yCb ]  = =  MODE_INTER || intra_chroma_pred_mode[ xCb ][ yCb ] = = 4)).

The variable QpY is derived as follows:

QpY = ( ( qPY_PRED + CuQpDeltaVal + 52 + 2 * QpBdOffsetY )%( 52 + QpBdOffsetY ) ) − QpBdOffsetY	(8-2)

QpY = QpY + (bModified ? deltaQPC0 : 0)

The luma quantization parameter Qp′Y is derived as follows:

Qp′Y = QpY + QpBdOffsetY		(8-3)

When ChromaArrayType is not equal to 0, the following applies.

–	…

–	The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC		(8-4)

Qp′Cr = qPCr + QpBdOffsetC		(8-5)

Qp′Cb  = Qp′Cb + (bModified ? deltaQPC1 : 0)

Qp′Cr  =  Qp′Cr + (bModified ? deltaQPC2 : 0)

### 8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y + (cu_residual_csc_flag [ xTbY ][ yTbY ] ? -5 : 0)	(8-6)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb + (cu_residual_csc_flag [ xTbY ][ yTbY ] ? -5 : 0)	(8-7)

–	Otherwise (cIdx is equal to 2),

qP = Qp′C + (cu_residual_csc_flag [ xTbY ][ yTbY ] ? -3 : 0)	(8-8)