| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0181v2 |
| --- | --- |

| Title: | Non-SCCE1: Block vector coding for Intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Krishna Rapaka Chao Pang Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-2042 krapaka@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method of coding intra block vector difference (BVD). It is argued that the distributions of BVD’s are different from that of inter motion vector difference’s (MVD’s) and hence using the MVD binarization to code BVD as in HM-14.0+RExt-7.0+SCC-1.0 is not efficient. It is asserted that the proposed method better adapts to BVD characteristics. It is reported that the proposed approach provides objective bitrate reductions of -2.9 %, -3.7 % for All Intra 1080p text and graphics RGB and YUV categories, respectively over HM-14.0+RExt-7.0+SCC-1.0 anchor. When combined with block vector prediction method in SCCE1 Test 3.1, the method provides bit rate reductions of -11.1%, -12.3 % for All Intra 1080p text and graphics RGB and YUV categories, respectively.

In the revision v1, RA and LD-B results of the proposed method are also added.

In the revision v2, coding for inter motion vector difference (MVD’s) is modified. It is reported that the proposed approach in combination with the proposed BVD coding method provides objective bit rate reductions of -3.5 %, -4.1 % for RA 1080p text and graphics RGB and YUV categories respectively and  -4.0% and -4.5% for LD 1080p text and graphics RGB and YUV categories respectively over HM-14.0+RExt-7.0+SCC-1.0 anchor.

# Proposed Method

For better adaptation to the BVD characteristics, this contribution proposes to code BVD as below:

First a context coded flag is coded to indicate BVD is zero. When BVD is not zero, another flag is coded to indicate if absolute level of BVD is greater than 16. If BVD is greater than 16, the remaining absolute level is coded using bypass Exp-Golomb codes with order 5. Otherwise, the remaining absolute level is coded using fixed length bypass code of length 4. One flag is coded to indicate sign of the BVD.

Separate contexts for horizontal and vertical component are used for the above two prefix flags. The text specification is detailed in section 3. Note that the number of regular bins in the proposed method is 4, which is the same as that of the MVD coding in HEVC.

In the revision v2, this contribution proposes to code MVD as below:

First a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag is coded to indicate if absolute level of MVD is greater than 1. If MVD is greater than 1, the remaining absolute level is coded using bypass Exp-Golomb codes with order 3. Separate contexts for horizontal and vertical component are used for the above two prefix flags. One flag is coded to indicate sign of the MVD. Note that the number of regular bins in the proposed method is 4, which is the same as that of the MVD coding in HEVC.

# Results

Experimental results are presented for the proposed BVD coding method. Two configurations are investigated under SCC test conditions [1]. For configuration 1, the proposed method is implemented on top of HM-14.0+RExt-7.0+SCC-1.0 and Table 1 shows BD-rates and rate savings.

For configuration 2, the proposed method is combined with block vector prediction method in SCCE1 Test 3.1 and Table 2 shows BD-rates and rate savings.

Table 1: Results for lossy configuration 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.9% | -2.8% | -2.8% |
| RGB, text & graphics with motion,720p | -1.7% | -1.8% | -1.8% |
| RGB, mixed content, 1440p | -0.9% | -0.9% | -0.9% |
| RGB, mixed content, 1080p | -1.2% | -1.1% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -3.7% | -3.4% | -3.4% |
| YUV, text & graphics with motion,720p | -2.3% | -2.3% | -2.3% |
| YUV, mixed content, 1440p | -1.2% | -1.2% | -1.3% |
| YUV, mixed content, 1080p | -1.4% | -1.3% | -1.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -1.7% | -1.7% |
| RGB, text & graphics with motion,720p | -1.3% | -1.2% | -1.3% |
| RGB, mixed content, 1440p | -0.7% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -0.8% | -0.7% | -0.8% |
| RGB, Animation, 720p | -0.2% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.9% | -1.8% | -1.8% |
| YUV, text & graphics with motion,720p | -1.5% | -1.5% | -1.6% |
| YUV, mixed content, 1440p | -0.8% | -0.7% | -0.7% |
| YUV, mixed content, 1080p | -1.0% | -1.1% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 88% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.4% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -0.8% | -0.7% | -0.7% |
| RGB, mixed content, 1440p | -0.5% | -0.8% | -0.6% |
| RGB, mixed content, 1080p | -0.6% | -0.5% | -0.7% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.4% | -1.4% |
| YUV, text & graphics with motion,720p | -0.8% | -0.8% | -0.9% |
| YUV, mixed content, 1440p | -0.4% | -0.7% | -0.5% |
| YUV, mixed content, 1080p | -0.2% | -0.9% | -0.1% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 95% |  |  |

Table 2: Results for lossy configuration 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -11.1% | -11.0% | -11.1% |
| RGB, text & graphics with motion,720p | -6.4% | -6.6% | -6.6% |
| RGB, mixed content, 1440p | -4.3% | -4.2% | -4.2% |
| RGB, mixed content, 1080p | -4.9% | -4.8% | -4.9% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -12.3% | -12.3% | -12.3% |
| YUV, text & graphics with motion,720p | -7.2% | -7.5% | -7.4% |
| YUV, mixed content, 1440p | -4.8% | -4.9% | -4.9% |
| YUV, mixed content, 1080p | -5.6% | -5.5% | -5.6% |
| YUV, Animation, 720p | -0.2% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.5% | -6.3% | -6.4% |
| RGB, text & graphics with motion,720p | -4.3% | -4.4% | -4.4% |
| RGB, mixed content, 1440p | -2.6% | -2.6% | -2.6% |
| RGB, mixed content, 1080p | -3.2% | -3.1% | -3.2% |
| RGB, Animation, 720p | -0.2% | -0.2% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -7.0% | -6.9% | -7.0% |
| YUV, text & graphics with motion,720p | -4.9% | -5.0% | -5.1% |
| YUV, mixed content, 1440p | -3.1% | -3.3% | -3.3% |
| YUV, mixed content, 1080p | -4.0% | -3.8% | -4.0% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.8% | -4.5% | -4.5% |
| RGB, text & graphics with motion,720p | -2.1% | -2.0% | -1.9% |
| RGB, mixed content, 1440p | -1.6% | -1.8% | -1.6% |
| RGB, mixed content, 1080p | -1.7% | -1.5% | -1.8% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -5.0% | -4.9% | -4.9% |
| YUV, text & graphics with motion,720p | -2.1% | -1.7% | -2.2% |
| YUV, mixed content, 1440p | -1.6% | -2.2% | -2.0% |
| YUV, mixed content, 1080p | -1.7% | -2.6% | -0.9% |
| YUV, Animation, 720p | -0.1% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |

Experimental results are presented for the proposed BVD+MVD coding method investigated under SCC test conditions [1] on top of HM-14.0+RExt-7.0+SCC-1.0. Table 3 shows BD-rates and rate savings for RA and LD cases.

Table 3: MVD Results for lossy configuration

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.5% | -3.5% | -3.5% |
| RGB, text & graphics with motion,720p | -1.7% | -1.6% | -1.6% |
| RGB, mixed content, 1440p | -0.9% | -0.8% | -0.8% |
| RGB, mixed content, 1080p | -0.8% | -0.7% | -0.8% |
| RGB, Animation, 720p | -0.3% | -0.3% | -0.2% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -4.1% | -3.9% | -3.9% |
| YUV, text & graphics with motion,720p | -1.9% | -1.8% | -1.9% |
| YUV, mixed content, 1440p | -0.9% | -0.9% | -0.8% |
| YUV, mixed content, 1080p | -1.0% | -1.0% | -1.1% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.0% | -3.9% | -3.8% |
| RGB, text & graphics with motion,720p | -1.7% | -1.5% | -1.4% |
| RGB, mixed content, 1440p | -0.6% | -0.8% | -0.6% |
| RGB, mixed content, 1080p | -0.9% | -0.7% | -1.0% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.3% | -0.1% | -0.2% |
| YUV, text & graphics with motion, 1080p | -4.5% | -4.4% | -4.4% |
| YUV, text & graphics with motion,720p | -1.8% | -1.9% | -2.0% |
| YUV, mixed content, 1440p | -0.5% | -1.0% | -0.6% |
| YUV, mixed content, 1080p | -0.4% | -1.0% | -0.4% |
| YUV, Animation, 720p | 0.0% | -0.6% | -0.4% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

# Text specification

The text changes for the proposed method are marked in yellow.

## Syntax

### Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| …. |  |
| …. |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && ! intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag[ x0 ][ y0 ] && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| if( PartMode = = PART_2NxN ) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| else if( PartMode = = PART_Nx2N ) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| else if( PartMode = = PART_NxN ) { |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0, 2) |  |
| mvd_coding( x0, y0 + ( nCbS / 2 ), 2) |  |
| mvd_coding( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), 2) |  |
| } |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| …. |  |
| …. |  |
| …. |  |
| } |  |

### Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if (intra_bc_flag[ x0 ][ y0 ]){ /* Intra BC*/ |  |
| intra_bc_bvd_coding( x0, y0, 2) |  |
| } else { /* MODE_INTER */ |  |
| …. | ae(v) |
| …. |  |
| } |  |

#### IntraBC Block vector difference syntax

| intra_bc_bvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| intra_bc_abs_bvd_greater0_flag[ 0 ] | ae(v) |
| intra_bc_abs_bvd_greater0_flag[ 1 ] | ae(v) |
| if(intra_bc_abs_bvd_greater0_flag[ 0 ] ) |  |
| intra_bc_abs_bvd_greater16_flag[ 0 ] | ae(v) |
| if(intra_bc_abs_bvd_greater0_flag[ 1 ] ) |  |
| intra_bc_abs_bvd_greater16_flag[ 1 ] | ae(v) |
| if(intra_bc_abs_bvd_greater0_flag[ 0 ] ) { |  |
| if(intra_bc_abs_bvd_greater16_flag[ 0 ] ) |  |
| intra_bc_abs_bvd_minus2[ 0 ] | ae(v) |
| intra_bc_bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if(intra_bc_abs_bvd_greater0_flag[ 1 ] ) { |  |
| if(intra_bc_abs_bvd_greater1_flag[ 1 ] ) |  |
| intra_bc_abs_bvd_minus2[ 1 ] | ae(v) |
| intra_bc_bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

Semantics

intra_bc_abs_bvd_greater0_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 0.

intra_bc_abs_bvd_greater16_flag[ compIdx ] specifies whether the absolute value of a block vector component difference is greater than 16.

When intra_bc_abs_bvd_greater16_flag [ compIdx ] is not present, it is inferred to be equal to 0.

intra_bc_abs_bvd_minus2[ compIdx ] plus 1 specifies the absolute value of a block vector component difference.

When intra_bc_abs_bvd_minus2[ compIdx ] is not present, it is inferred to be equal to −1.

intra_bc_bvd_sign_flag[ compIdx ] specifies the sign of a block vector component difference as follows:

If intra_bc_bvd_sign_flag[ compIdx ] is equal to 0, the corresponding block vector component difference has a positive value.

Otherwise (intra_bc_bvd_sign_flag[ compIdx ] is equal to 1), the corresponding block vector component difference has a negative value.

When intra_bc_bvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

The block vector difference BvdIntra [ compIdx ] for compIdx = 0..1 is derived as follows:

BvdIntra [ compIdx ] = intra_bc_abs_bvd_greater0_flag [ compIdx ] *
		( intra_bc_abs_bvd_minus1 [ compIdx ] + 1 ) * 	( 1 − 2 * intra_bc_bvd_sign_flag [ compIdx ] )

| Table 9-1 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| intra_bc_bvd_coding ( ) | intra_bc_abs_bvd_greater0_flag[ ] | FL | cMax = 1 |
|  | intra_bc_abs_bvd_greater16_flag[ ] | FL | cMax = 1 |
|  | intra_bc_abs_bvd_minus1 [ ] | EG4 | - |
|  | intra_bc_bvd_sign_flag [ ] | FL | cMax = 1 |

The text changes for the proposed MVD coding method are marked in yellow.

| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1EG3 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, Apr. 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).