| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0182 |
| --- | --- |

| Title: | Non-SCCE1: On Intra block vector difference redundancy removal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Joel Sole Marta Karczewicz | Tel: Email: | +1-858-658-2042 krapaka@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method to remove redundancies in block vector difference coding. It is argued that when the current block’s BV predictor happens to be default predictor (no BV reconstruction required) there exist redundancies in the BVD which can be inferred. This inference is used to conditionally signal the sign and the BVD value of the vertical component thus removing redundancy in signalling. It is reported that the proposed approach over SCCE1 Test 3.1 (Encoder configuration 2) provides objective bit rate reductions of -0.3 % and -0.4% for All Intra 1080p text and graphics RGB and YUV categories.

# Proposed Method

There exist redundancies in the BVD when the current block’s BV predictor happens to be the default predictor. This contribution proposes changes to remove those redundancies.

In the figure 1 shown below, for the current block (marked in green), the blocks marked with yellow are valid BVD’s and those marked with blue are invalid as they fall in the non-decoded region. It may be observed, for example, for predictor’s p1 when the BVDx horizontal component is greater than width of the block, then in order for the BVD to not fall in the non-decoded region, the BVDy vertical component has to be negative. Additionally the BVDy components also need to be greater than height of the current block. Similar redundancies exist when BVDx is greater than zero for predictor’s p2 and p3.

Figure 1: An example IBC BVD distribution for a CU in a picture

In most of the BV prediction methods in the SCCE1, three different default predictors were proposed which are P1 = (-2*W, 0), P2= (-W, 0) and P3 = (2*W, 0). In the proposed method, when the current block’s BV predictor happens to be default predictor, simple checks are used to avoid signaling of redundant sign and the BVD value.

For P1, When BVDx is greater than W, sign is not signalled and BVDy minus height of the block is coded.

For P2 or P3, When BVDx is greater than zero, sign is not signalled and BVDy minus height of the block is coded.

Note that the checks do not introduce any parsing dependency and they only infer the availability and mode information of the blocks and are similar in nature to the checks performed for the parsing of some syntax elements in HEVC.

# Results

Experimental results are provided for the proposed method implemented on over SCCE1 Test 3.1 (Encoder configuration 2) under SCC test conditions [1]. Two configurations are investigated. For configuration 1, default predictors of (-2W, 0) and (-W, 0) are used and Table 1 shows BD-rates and rate savings.

For Configuration 2, default predictors of (-2W, 0) and (2W, 0) are used and Table 2 shows BD-rates and rate savings.

Table 1: Results for lossy All Intra configuration 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% |
| YUV, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.3% | -0.2% | -0.2% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: Results for lossy All Intra configuration 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.3% |
| RGB, text & graphics with motion,720p | -0.3% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% |
| YUV, mixed content, 1080p | -0.2% | 0.0% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

# References

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, Apr. 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).