| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0184 |
| --- | --- |

| Title: | SCCE1: Test 1.1 – Intra block copy with different search areas |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal reports results of SCCE1 Test 1.1 on Intra block copy with different search areas. In the test, both compression performance and memory access are evaluated and compared.

# Technical Description

In SCCE1 Test 1.1, different search areas are tested for Intra block copy, which includes

- Search area is 2 CTU (left and current)

- Search area is surrounding the current CTU with the size of 2×3 CTUs

- Search area is surrounding the current CTU with the size of 3×5 CTUs

- Search area is the entire frame

- Search area is the entire frame for intra frames and local search (e.g., 2 CTUs) for inter frames

For each tested search area, both compression performance and memory access (including memory read and write) are calculated and compared.

# Results

In this section, the experimental results of the proposed method are presented. All the simulations are conducted on top of SCM1.0, and SCC common test conditions [1] are used.

## Compression performance results

In this subsection, the objective compression performance results are presented for Intra block copy with different search areas. The BD-rates are provided for the lossy configuration and the rate savings are provided for the lossless configuration.

Table 1 Lossy results for a)

Table 2 Lossless results for a)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | -31.6% | -21.8% | -17.0% |
| RGB, text & graphics with motion,720p | -11.0% | -6.9% | -2.7% |
| RGB, mixed content, 1440p | -11.9% | -2.4% | -1.4% |
| RGB, mixed content, 1080p | -11.1% | -2.0% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -34.1% | -23.5% | -18.1% |
| YUV, text & graphics with motion,720p | -13.6% | -8.9% | -3.4% |
| YUV, mixed content, 1440p | -12.5% | -2.5% | -1.5% |
| YUV, mixed content, 1080p | -12.1% | -2.2% | -1.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% | 104% | 100% |
| Dec Time[%] | 103% | 105% | 100% |

Table 3 Lossy results for b)

Table 4 Lossless results for b)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | -19.5% | -13.4% | -9.9% |
| RGB, text & graphics with motion,720p | -6.8% | -4.4% | -2.0% |
| RGB, mixed content, 1440p | -9.8% | -1.9% | -1.0% |
| RGB, mixed content, 1080p | -8.0% | -1.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -21.3% | -14.6% | -10.8% |
| YUV, text & graphics with motion,720p | -8.5% | -5.8% | -2.5% |
| YUV, mixed content, 1440p | -10.0% | -1.9% | -1.0% |
| YUV, mixed content, 1080p | -8.4% | -1.5% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% | 110% | 102% |
| Dec Time[%] | 107% | 111% | 101% |

Table 5 Lossy results for c)

Table 6 Lossless results for c)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | -11.6% | -7.7% | -5.2% |
| RGB, text & graphics with motion,720p | -3.7% | -2.5% | -1.2% |
| RGB, mixed content, 1440p | -8.5% | -1.6% | -0.8% |
| RGB, mixed content, 1080p | -5.1% | -0.9% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -12.7% | -8.5% | -5.8% |
| YUV, text & graphics with motion,720p | -4.7% | -3.3% | -1.5% |
| YUV, mixed content, 1440p | -8.5% | -1.6% | -0.8% |
| YUV, mixed content, 1080p | -5.1% | -1.0% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 89% | 100% | 94% |
| Dec Time[%] | 95% | 102% | 94% |

Table 7 Lossy results for d)

Table 8 Lossless results for d)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% |

Table 9 Lossy results for e)

Table 10 Lossless results for e)

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 0.0% | -11.7% | -14.7% |
| RGB, text & graphics with motion,720p | 0.0% | -0.7% | -1.8% |
| RGB, mixed content, 1440p | 0.0% | -1.1% | -1.1% |
| RGB, mixed content, 1080p | 0.0% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -12.4% | -15.6% |
| YUV, text & graphics with motion,720p | 0.0% | -0.8% | -2.3% |
| YUV, mixed content, 1440p | 0.0% | -1.2% | -1.2% |
| YUV, mixed content, 1080p | 0.0% | -0.7% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 102% | 93% |
| Dec Time[%] | 99% | 106% | 96% |

## Memory access results

In this subsection, the average memory access results are presented for Intra block copy with different search areas. During the test, it is assumed that memory block size is 4x2 for both luma and chroma. For each tested search area, the following 8 different memory configurations are tested.

- Align  8bit/Burst  8bit

- Align 32bit/Burst 64bit

- Align 32bit/Burst128bit

- Align 64bit/Burst128bit

- Align 64bit/Burst256bit

- Align 64bit/Burst512bit

- Align 64bit/Burst256bit FIFOCache

- Align 64bit/Burst512bit FIFOCache

The detailed results can be found in the attached excel files.

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-Q1015, Valencia, ES, March 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).