| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0186 |
| --- | --- |

| Title: | SCCE1: Test 3.6 – Block vector coding for Intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Krishna Rapaka, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal reports results of SCCE1 Test 3.6 on block vector coding for Intra block copy. In the proposed method, the block vector difference coding is changed, while the block vector predictor keeps the same. The performance is reported to be an average BD-rate of -2.3% and -2.9%, respectively for 1080p RGB and YUV SC text & graphics with motion sequences under AI configuration.

# Technical Description

In the proposed technique, the block vector (BV) predictor is not changed. The resulting block vector difference (BVD) is coded as follows: first, one flag indicates whether BVD is zero. When BVD is not zero, Exp-Golomb codes with order 3 are used to encode the remaining absolute level of the BVD. At last, one flag is used to code its sign.

Compared with the current BVD coding method, which shares HEVC MVD coding, the number of regular bins has been reduced from 4 to 2.

# Results

In this section, the experimental results of the proposed method are presented, and BD-rates are provided for the lossy configuration (Table 1) and rate savings for the lossless configuration (Table 2). The method is tested on top of SCM1.0. SCC common test conditions [1] are used.

Table 1 Lossy results

Table 2 Lossless results

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 1.1% | 0.8% | 0.7% |
| RGB, text & graphics with motion,720p | 0.3% | 0.2% | 0.1% |
| RGB, mixed content, 1440p | 0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.2% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 0.9% | 0.7% |
| YUV, text & graphics with motion,720p | 0.4% | 0.2% | 0.1% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.2% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% | 104% | 100% |
| Dec Time[%] | 103% | 105% | 100% |

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-Q1015, Valencia, ES, March 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).