| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0187 |
| --- | --- |

| Title: | SCCE1: Test 5.2 – Combination of Test 3.1 and Test 3.6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Ying Chen, Krishna Rapaka, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal reports results of SCCE1 Test 5.1 on combination of Test 3.1 and Test 3.6. The performance is reported to be an average BD-rate of -10.9% and -12.0%, respectively for 1080p RGB and YUV SC text & graphics with motion sequences under AI configuration.

# Technical Description

In SCCE1 Test 5.2, the combination of Test 3.1 and Test 3.6 is tested. Test 3.1 is on block vector prediction method for Intra block copy, and Test 3.6 is on block vector difference coding for Intra block copy.

# Results

In this section, the experimental results of the combination of Test 3.1 and Test 3.6 are presented. All the simulations are conducted on top of SCM1.0, and SCC common test conditions [1] are used. The following three tests are conducted with different encoder optimization techniques.

- Test1 : lossy results (Table 1) and lossless results (Table 2) without encoder optimization

- Test2 : lossy results (Table 3) and lossless results (Table 4) with BV cost check at all possible search positions which has been reported by Canon in SCCE1 Test 3.3

- Test3 : lossy results (Table 5) and lossless results (Table 6) with BV cost check at all possible search positions and full RD check at BV predictor position which has been reported by Canon in SCCE1 Test 3.3

Table 1 Lossy results for Test 1

Table 2 Lossless results for Test 1

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 2.9% | 1.8% | 1.4% |
| RGB, text & graphics with motion,720p | 0.9% | 0.5% | 0.2% |
| RGB, mixed content, 1440p | 0.3% | 0.1% | 0.1% |
| RGB, mixed content, 1080p | 0.4% | 0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 3.0% | 1.9% | 1.4% |
| YUV, text & graphics with motion,720p | 0.9% | 0.6% | 0.2% |
| YUV, mixed content, 1440p | 0.4% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.5% | 0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 104% | 100% |
| Dec Time[%] | 101% | 106% | 99% |

Table 3 Lossy results for Test 2

Table 4 Lossless results for Test 2

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 7.0% | 4.6% | 3.7% |
| RGB, text & graphics with motion,720p | 2.1% | 1.3% | 0.6% |
| RGB, mixed content, 1440p | 1.1% | 0.3% | 0.2% |
| RGB, mixed content, 1080p | 1.5% | 0.3% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.4% | 4.9% | 3.8% |
| YUV, text & graphics with motion,720p | 2.5% | 1.6% | 0.6% |
| YUV, mixed content, 1440p | 1.3% | 0.3% | 0.3% |
| YUV, mixed content, 1080p | 1.6% | 0.3% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% | 104% | 100% |
| Dec Time[%] | 98% | 104% | 97% |

Table 5 Lossy results for Test 3

Table 6 Lossless results for Test 3

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 8.3% | 5.9% | 5.1% |
| RGB, text & graphics with motion,720p | 2.3% | 1.5% | 0.7% |
| RGB, mixed content, 1440p | 1.5% | 0.4% | 0.3% |
| RGB, mixed content, 1080p | 1.9% | 0.3% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 8.7% | 6.2% | 5.2% |
| YUV, text & graphics with motion,720p | 2.8% | 1.8% | 0.8% |
| YUV, mixed content, 1440p | 1.7% | 0.4% | 0.3% |
| YUV, mixed content, 1080p | 1.9% | 0.4% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% | 111% | 101% |
| Dec Time[%] | 95% | 106% | 97% |

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-Q1015, Valencia, ES, March 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).