| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0188 |
| --- | --- |

| Title: | Non-SCCE1: Memory bandwidth reduction for Intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Krishna Rapaka, Ted Hsieh, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In current SCM, the search region for Intra block copy is the entire frame. Consequently, compared to HEVC, this leads to about 10% memory bandwidth increase in the worst case. In this contribution, two different methods are proposed to keep worst-case memory bandwidth the same as HEVC.

# Introduction

In current SCM, the search region for Intra block copy (Intra BC) is the full frame, which is too large to be stored in the on-chip memory. Therefore, both the filtered and unfiltered reconstructed pixels need to be written to the off-chip memory, which results in increased memory bandwidth, including the worst-case, when comparing to HEVC.

# Memory bandwidth evaluation

In this evaluation, both memory read and write are taken into account in the bandwidth calculation. For memory read, bandwidth is calculated as number of reference pixels fetched from the memory based on 4×2 and 8×2 memory blocks as described in [1]. It assumes that memory can be accessed and read based on the memory blocks, the chroma storage is not interleaved and only integer number of blocks can be fetched. Let’s denote a block size to be compensated as M×N, memory block as m×n, and L is the interpolation filter length. Note that no interpolation is needed for Intra BC. For each pixel within M×N block the number of pixels per fetched from the memory can be calculated as follows.

For memory write, both the filtered and unfiltered reconstructed pixels are written to memory. Note that it is assumed no additional pixels are needed to be written to the memory.

In Table 1, we have listed the results of worst-case memory bandwidth for HEVC and SCM, which is 8×8 bi-predicted blocks, respectively. According to Table 1, the worst-case memory bandwidth is increased by 11% and 9% for 4×2 and 8×2 memory blocks in SCM.

Table 1 Worst-case memory bandwidth comparison of HEVC and SCM

| Memory configuration | HEVC | SCM |
| --- | --- | --- |
| 4×2 | 25.0 | 28.0 |
| 8×2 | 33.0 | 36.0 |

# Proposed

In this contribution, two methods are proposed to maintain the worst-case memory bandwidth as HEVC.

## Method 1

In the first method, it is restricted that the any reference pixel for Intra BC cannot be from bi-predicted blocks. Thus, the unfiltered reconstructed pixels are not needed to be written to the memory, and the worst-case memory bandwidth is the same as HEVC.

## Method 2

In the second method, a smaller search area (left and current CTB), which can be stored in on-chip memory, is used for Inter frames. In this case, comparing to HEVC, the worst-case memory bandwidth is not increased. This method has been tested in SCCE1 Test 1.1.

# Results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM1.0, and SCC common test conditions [1] are used. The BD-rates are provided for the lossy configuration and the rate savings are provided for the lossless configuration.

- Lossy results (Table 2) and lossless results (Table 3) for Method 1

- Lossy results (Table 4) and lossless results (Table 5) for Method 2

Table 2 Lossy results for Method 1

Table 3 Lossless results for method 1

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 0.0% | -4.7% | -3.9% |
| RGB, text & graphics with motion,720p | 0.0% | -0.4% | -0.5% |
| RGB, mixed content, 1440p | 0.0% | -0.2% | -0.4% |
| RGB, mixed content, 1080p | 0.0% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -5.3% | -5.3% |
| YUV, text & graphics with motion,720p | 0.0% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 109% | 102% |
| Dec Time[%] | 101% | 108% | 101% |

Table 4 Lossy results for method 2

Table 5 Lossless results for method 2

| Bit-rate saving (Average) | AI | RA | LB |
| --- | --- | --- | --- |
| RGB, text & graphics with motion, 1080p | 0.0% | -11.7% | -14.7% |
| RGB, text & graphics with motion,720p | 0.0% | -0.7% | -1.8% |
| RGB, mixed content, 1440p | 0.0% | -1.1% | -1.1% |
| RGB, mixed content, 1080p | 0.0% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -12.4% | -15.6% |
| YUV, text & graphics with motion,720p | 0.0% | -0.8% | -2.3% |
| YUV, mixed content, 1440p | 0.0% | -1.2% | -1.2% |
| YUV, mixed content, 1080p | 0.0% | -0.7% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 102% | 93% |
| Dec Time[%] | 99% | 106% | 96% |

# Draft Text Changes

The proposed text changes are based on the document JCTVC-P1005. The changes are marked in yellow.

## Method 1

8.4.4 Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the (nCbS)x(nCbX) array [Ed. some callees still call this a single vector] of block vectors bvIntra.

The variables nCbS, nPbSw, and nPbSh are derived as follows:

nCbS = 1  <<  log2CbSize		(8-1)

nPbSw = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_2NxN ? 1 : 2 )	(8-2)

nPbSh = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_Nx2N ? 1 : 2 )	(8-3)

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

Depending upon PartMode, the variable numPartitions is derived as follows:

–	If PartMode is equal to PART_2Nx2N, numPartitions is set equal to 1.

–	Otherwise, if PartMode is equal to either PART_2NxN or PART_Nx2N, numPartitions is set equal to 2.

–	Otherwise (PartMode is equal to PART_NxN), numPartitions is set equal to 4.

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] − nCbS	(8-4)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ]	(8-5)

- Otherwise, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] + BvpIntra[ 0 ]	(8-6)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ] + BvpIntra[ 1 ]	(8-7)

- The value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]	(8-8)

It is a requirement of bitstream conformance that all of the following conditions are true:

–	If the slice type is not I, The value of bvIntra[ xPb ][ yPb ][ 0 ] shall be greater than or equal to – ( xPb % CtbSizeY + 64 ).

–	The value of bvIntra[ xPb ][ yPb ][ 1 ] shall be greater than or equal to – ( yPb % CtbSizeY ).

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, the output shall be equal to TRUE.

–	One or both of the following conditions shall be true:

–	bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw  <=  0

–	bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh  <=  0

- inter_pred_idc[ x ][ y ] shall be equal to PRED_L0 or PRED_L1, with x= xPb + BvIntra[ xPb ][ yPb ][ 0 ] . xPb + BvIntra[ xPb ][ yPb ] [ 0 ]+ (1 << log2CbSize) – 1, y= yPb + BvIntra[ xPb ][ yPb ] [ 1 ].. y0+ BvIntra[ xPb ][ yPb ] [ 1 ]+ (1 << log2CbSize) – 1 ;

## Method 2

8.4.4 Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the (nCbS)x(nCbX) array [Ed. some callees still call this a single vector] of block vectors bvIntra.

The variables nCbS, nPbSw, and nPbSh are derived as follows:

nCbS = 1  <<  log2CbSize		(8-9)

nPbSw = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_2NxN ? 1 : 2 )	(8-10)

nPbSh = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_Nx2N ? 1 : 2 )	(8-11)

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

Depending upon PartMode, the variable numPartitions is derived as follows:

–	If PartMode is equal to PART_2Nx2N, numPartitions is set equal to 1.

–	Otherwise, if PartMode is equal to either PART_2NxN or PART_Nx2N, numPartitions is set equal to 2.

–	Otherwise (PartMode is equal to PART_NxN), numPartitions is set equal to 4.

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] − nCbS	(8-12)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ]	(8-13)

- Otherwise, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] + BvpIntra[ 0 ]	(8-14)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ] + BvpIntra[ 1 ]	(8-15)

- The value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]	(8-16)

It is a requirement of bitstream conformance that all of the following conditions are true:

–	For P and B slice types, tThe value of bvIntra[ xPb ][ yPb ][ 0 ] shall be greater than or equal to – ( xPb % CtbSizeY + 64 ).

–	For P and B slice types, tThe value of bvIntra[ xPb ][ yPb ][ 1 ] shall be greater than or equal to – ( yPb % CtbSizeY ).

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ], yPb + bvIntra[ xPb ][ yPb ][ 1 ] ) as inputs, the output shall be equal to TRUE.

–	When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbY, yNbY ) set equal to ( xPb + bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw − 1, yPb + bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh – 1 ) as inputs, the output shall be equal to TRUE.

–	When the slice type is I and entropy_coding_sync_enabled_flag is equal to 1, the following condition shall be true

–	The value of xPb % CtbSizeY + bvIntra[ xPb ][ yPb ][ 0 ]  shall be less than or equal to  (CurrCtby – RefCtby) << 1

where CurrCtby =  (yPb/ CtbSizeY) * CtbSizeY

RefCtby = ((yPb + bvIntra[ xPb ][ yPb ][ 1 ])/ CtbSizeY) * CtbSizeY

–	One or both of the following conditions shall be true:

–	bvIntra[ xPb ][ yPb ][ 0 ] + nPbSw  <=  0

–	bvIntra[ xPb ][ yPb ][ 1 ] + nPbSh  <=  0

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-Q1015, Valencia, ES, March 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).