| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0189 |
| --- | --- |

| Title: | Non-SCCE1: Intra block copy deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Ying Chen, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In current SCM, Intra block copy is treated as Intra during the deblocking process. In this contribution, it is proposed to treat Intra block copy as Inter instead. The performance is reported to be an average BD-rate of -3.5% and -3.4% respectively for U and V components of 1080p YUV SC mixed content sequences under LB configuration and no BD-rate penalty for other sequences categories defined in SCC common test condition.

# Introduction

In modern video codecs, deblocking applies to the boundaries of coded blocks. In HEVC version 1, whenever Intra coded block is involved, a stronger filter applies (with Bs value being 2). When Intra coded block is not involved, if the reference picture and motion vector are both the same and there is no non-zero residual coefficient, no filtering is needed (with Bs value being 0). Otherwise, filtering is needed but with a relatively less strength value (with Bs value being 1) compared to the case for Intra block boundaries.

However, as Intra BC often selects the region which contains sharp edges (e.g., text), it may be not appropriate to use strong deblocking filters as Intra to filter these edges, especially for the boundary of two Intra BC blocks, which may result in worse subjective and objective video quality. Similar situation happens along the boundary of an Inter coded block and an Intra BC coded block.

Typically, in video coding extensions, it is preferred not to introduce changes for de-blocking module itself as such change might cause noticeable implementation burden.

# Proposal

In this contribution, the Intra BC is treated as Inter during the deblocking process instead. Moreover, in order to keep the deblocking module unchanged, after an Intra BC block is decoded and before the deblocking process, it is converted to be a uni-directionally predictive Inter block with the block vector being its motion vector and the current picture being the reference picture.

# Results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM1.0, and SCC common test conditions [1] are used. The BD-rates are provided for the lossy configuration (Table 1). The proposed method doesn’t affect the lossless case.

Table 1 Lossy results

# Draft Text Changes

The proposed text changes are based on the document JCTVC-P1005. The changes are marked in yellow.

8.4.4 Derivation process for block vector components in intra block copying prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is the (nCbS)x(nCbX) array of block vectors bvIntra.

The variables nCbS, nPbSw, and nPbSh are derived as follows:

nCbS = 1  <<  log2CbSize		(8-1)

nPbSw = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_2NxN ? 1 : 2 )	(8-2)

nPbSh = nCbS / ( PartMode = = PART_2Nx2N  | |  PartMode = = PART_Nx2N ? 1 : 2 )	(8-3)

The variable BvpIntra[ compIdx ] specifies a block vector predictor. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

Depending upon PartMode, the variable numPartitions is derived as follows:

–	If PartMode is equal to PART_2Nx2N, numPartitions is set equal to 1.

–	Otherwise, if PartMode is equal to either PART_2NxN or PART_Nx2N, numPartitions is set equal to 2.

–	Otherwise (PartMode is equal to PART_NxN), numPartitions is set equal to 4.

The array of block vectors bvIntra is derived by the following ordered steps, for the variable blkIdx proceeding over the values 0..( numPartitions − 1 ):

- The variable blkInc is set equal to ( PartMode = = PART_2NxN ? 2 : 1 ).

- The variable xPb is set equal to xCb + nPbSw * ( blkIdx * blkInc % 2 ).

- The variable yPb is set equal to yCb + nPbSh * ( blkIdx / 2 )

- The following ordered steps apply, for the variable compIdx proceeding over the values 0..1:

- Depending upon the number of times this process has been invoked for the current coding tree unit, the following applies:

- If this process is invoked for the first time for the current coding tree unit, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] − nCbS	(8-4)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ]	(8-5)

- Otherwise, bvIntra[ xPb ][ yPb ][ compIdx ] is derived as follows:

bvIntra[ xPb ][ yPb ][ 0 ] = BvdIntra[ xPb ][ yPb ][ 0 ] + BvpIntra[ 0 ]	(8-6)

bvIntra[ xPb ][ yPb ][ 1 ] = BvdIntra[ xPb ][ yPb ][ 1 ] + BvpIntra[ 1 ]	(8-7)

- The value of BvpIntra[ compIdx ] is updated to be equal to bvIntra[ xPb ][ yPb ][ compIdx ].

- For use in derivation processes of variables invoked later in the decoding process, the following assignments are made for x = 0..nPbSw − 1 and y = 0..nPbSh − 1:

bvIntra[ xPb + x ][ yPb + y ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ]

- The prediction list utilization flags predFlagL0 and predFlagL1 the reference indices refIdxL0 and refIdxL1 and luma motion vectors mvL0 and mvL1are derived as

predFlagL0[ xPb ][ yPb ] = 1

predFlagL1[ xPb ][ yPb ] = 0

mvL0[ xPb ][ yPb ][ compIdx ] = bvIntra[ xPb ][ yPb ][ compIdx ] << 2

mvL1[ xPb ][ yPb ][ compIdx ] = 0

refIdxL0[ xPb ][ yPb ] = num_ref_idx_l0_active_minus1 + 1

refIdxL1[ xPb ][ yPb ] = -1

- CuPredMode[ xPb ][  yPb ] is set to MODE_INTER.

# References

[1] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-Q1015, Valencia, ES, March 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).