| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0190 |
| --- | --- |

| Title: | SCCE1 Test 2.1: IntraBC coded as Inter PU |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  | Shawmin Lei |  |
| Author(s) or Contact(s): | Xiaozhong Xu Shan Liu Shawmin Lei | Tel: Email: |  | +886-3-5670766 ext. 25555 xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek |  |  |  |

_____________________________

# Abstract

In this contribution, the intra block copy operation (IntraBC) is operated at PU level, to align with a motion compensated PU in inter slice. AMVP is used for PU level BV prediction, as specified in SCCE1 Test 3.4 (JCTVC-R0061). Inter PU syntax is used for IntraBC signaling. With the use of 4x4 IntraBC mode, it is reported that the proposed approach provides objective bitrate reductions of -3.9 %, -4.6 % for All Intra 1080p text and graphics RGB and YUV categories, respectively; bitrate reductions of -2.0 %, -1.8 % for Random Access 1080p text and graphics RGB and YUV categories, respectively; bitrate reductions of -1.6 %, -1.2 % for Low delay B 1080p text and graphics RGB and YUV categories, respectively

# Introduction

In this contribution, the following changes are made:

- The spatial AMVP is used for BV prediction for intra block copy [1].

- Use prediction_unit() syntax to signal the usage of IntraBC.

  - The partition mode coding of IntraBC is unified with inter partition modes.

  - The vector prediction of IntraBC is unified with spatial AMVP for inter PU vector prediction.

  - The mvd coding of IntraBC is unified with inter mvd coding.

  - IntraBC mode is treated as an inter mode with inter_pred_idc equals to 0 (L0). In this mode, pred_mode_flag = 0 in intra slice; pred_mode_flag = 0 and ref_idx_l0 = 1 in B and P slices.

  - 4x4 partition mode is turn on or off for IntraBC.

# Results

The proposed method is integrated on HM-14.0+RExt-7.0+SCM1.0 [2].	The simulations are performed under the updated CTC [3]. Two set of results are presented. In simulation A, 4x4 IntraBC mode is allowed. In simulation B, 4x4 IntraBC mode is not allowed.

Table 1: Results for lossy configuration, with 4x4 IntraBC

Table 2: Results for lossless configuration, with 4x4 IntraBC

Table 3: Results for lossy configuration, without 4x4 IntraBC

Table 4: Results for lossless configuration, without 4x4 IntraBC

# Reference

[1] X. Xu, S. Liu and S. Lei, “SCCE1 Test 3.4: IntraBC BV prediction”, JCTVC-R0016, June 2014

[2] HM-14.0+RExt-7.0+SCM-1.0, https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-14.0%2BRExt-7.0%2BSCM-1.0

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, JCTVC-Q1015, Apr. 2014

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).