| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0197 |
| --- | --- |

| Title: | Crosscheck of SCCE4 Test 3.2 CU based 1D Dictionary with only 2CTU reference |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Jing Ye, Shan Liu 2860 Junction Ave, San Jose, CA, 95134 | Tel: Email: | +1-(408) 526-1899 ext88134 jing.ye@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of SCCE4 Test 3.2 CU based 1D Dictionary with only 2CTU reference JCTVC-R0140 [2]. The software package provided by the proponent was tested under the screen content common test conditions specified in JCTVC-Q1015 [1]. The BD-rate results perfectly match the data provided by the proponent while decrease of encoding time (-10%)) relative to the proponent’s results is observed. Table 1 and  show the simulation results for lossy and lossless coding, respectively. Those results are under full frame IBC.

# Test results

Table 1. Simulation results of SCCE4 Test 3.2 for lossy coding.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -11.7% | -12.0% | -12.0% |
| RGB, text & graphics with motion,720p | -5.1% | -5.1% | -5.2% |
| RGB, mixed content, 1440p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1080p | -1.5% | -1.5% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -10.5% | -11.3% | -10.7% |
| YUV, text & graphics with motion,720p | -3.7% | -5.1% | -6.4% |
| YUV, mixed content, 1440p | -0.3% | -0.9% | -0.7% |
| YUV, mixed content, 1080p | -0.7% | -0.9% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 179% |  |  |
| Dec Time[%] | 106% |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.9% | -7.3% | -7.4% |
| RGB, text & graphics with motion,720p | -5.0% | -4.7% | -5.0% |
| RGB, mixed content, 1440p | -0.9% | -0.8% | -0.8% |
| RGB, mixed content, 1080p | -1.8% | -1.9% | -1.6% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -6.7% | -7.8% | -7.5% |
| YUV, text & graphics with motion,720p | -4.4% | -5.8% | -7.0% |
| YUV, mixed content, 1440p | -0.3% | -1.4% | -1.2% |
| YUV, mixed content, 1080p | -1.0% | -2.1% | -1.7% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.2% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 113% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.5% | -3.9% | -3.9% |
| RGB, text & graphics with motion,720p | -2.9% | -2.8% | -2.8% |
| RGB, mixed content, 1440p | -0.6% | -0.8% | -0.5% |
| RGB, mixed content, 1080p | -0.9% | -0.7% | -1.2% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.8% | -4.1% | -3.7% |
| YUV, text & graphics with motion,720p | -2.9% | -3.8% | -4.8% |
| YUV, mixed content, 1440p | -0.7% | -1.4% | -1.5% |
| YUV, mixed content, 1080p | -0.3% | -1.5% | 0.0% |
| YUV, Animation, 720p | 0.1% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 108% |  |  |

Table 2. Simulation results of SCCE4 Test 3.2 for lossless coding.

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 18.5% | 19.0% | 9.9% | 27.3% |
| RGB, text & graphics with motion,720p | 12.3% | 10.0% | 0.8% | 21.4% |
| RGB, mixed content, 1440p | 1.6% | 1.9% | 0.7% | 3.0% |
| RGB, mixed content, 1080p | 1.8% | 1.8% | 1.8% | 1.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 20.9% | 21.0% | 8.2% | 31.9% |
| YUV, text & graphics with motion,720p | 10.7% | 9.9% | 0.6% | 18.4% |
| YUV, mixed content, 1440p | 1.4% | 1.7% | 0.3% | 3.0% |
| YUV, mixed content, 1080p | 1.7% | 1.7% | 1.7% | 1.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 241% |  |  |  |
| Dec Time[%] | 105% |  |  |  |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 7.4% | 11.3% | 6.9% | 18.5% |
| RGB, text & graphics with motion,720p | 2.0% | 4.7% | 0.4% | 8.8% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.2% | 0.3% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.9% | 12.9% | 5.6% | 22.0% |
| YUV, text & graphics with motion,720p | 1.6% | 5.3% | 0.3% | 12.6% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 88.0% | 39.5% | 7.3% | 100.0% |
| RGB, text & graphics with motion,720p | 1.4% | 3.3% | 0.3% | 6.2% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 85.8% | 41.1% | 9.4% | 100.0% |
| YUV, text & graphics with motion,720p | 1.0% | 3.6% | 0.3% | 8.3% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Reference

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, Mar. 2014.

- Feng Zou, Ying Chen, Joel Sole, Marta Karczewicz, “SCCE4: Subset 3.2 CU based 1D Dictionary with only 2CTU reference” JCTVC-R0140, Sapporo, JP, 30 June, 2014.