| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0200_r3 |
| --- | --- |

| Title: | SCCE3 Test D.2: Independent Uniform Prediction (IUP) Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xingyu Zhang Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway, Cambridge, MA 02139, U.S.A | Tel: Email: | +1-617-621-7500 xzhang@merl.com cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories (MERL) |  |  |

_____________________________

# Abstract

This document proposes the Independent Uniform Prediction (IUP) mode as studied in SCCE3 Test D2. IUP targets flat or mostly flat areas in a picture by predicting a CU using a uniform color. The improvements in average BD-Rates for AI test conditions for RGB text & graphics with motion are:

Without palette in anchor and tested configurations:

SCM 2-CTU:	1.6% G, 1.5% Y for 1080p; 1.5% G, 1.5% Y for 720p

SCM Full:		1.4% G, 1.4% Y for 1080p; 1.3% G, 1.5% Y for 720p

With palette in anchor and tested configurations:

SCCE3 2-CTU:	0.6% G, 0.5% Y for 1080p; 0.9% G, 0.9% Y for 720p

SCCE3 Full:	0.6% G, 0.5% Y for 1080p; 1.0% G, 0.9% Y for 720p

Encoder run-time ratios range from 95% to 102% and decoder run-time ratios range from 94% to 101%.

Additional experiments were performed during the JCT-VC meeting in which for a given slice, the uniform colors available to IUP are computed from the previous slice, not from the current slice. This is an encoder-only change, i.e. the colors available to a given slice are still signaled within that slice. The improvements in average BD-Rates for these additional experiments, for AI test conditions for RGB text & graphics with motion are:

Without palette in anchor and tested configurations:

SCM Full:		1.2% G, 1.3% Y for 1080p; 1.3% G, 1.5% Y for 720p

# Independent Uniform Prediction mode

The prediction modes currently available in HEVC-RExt are based primarily on data available from nearby previously-decoded blocks. Intra-block directional prediction is based upon adjacent pixels from neighboring blocks. Intra block copy mode and motion estimation for Inter-block coding applies a limited-range search to nearby pixels in the current frame or in a picture reference buffer. Unlike typical camera-captured videos, video from a computer, or screen content video, often follows a consistent theme. This theme defines graphical elements such as background color, window colors and border shapes. Most of the existing Intra prediction schemes, due to their localized prediction process, cannot exploit the commonalities in a theme globally over the entire picture or sequence of pictures, due to the localized nature of the Intra and Inter prediction process. The recently proposed palette modes exploit the reduced number of colors that screen content pictures typically contain, however, the palette is adapted while coding the picture, and in recent implementations, is reset when starting each row of CTBs of a slice. This local adaptation is useful when the palette or set of used colors is consistent over an entire row of CTBs or picture, but when a picture comprises several similar but distant regions, the palette may not be optimal when entering each region. The purpose of the independent uniform prediction (IUP) mode, originally proposed in JCTVC-Q0036 [1], is to compute a small set of global block predictors to target regions of a picture that have a limited number of commonly-occurring colors that are typical in a computing environment, such as background colors. This mode is available for coding blocks in both Inter and Intra slices.

In the slice encoding process, before CTUs in a slice are being encoded, a histogram of the pixel values (RGB or YCbCr triplets) is maintained. To limit the complexity of this process, only the center pixel from each CTU is used to compute the histogram. Before encoding the slice, these triplets form the set S1. The num_uniform_colors most frequently occurring triplets are saved as set S2. The number of colors num_uniform_colors and the color triplet values are signaled in the slice header.

In the slice decoding process, the num_uniform_colors and the color triplet values are decoded from slice header. The CU-level decoding process is shown in Figure 1. A flag independent_uniform_pred_flag is parsed from the bitstream to indicate whether the independent uniform prediction mode is used for the current CU. If independent_uniform_pred_flag is equal to 1 and num_uniform_colors >1, then uniform_color_index is parsed from the bitstream to indicate which color triplet from set S2 is used in the current CU. If independent_uniform_pred_flag is true and num_uniform_colors ==1, then uniform_color_index is inferred to be 1.

Figure 1. CU decoding process for independent uniform prediction

For predicting a CU using the independent uniform prediction mode, a uniform predictor block is formed for each color component, where all pixels in the predictor block have the same value as the corresponding element in the selected color triplet. This predictor block is then treated the same way as in other prediction methods, where the predictor is added to the reconstructed residual block to form the reconstructed block.

Because the independent uniform prediction mode targets very commonly used colors used over an entire picture, we limited num_uniform_colors in this proposal to 2. Additional details on the use of IUP mode and the color section can be found in JCTVC-Q0036 [1].

- Experimental Results

The test conditions specified by the SCCE3 description [2] were used for these experiments.

## Lossy coding results

### SCM-1.0 common test condition with intraBC using full frame search

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.4% | -1.5% | -1.4% | -0.5% | -0.5% | -0.5% | -0.1% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | -1.3% | -1.6% | -1.5% | -0.7% | -0.8% | -0.9% | -0.1% | -0.3% | -0.2% |
| RGB, mixed content, 1440p | -0.9% | -0.8% | -0.8% | -0.4% | -0.3% | -0.3% | -0.4% | -0.5% | -0.3% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% | -0.3% | -0.3% | -0.4% | -0.3% | 0.0% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.4% | -1.4% | -1.4% | -0.6% | -0.7% | -0.6% | -0.2% | -0.2% | -0.1% |
| YUV, text & graphics with motion,720p | -1.5% | -1.9% | -2.2% | -0.8% | -1.2% | -1.2% | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p | -0.6% | -0.9% | -0.9% | -0.2% | -0.6% | -0.6% | -0.2% | -0.5% | -0.4% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.7% | -0.3% | -0.8% | -0.6% | 0.2% | -0.7% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  | 101% |  |  |

### SCM-1.0 common test condition with intraBC using full frame search + JCTVC-Q0094 (Palette mode) + bug-fixes of JCTVC-Q0047

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.6% | -0.6% | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| RGB, text & graphics with motion,720p | -1.0% | -1.1% | -1.1% | -0.4% | -0.5% | -0.4% | 0.3% | 0.1% | 0.2% |
| RGB, mixed content, 1440p | -0.6% | -0.6% | -0.6% | -0.2% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% | -0.2% | -0.1% | -0.2% | -0.1% | 0.1% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% | -0.1% | -0.1% | -0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | -0.9% | -1.5% | -1.4% | -0.4% | -0.8% | -0.8% | 0.1% | 0.2% | -0.3% |
| YUV, mixed content, 1440p | -0.5% | -0.6% | -0.6% | -0.2% | -0.1% | -0.3% | -0.2% | -0.4% | -0.2% |
| YUV, mixed content, 1080p | -0.4% | -0.4% | -0.4% | 0.0% | -0.1% | -0.1% | 0.0% | -1.7% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  | 100% |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.6% | -1.7% | -1.6% | -0.6% | -0.6% | -0.5% | -0.2% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | -1.5% | -1.6% | -1.5% | -0.8% | -0.9% | -0.9% | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p | -1.0% | -0.9% | -0.9% | -0.5% | -0.4% | -0.5% | -0.5% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.2% | -0.3% | -0.3% | 0.1% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.5% | -1.5% | -0.6% | -0.7% | -0.7% | -0.1% | -0.2% | 0.0% |
| YUV, text & graphics with motion,720p | -1.5% | -1.8% | -2.1% | -0.9% | -1.4% | -1.4% | -0.1% | -0.4% | -0.5% |
| YUV, mixed content, 1440p | -0.7% | -0.9% | -0.9% | -0.4% | -0.6% | -0.5% | -0.4% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.8% | -1.0% | -0.9% | -0.5% | -0.6% | -0.6% | 0.0% | 0.5% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 100% |  |  | 101% |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU) + JCTVC-Q0094 (Palette Mode) + bug-fixes of JCTVC-Q0047

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.5% | -0.6% | -0.1% | -0.1% | -0.1% | 0.1% | 0.2% | 0.1% |
| RGB, text & graphics with motion,720p | -0.9% | -0.9% | -0.9% | -0.3% | -0.4% | -0.4% | 0.1% | 0.2% | 0.2% |
| RGB, mixed content, 1440p | -0.7% | -0.7% | -0.7% | -0.2% | -0.2% | -0.2% | -0.2% | 0.0% | -0.1% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.5% | -0.2% | -0.2% | -0.2% | 0.0% | -0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion,720p | -0.9% | -1.2% | -1.2% | -0.5% | -0.7% | -0.8% | 0.3% | -0.2% | 0.1% |
| YUV, mixed content, 1440p | -0.6% | -0.6% | -0.5% | -0.2% | -0.3% | -0.3% | -0.1% | 0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.4% | -0.5% | -0.4% | 0.0% | -0.3% | -0.2% | 0.0% | -0.4% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  | 100% |  |  |

## Lossless coding results

### 2.2.1 SCM-1.0 common test condition with intraBC using full frame search

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.4% |  | 0.4% |  | 0.1% |  | 0.6% |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.0% |  | -0.1% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.5% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.3% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.1% |
| RGB, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.2% |  | 0.2% |  | 0.1% |  | 0.5% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.0% |  | -0.1% |  | 0.1% |
| YUV, text & graphics with motion,720p |  |  | 0.2% |  | 0.3% |  | 0.1% |  | 0.7% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.4% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.0% |
| YUV, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 99% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |  | 96% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 94% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |  | 98% |  |  |  |  |  |  |

### SCM-1.0 common test condition with intraBC using full frame search + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.2% |  | 0.2% |  | 0.1% |  | 0.4% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | -0.1% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.4% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.2% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.0% |
| RGB, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.2% |  | 0.2% |  | 0.1% |  | 0.3% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.1% |
| YUV, text & graphics with motion,720p |  |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.5% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.3% |  | 0.0% |  | 0.0% |  | -0.2% |  | 0.0% |
| YUV, mixed content, 1440p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 95% |  |  |  |  |  |  |  | 102% |  |  |  |  |  |  |  | 97% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 97% |  |  |  |  |  |  |  | 100% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU)

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.5% |  | 0.5% |  | 0.2% |  | 0.7% |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.3% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.2% |  | 0.3% |  | 0.1% |  | 0.7% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.4% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.1% |
| RGB, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.3% |  | 0.3% |  | 0.1% |  | 0.6% |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.3% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion,720p |  |  | 0.2% |  | 0.3% |  | 0.1% |  | 0.8% |  | 0.0% |  | 0.2% |  | 0.0% |  | 0.6% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.0% |
| YUV, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 99% |  |  |  |  |  |  |  | 101% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 94% |  |  |  |  |  |  |  | 102% |  |  |  |  |  |  |  | 98% |  |  |  |  |  |  |

### SCM-1.0 common test condition with intraBC using 2-CTU search (i.e., current CTU and left CTU) + JCTVC-Q0094 + bug-fixes of JCTVC-Q0047

|  |  |  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  |  |  |  |  | Low Delay B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |  | Bit-rate saving (Total) |  | Bit-rate saving (Average) |  | Bit-rate saving (Min) |  | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.2% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.1% |  | 0.0% |  | -0.2% |  | 0.1% |
| RGB, text & graphics with motion,720p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.2% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | -0.2% |  | 0.0% |
| RGB, mixed content, 1440p |  |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, mixed content, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.1% |  | 0.0% |  | -0.1% |  | 0.1% |
| YUV, text & graphics with motion,720p |  |  | 0.1% |  | 0.1% |  | 0.1% |  | 0.3% |  | 0.0% |  | 0.0% |  | -0.1% |  | 0.1% |  | 0.0% |  | 0.0% |  | -0.2% |  | 0.0% |
| YUV, mixed content, 1440p |  |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, mixed content, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, Animation, 720p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| YUV, camera captured, 1080p |  |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] |  |  | 96% |  |  |  |  |  |  |  | 98% |  |  |  |  |  |  |  | 97% |  |  |  |  |  |  |
| Dec Time[%] |  |  | 98% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |  | 99% |  |  |  |  |  |  |

# Modifications to working draft text

Modifications to the working draft text are provided in the accompanying files JCTVC-R0200-WDtext_P1005_based.docx and JCTVC-R0200-WDtext_PLT_based.docx.

# Additional results using computed colors on the next slice

Instead of using pixels in the current slice to select the uniform colors available for use by IUP, the uniform colors are used on the next slice. This is an encoder-only change, i.e. the signaling is identical to the original proposal, and the uniform colors used for a slice are still signaled within that slice. For the first slice, since no previous slice is available, the encoder can specify that no colors are available to IUP, or it can specify some arbitrary fixed colors. All-intra lossy coding BD-Rate changes are shown for two examples.

## SCM-1.0 common test condition with intraBC using full frame search and num_uniform_colors = 0 for IUP

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -1.3% | -1.6% | -1.5% |
| RGB, mixed content, 1440p | -0.9% | -0.8% | -0.8% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.3% | -1.3% | -1.3% |
| YUV, text & graphics with motion,720p | -1.5% | -1.9% | -2.2% |
| YUV, mixed content, 1440p | -0.6% | -0.9% | -0.9% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] |  |  |  |

## SCM-1.0 common test condition with intraBC using full frame search, num_uniform_colors = 1 for IUP and the uniform color is 255 for all components (white)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.2% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -1.3% | -1.6% | -1.5% |
| RGB, mixed content, 1440p | -0.9% | -0.8% | -0.8% |
| RGB, mixed content, 1080p | -0.6% | -0.6% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.3% | -1.3% | -1.3% |
| YUV, text & graphics with motion,720p | -1.5% | -1.9% | -2.2% |
| YUV, mixed content, 1440p | -0.6% | -0.9% | -0.9% |
| YUV, mixed content, 1080p | -0.6% | -0.8% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] |  |  |  |

- Conclusions

Independent Uniform Prediction (IUP) mode is capable of providing improvements in compression efficiency both with and without palette mode, with no increase in encoder or decoder run times. The improvements in average BD-Rates for AI test conditions for RGB text & graphics with motion are:

Without palette in anchor and tested configurations:

SCM 2-CTU:	1.6% G, 1.5% Y for 1080p; 1.5% G, 1.5% Y for 720p

SCM Full:		1.4% G, 1.4% Y for 1080p; 1.3% G, 1.5% Y for 720p

With palette in anchor and tested configurations:

SCCE3 2-CTU:	0.6% G, 0.5% Y for 1080p; 0.9% G, 0.9% Y for 720p

SCCE3 Full:	0.6% G, 0.5% Y for 1080p; 1.0% G, 0.9% Y for 720p

Encoder run-time ratios range from 95% to 102% and decoder run-time ratios range from 94% to 101%.

Additional experiments were performed during the JCT-VC meeting in which for a given slice, the uniform colors available to IUP are computed from the previous slice, not from the current slice. This is an encoder-only change, i.e. the colors available to a given slice are still signaled within that slice. The improvements in average BD-Rates for these additional experiments, for AI test conditions for RGB text & graphics with motion are:

Without palette in anchor and tested configurations:

SCM Full:		1.2% G, 1.3% Y for 1080p; 1.3% G, 1.5% Y for 720p

The performance from the modified IUP color selection method is similar to that of the originally used method. Encoder run-time ratios range from 95% to 102% and decoder run-time ratios range from 94% to 101%. We propose that IUP mode be added to the screen-content coding extension.

# References

- R. Cohen, X. Zhang, A. Vetro, K. Sugimoto, A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami, “Description of screen content coding technology proposal by Mitsubishi Electric Corporation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q0036, 17th Meeting: Valencia, ES, 27 March – 4 April, 2014.

- Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Q1123, 17th Meeting: Valencia, ES, 27 March – 4 April, 2014.

# Patent rights declaration

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).