| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0202 |
| --- | --- |

| Title: | Non-SCCE3: Copy from previous row mode for palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Marta Karczewicz Rajan Joshi Joel Sole | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal presents a “copy from previous rows” mode for the palette coding. The proposed method enables to copy pixels from previously coded rows beyond the row above in the current CU. This is defined as a palette mode besides the existing “Copy from left”, “Copy above” and “Escape” modes. The reference row index is coded in the bitstream to indicate which row is copied from. The simulation results report that the proposed method achieves 0.9% and 1.0% BR-rate savings for 1080p text and graphics RGB and YUV respectively against the SCCE3 anchor in the full frame intra BC test condition.

# Introduction

In the Valencia meeting, SCCE3 [1] was established to study different palette coding methods. In SCCE3 palette reference software, there are three modes available: “Copy from left” (index coding), “Copy above” and “Escape” modes. To further improve the coding efficiency of palette coding, other previously coded pixels in the current CU can be utilized as reference instead of only using the nearest left/above pixel, which are the case in existing “Copy from left” and “Copy from above” modes.

# Proposed copy from previous row mode

To achieve a better coding efficiency, it is proposed that all the previous coded rows are available as reference, and this mode is called “Copy from previous row”. The proposed mode is added in the available palette mode candidate list. The row index information is coded when the current string chooses “Copy from previous row” mode. The row index is coded using truncated binary codeword. Basically, a shorter codeword is designed for the row which is near to the current row. Like with other modes, the matching length is coded in the bitstream. An illustration of the proposed mode is shown in Fig. 1.

Fig. 1. Copy from previous row with Row 0 as the reference

To reduce the encoder complexity, the proposed mode is enabled only when the palette size is larger than 2 for the current CU, and “Copy from above” is invalid. To reduce the redundancy with “Copy from above”, the reference rows exclude the above row, and therefore, the proposed “Copy from previous row” mode is valid starting from the third row of the current CU.

# Simulation results

The proposed scheme is implemented on top of SCM1.0 based SCCE3 palette coding reference software and tested using the common test condition defined in [2]. Table 1 demonstrates the coding performance under full frame intra BC against SCCE3 palette reference software. Note that the encoding time is due to the structure of the encoder in SCCE3, but the new encoders proposed will help this method and additional results showing the performance and encoding time with other encoders will be provided.

Table 1. Results in Full frame Intra BC

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.0% | -1.0% |
| RGB, text & graphics with motion,720p | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.7% | -0.6% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.0% | -0.9% |
| YUV, text & graphics with motion,720p | -0.4% | -0.4% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.4% |
| YUV, mixed content, 1080p | -0.7% | -2.5% | -2.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.9% | -0.7% | -0.8% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -0.6% |
| YUV, mixed content, 1440p | -0.1% | -0.5% | -0.7% |
| YUV, mixed content, 1080p | -0.7% | -2.3% | -1.9% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, text & graphics with motion,720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.9% | -1.0% | -1.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.5% | -0.5% |
| YUV, text & graphics with motion,720p | -0.5% | -0.5% | -0.3% |
| YUV, mixed content, 1440p | -0.5% | -1.0% | -1.2% |
| YUV, mixed content, 1080p | -0.8% | -3.2% | -2.2% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

The proposed “Copy from previous row” mode is a simple method that fits well with the current “Copy from above” in the palette mode coding. The proposed method shows additional coding gains.

# References

[1] Y.-W. Huang, P. Onno, R. Joshi, R. Cohen, X. Xiu, Z. Ma, “HEVC Screen Content Core Experiment 3 (SCCE3): Palette mode”, JCT-VC Document, JCTVC-Q1123, Valencia, Spain, March 2014.

[2] H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-Q1015, March 2014

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).