| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0203 |
| --- | --- |

| Title: | Non-SCCE1: Default vector selection for intra block copy BV prediction |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  | Shawmin Lei |  |
| Author(s) or Contact(s): | Xiaozhong Xu Tzu-Der Chuang Shan Liu Shawmin Lei | Tel: Email: |  | +886-3-5670766 ext. 25555 xiaozhong.xu@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek |  |  |  |

_____________________________

# Abstract

This contribution proposes to use previously coded BV(s) in IntraBC mode as the default BV predictor, in spatial based IntraBC BV prediction schemes. In SCCE1 Test 3.4 (JCTVC-R0061), two candidate BV lists are constructed based on spatially adjacent and IntraBC coded blocks. When one or two of the lists is empty, the last one or two coded IntraBC BV will be used. It is reported that together with Test 3.4, the proposed approach with last 2 coded BVs used, provides objective bitrate reductions of -3.9 %, -4.6 % for All Intra 1080p text and graphics RGB and YUV categories, respectively; and with last one coded BV, provides objective bitrate reductions of -4.0 %, -4.8 % for All Intra 1080p text and graphics RGB and YUV categories, respectively. With encoder only optimization, it is reported that with last 2 coded BVs, the objective bitrate reductions are -7.7 %, -8.3 % for All Intra 1080p text and graphics RGB and YUV categories, respectively; with last one BV, the objective bitrate reductions are -7.5 %, -8.2 % for All Intra 1080p text and graphics RGB and YUV categories, respectively.

# Introduction

The proposed technique uses SCCE1 Test 3.4 [1] scheme for IntraBC BV prediction. In Test 3.4, when one or two of the candidates are not available, some preset constant BV values will be used to fill the candidate lists. In this proposal, an N-unit storage is used to store the last N coded IntraBC BVs. When some or all entries of the candidate lists are empty, the stored BV(s) will be used. For initialization at each CTU, (-2*CU_width, 0), (-2*CU_width, 0) and (-CU_width, 0) will be used. If N = 1, the first default vector will be the last coded BV, and the second default vector will be the preset constant ((-2*CU_width, 0) in this case).

# Results

The proposed method is integrated on HM-14.0+RExt-7.0+SCM-1.0 [2].	The simulations are performed under the updated CTC [3]. Two set of results are presented. In simulation A, the original release of this SCCE test is included. In simulation B, some encoder only changes/bug fixes are also integrated (referred as config. 2 in SCCE1 email reflector). 1 second short simulation results are provided and will be updated with full length results shortly.

Table 1: Results for simulation A lossy configuration, 2 last BVs used

Table 2: Results for simulation A lossless configuration, 2 last BVs used

Table 3: Results for simulation A lossy configuration, 1 last BV used

Table 4: Results for simulation A lossless configuration, 1 last BV used

Table 5: Results for simulation B lossy configuration, 2 last BVs used

Table 6: Results for simulation B lossless configuration, 2last BVs used

Table 7: Results for simulation B lossy configuration, 1 last BV used

Table 8: Results for simulation B lossless configuration, 1 last BV used

# Reference

[1] J. Sole, S. Liu, “HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions”, JCTVC-Q1121, Apr. 2014

[2] HM-14.0+RExt-7.0+SCM-1.0, https://hevc.hhi.fraunhofer.de/trac/hevc/browser/tags/HM-14.0%2BRExt-7.0%2BSCM-1.0

[3] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”, JCTVC-Q1015, Apr. 2014

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).