| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0207 |
| --- | --- |

| Title: | Crosscheck of SCCE3: Test A.8 - Improvements on palette prediction vector signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9710 Scranton Road, Suite 250 San Diego, CA 92121 | Tel: Email: | +1-858-210-4830 Xiaoyu.xiu@interdigital.com |
| Source: | InterDigital Communications, LLC |  |  |

_____________________________

# Abstract

This document presents the cross checking results of JCTVC-R0063 on improvements on palette prediction vector signaling for palette mode proposed by Canon. The verification task was done. The BD-rate results match those provided by proponent.

# Introduction

JCTVC-R0063 [1] proposes a two level hierarchical structure for palette prediction vector signaling. First, the binary palette prediction vector is separated into groups. Each group contains 4 bins except for the last group, which may have less than 4 bins.  The source code of this method is provided by proponent. This verification is carried out by using the provided source code. The BD-rate results match those provided by proponent.

# Results

The detailed results are presented and bundled in the same archive.

## Full IBC

Table 1. BD-rate comparison to SCCE3 anchor of full intra block copy search for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.3% | -3.3% | -3.2% |
| RGB, text & graphics with motion,720p | -3.1% | -3.0% | -3.0% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.4% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.4% | -3.6% | -3.5% |
| YUV, text & graphics with motion,720p | -2.3% | -2.8% | -3.3% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.5% | -0.5% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.0% | -2.0% | -1.9% |
| RGB, text & graphics with motion,720p | -2.8% | -2.6% | -2.6% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.4% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.0% | -2.2% | -2.2% |
| YUV, text & graphics with motion,720p | -2.0% | -2.4% | -2.9% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.4% | -0.6% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.4% | -1.3% |
| RGB, text & graphics with motion,720p | -1.7% | -1.6% | -1.6% |
| RGB, mixed content, 1440p | -0.4% | -0.4% | -0.4% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.1% | -1.4% | -1.3% |
| YUV, text & graphics with motion,720p | -0.8% | -1.0% | -1.6% |
| YUV, mixed content, 1440p | -0.3% | -0.1% | -0.3% |
| YUV, mixed content, 1080p | -0.4% | -1.0% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Bit-rate comparison to SCCE3 anchor of full intra block copy search for lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.3% | 3.4% | 1.3% | 6.5% |
| RGB, text & graphics with motion,720p | 2.4% | 2.5% | 0.1% | 5.1% |
| RGB, mixed content, 1440p | 0.3% | 0.3% | 0.3% | 0.4% |
| RGB, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.5% | 4.7% | 1.4% | 7.8% |
| YUV, text & graphics with motion,720p | 2.8% | 3.0% | 0.1% | 5.9% |
| YUV, mixed content, 1440p | 0.2% | 0.2% | 0.1% | 0.3% |
| YUV, mixed content, 1080p | 0.2% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.1% | 2.1% | 1.0% | 4.4% |
| RGB, text & graphics with motion,720p | 0.3% | 1.3% | 0.0% | 3.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.4% | 3.0% | 1.0% | 5.2% |
| YUV, text & graphics with motion,720p | 0.4% | 1.8% | 0.0% | 4.7% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.0% | 1.6% | 0.8% | 3.0% |
| RGB, text & graphics with motion,720p | 0.2% | 0.9% | 0.0% | 2.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.2% | 2.3% | 0.9% | 3.5% |
| YUV, text & graphics with motion,720p | 0.2% | 1.2% | 0.0% | 2.8% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 103% |  |  |  |

## 2-CTU IBC

Table 1. BD-rate comparison to SCCE3 anchor of 2-CTU intra block copy search for lossy coding

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -4.0% | -4.0% | -3.9% |
| RGB, text & graphics with motion,720p | -3.9% | -3.8% | -3.8% |
| RGB, mixed content, 1440p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1080p | -0.5% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -3.9% | -4.1% | -4.1% |
| YUV, text & graphics with motion,720p | -3.1% | -3.5% | -3.8% |
| YUV, mixed content, 1440p | -0.3% | -0.5% | -0.5% |
| YUV, mixed content, 1080p | -0.6% | -0.9% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.6% | -2.5% | -2.5% |
| RGB, text & graphics with motion,720p | -3.4% | -3.2% | -3.3% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -2.4% | -2.7% | -2.6% |
| YUV, text & graphics with motion,720p | -2.7% | -2.9% | -3.3% |
| YUV, mixed content, 1440p | -0.3% | -0.6% | -0.5% |
| YUV, mixed content, 1080p | -0.4% | -0.7% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -1.7% | -1.7% |
| RGB, text & graphics with motion,720p | -2.0% | -1.7% | -1.8% |
| RGB, mixed content, 1440p | -0.3% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.7% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -1.4% | -1.6% | -1.6% |
| YUV, text & graphics with motion,720p | -1.1% | -1.4% | -1.4% |
| YUV, mixed content, 1440p | -0.4% | -0.6% | -0.7% |
| YUV, mixed content, 1080p | -0.3% | -0.8% | -1.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 102% |  |  |

Table 2. Bit-rate comparison to SCCE3 anchor of 2-CTU intra block copy search for lossless coding

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 3.8% | 3.7% | 1.5% | 7.3% |
| RGB, text & graphics with motion,720p | 2.5% | 2.9% | 0.1% | 5.6% |
| RGB, mixed content, 1440p | 0.9% | 0.8% | 0.4% | 1.1% |
| RGB, mixed content, 1080p | 0.4% | 0.4% | 0.4% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 4.6% | 4.7% | 1.7% | 7.6% |
| YUV, text & graphics with motion,720p | 2.9% | 3.3% | 0.1% | 6.5% |
| YUV, mixed content, 1440p | 0.4% | 0.4% | 0.4% | 0.5% |
| YUV, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.3% | 2.6% | 1.1% | 5.4% |
| RGB, text & graphics with motion,720p | 0.4% | 1.7% | 0.1% | 4.7% |
| RGB, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.6% | 3.4% | 1.2% | 5.6% |
| YUV, text & graphics with motion,720p | 0.4% | 2.0% | 0.0% | 5.5% |
| YUV, mixed content, 1440p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |  |
| Dec Time[%] | 96% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 1.2% | 2.0% | 1.0% | 4.0% |
| RGB, text & graphics with motion,720p | 0.2% | 1.0% | 0.0% | 2.5% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.4% | 2.8% | 1.1% | 4.2% |
| YUV, text & graphics with motion,720p | 0.3% | 1.3% | 0.0% | 3.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

# Reference

[1] M. Karczewicz, W. Pu, V. Seregin, R. Joshi, J. Sole, “SCCE3: Test A.8 - Improvements on Palette Prediction Vector Signaling,” Document of   Joint Collaborative Team on Video Coding, JCTVC-R0063, June 2014.