| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014 | Document: JCTVC-R0208 M33849 |
| --- | --- |

| Title: | Non-SCCE1: Additional results on intra block copy (IBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA 92127 USA | Email: | minhua@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution reports additional test results on intra block copy (IBC). In the tests, the full frame IBC is used for I-frame coding, while the local IBC is used in non-I frames. The four local search range settings (i.e. 2CTUs, 3CTUs, 4CTUs and 4CTUs + 4 upper lines case) are tested.

# Introduction

This document reports additional tests results on intra block copy (IBC). In the tests, the full frame IBC is used for I-frame coding while the local intra block copy (IBC) is used in non-I frame coding. The IBC search range for local IBC is set as follows:

- The current CTU and one neighboring CTU to the left (i.e. 2CTUs case). FII+2C

- The current CTU and two neighboring CTUs to the left (i.e. 3CTUs case). FII+3C

- The current CTU and three neighboring CTUs to the left (i.e. 4 CTUs case). FII+4C

- The current CTU, three neighboring CTUs to the left and four upper sample rows (i.e. 4CTUs + 4 upper lines case). FII+4C+4

For the sake of comparison, the SCCE1 test1.2 results [3] are also listed in this document. In SCCE1 test1.2 local IBC is used for all the frames. They are marked as “2C”, “3C”, “4C” and “4C+4” for local IBC search range of 2, 3, 4 CTUs and 4 CTUs plus 4 upper lines.

# Test Settings and Conditions

The simulations of this document have used the software modified from the JCTVC-R0141 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [2] are followed.

# Experimental results

The experimental results are summarized in Table 1 to Table 2. Anchor used here is SCM1 which uses full frame IBC in all frames. Please be advised that runtime reported here may not be accurate due to heterogeneous nature of the LSF farm.

Table 1. Summary results on IBC lossy coding

Table 2. Summary results on IBC lossless coding

# Comments

Full frame IBC in I-frames and local IBC in non-I frames could be a good design trade-off in terms of IBC coding efficiency, memory bandwidth and memory size.  It has been observed that enlarging the local IBC search range can generally further improve IBC coding efficiency; adding full frame IBC in I-frame on the top of local IBC is beneficial to lossy coding, but not to lossless coding. In lossless coding the implicit RDPCM apparently interacts with IBC and leaves very limited room for full-frame IBC to further improve coding efficiency.

# References

[1]	J. Sole, S. Liu, “HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions”. JCT-VC Document, JCTVC-Q1121, Valencia, ES, April 2014.

[2]  H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”,  JCT-VC Document, JCTVC-Q1015, Valencia, ES, April 2014.

[3] M. Zhou, “SCCE1: Cross-verification of test 1.2  results”, JCT-VC Document, JCTVC-R0211, Sapporo, JP, 30 June – 9 July, 2014

# Patent rights declaration(s)

Broadcom Corporation does not have current or pending patent rights relating to the technology described in this contribution.