| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0209 |
| --- | --- |

| Title: | AHG13: Re-sampling process with generic phase |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | E. Alshina | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This document explains detail of re-sampling process with generic phase (not phase and delta as in current draft). Normative changes related to this contribution are part of recommendation of AhG13 where “system of 3 flags“ for re-sampling process phases derivation is replaced by explicit signaling for phases. This document describes proper settings of re-sampling phases for different use cases and estimates potential effect on accuracy of re-sampling process.

# Introduction

This contribution is complete version of [2] where responding to the AhG13 mandate about the study of generic solutions for phase adjustment in re-sampling process merging of phase and delta currently used in re-sampling process was demonstrated.

After Valencia meeting this approach was extensively studied and discussed during AhG13 teleconferences. The agreed during AhG13 teleconferences version of the re-sampling process modification is described in recommended specification text attached to AhG13 report [3].

This document explains the part of suggested by AhG14 spec changes related to phase adjustment.

# Suggested re-sampling process modifications

Re-sampling process in current SHVS specification draft [1] uses 2 parameters controls fractional position: phase (reflects fractional shift in EL resolution) and delta (reflects fractional shift in BL resolution). These parameters are derived from system of 3 flags (CrossLayerPhaseAlignmentFlag , VertPhasePositionAdjustFlag and VertPhasePositionFlag). But as one can see from (H-13)-(H-16) only combination of delta and phase participates in re-sampling process; these parameters are not used separately.

Current specification text (includes re-placement of base layer conformance window parameters by reference layer off-sets).

The variables phaseX, phaseY, deltaX and deltaY are derived as follows:

–	If cIdx is equal to 0, the following applies:

phaseX = CrossLayerPhaseAlignmentFlag << 1		(H-XX)
phaseY = CrossLayerPhaseAlignmentFlag << 1		(H-XX)

deltaX = ( RefLayerLeftOffset ) << 4) −
		( CrossLayerPhaseAlignmentFlag << 3 )		(H-XX)
deltaY = ( RefLayerTopOffset ) << 4) − ( ( ( CrossLayerPhaseAlignmentFlag
		<< (3 – VertPhasePositionAdjustFlag ) ) + ( VertPhasePositionFlag << 3 ) )	(H-XX)

–	Otherwise (cIdx is equal to 1), the following applies:

phaseX = CrossLayerPhaseAlignmentFlag		(H-XX)
phaseY = CrossLayerPhaseAlignmentFlag + 1		(H-XX)

deltaX = ( ( RefLayerLeftOffset / RefLayerSubWidthC ) << 4) − ( CrossLayerPhaseAlignmentFlag << 2 )	(H-XX)
deltaY = ( ( RefLayerTopOffset / RefLayerSubHeightC ) << 4) − ( ( ( CrossLayerPhaseAlignmentFlag + 1 )
		 << ( 2 − VertPhasePositionAdjustFlag ) ) + (VertPhasePositionFlag << 3 ) )	(H-XX)

The variables addX and addY are derived as follows:

addX = ( ScaleFactorX * phaseX + 2 ) >> 2		(H-13) 
addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(H-14)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 ) +  deltaX	(H-15)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) +  deltaY	(H-16)

end.

In this form delta includes reference layer offsets (which  value could be up to BL picture size) and combination of 3 flags (dX and dY) which namely describes  fractional phase shift:

deltaX = RefOffsetX –dX,

deltaY = RefOffsetY−dY;

here

RefOffsetX = (RefLayerLeftOffset / ( ( cIdx = = 0 )  ?  1 :  RefLayerSubWidthC ))<<4,	
RefOffsetY = (RefLayerTopOffset / ( ( cIdx = = 0 )  ?  1 :  RefLayerSubHeightC ))<<4.

In JCTVC-Q0203 it was demonstrated how formulas (H-13)-(H-16) can be equivalently re-written as follows:

addX = ( ScaleFactorX * phaseX –(dX<<14) + 2 ) >> 2	(H-13’) 
addY = ( ScaleFactorY * phaseY −(dY<<14) + 2 ) >> 2	(H-14’)

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 )   + RefOffsetX,    (H-15’)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 )  +  RefOffsetY.    (H-16’)

And further phaseX/Y and dX/Y can be combined in (H-13’)  and (H-14’) into the one parameter:

addX = ( ScaleFactorX * (4*phaseX –dX * ScaledRefLayerPicWidthInSamplesY/ RefLayerRefRegionWidthInSamplesY + 8 ) >> 4	                                                                               (H-13’’) 
addY = ( ScaleFactorY * (4*phaseY −dY * ScaledRefLayerPicHeightInSamplesY / RefLayerRefRegionHeightInSamplesY + 8 ) >> 4	                                                                            (H-14’’)

Here scale factors are :

ScaleFactorX = ( ( RefLayerRefRegionWidthInSamplesY << 16 ) +
		( ScaledRefLayerPicWidthInSamplesY >> 1 ) ) / ScaledRefLayerPicWidthInSamplesY	(H-1)
ScaleFactorY = ( ( RefLayerRefRegionHeightInSamplesY << 16 ) +
		( ScaledRefLayerPicHeightInSamplesY >> 1 ) ) / ScaledRefLayerPicHeightInSamplesY	(H-2)

Suggestion in [3] was to signal combination of phase and d highlighted in grey in (H-13’’), (H-14’’)  and so exclude phase and delta derivation from decoding process and re-write horizontal, vertical, luma an dchroma interpolation in generic form.

Suggested text for re-sampling process.

The variables phaseX and phaseY are determined by syntax elements  phase_hor_luma [rLId ], phase_ver_luma [rLId ], phase_hor_chroma [rLId ], phase_ver_chroma [rLId ]:

–	If cIdx is equal to 0, the following applies:

phaseX = phase_hor_luma [rLId ],		(H-7)
phaseY = phase_ver_luma [rLId ],		(H-8)

–	Otherwise (cIdx is equal to 1), the following applies:

phaseX = phase_hor_chroma [rLId ],		(H-3)
phaseY = phase_ver_chroma [rLId ],		(H-4)

The variables addX and addY are derived as follows:

addX = ( ScaleFactorX * phaseX + 8  ) >> 4		(H-5) 
addY = ( ScaleFactorY * phaseY + 8  ) >> 4		(H-6)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP − offsetX ) * ScaleFactorX − addX + ( 1 << 11 ) ) >> 12 ) + RefOffsetX	(H-7)
yRef16 = ( ( ( yP − offsetY ) * ScaleFactorY − addY + ( 1 << 11 ) ) >> 12 ) + RefOffsetY	(H-8)

# Recommendation about re-sampling phases settings

Re-sampling process for Luma is formally described by following equation:

xRef= (xP-α)*S + β.

Here:

xRef – reference layer coordinate,

xP – current layer coordinate ,

S – “inverse scaling” factor ,

α – starting position relatively to top-left corner in current layer (“EL cropping parameter”),

β – starting position relatively to top-left corner in reference layer (“BL cropping parameter”)

Figures 1 and 2 illustrate the meaning of α and β. Both α and β may have fractional part (for example in case of down-sampler with center alignment). Positive values α and β correspond to cropping of coded picture, negative to padding.

Re-sampling process for Chroma is formally described by following equation:

xRef= (xP-(α/SC-Elcp/SC))*(SC/RLSC)*S + β/RLSC- BLcp/RLSC.

Following parameters which specify color format in current and reference layer:

SC = SubWidthC or SubHeightC;

RLSC = RefLayerSubWidthC or  RefLayerSubHeightC.

SC, RLSC =2 for 4:2:0.

The location of Chroma samples relatively to Luma is specified by

ELcp = EL Ch. Phase,

BLcp = BL Ch. Phase.

0≤Elcp, BLcp ≤1 (Fig.3)

Fig. 1. Explanation about α parameter meaning Fig. 2. Explanation about β parameter meaning

Luma re-sampling parameters settings.

In (H-15) and (H-16) for Luma re-sampling

- offsetX(Y) integer number, multiple of SC (derived for syntax element as follows: scaled_ref_layer_offset[ rLId ]*SC)

- RefOffsetX(Y) is integer number, multiple of RLSC (derived for syntax element as follows: (ref_layer_offset[ rLId ]*RLSC)<<4)

Let’s re-write re-sampling process for Luma separating offset (integer multiple of SC), RefOffset (integer multiple of RLSC) and put information remaining part (not included into offset or RefOffset) of α and β into PhaseL.

Any number d can be reprecented as combination of “integer” and “fractional” parts:

d = [d] m +{d}m

Here

[d] m=m*int(d/m) – integer multiple of m

{d}m =d-[d] m– fractional number in a range of  0≤{d}m<m

So,

xRef= (xP-α)*S + β

= (xP-[α] SC – {α} SC)*S + [β] 16*RLSC+{β}16*RLSC

= (xP – [α] SC– ({α} SC – {β}16*RLSC /S) )*S + [β] 16*RLSC

= (xP – [α] SC–[ ({α} SC – {β}16*RLSC /S)]SC –{({α} SC – {β}16*RLSC /S)}SC)*S + [β] 16*RLSC.

[α] SC+[ ({α} SC – {β}16*RLSC /S)]SC  becomes offSet in (H-15) and (H-16) for Luma

[β] 16*RLSC  becomes RefOffSet in (H-15) and (H-16) for Luma

{({α} SC – {β}16*RLSC /S)}SC becomes re-sampling phase.

0≤{({α} SC – {β}16*RLSC /S)}SC<SC.

Integer reperentation of re-sampling phase with accuracy 1/16 is

PhaseL = int(16*{({α} SC – {β}16*RLSC /S)}SC).  (1)

This value is the range of

0≤ PhaseL <16*SC.

For 4:2:0 color format

0≤ PhaseL≤31.

The deafult value for PhaseL is 0 (both vertical and horizontal interpolation).

Chroma re-sampling parameters settings.

In (H-15) and (H-16) for Chroma re-sampling

- offsetX(Y) integer number (scaled_ref_layer_offset[ rLId ])

- RefOffsetX(Y) is integer number (ref_layer_offset[ rLId ])

Let’s re-write re-sampling process for Chroma separating offset, RefOffset and put information remaining part (not included into offset or RefOffset) of α and β into PhaseC.

xRef= (xP-(α/SC-Elcp/SC))*SC*S/RLSC+ β/RLSC- BLcp/RLSC

= (xP-([α]SC+{α}SC)/SC+Elcp/SC)*SC*S/RLSC+ ([β] 16* RLSC+{β} 16* RLSC)/RLSC- BLcp/RLSC

= (xP-[α]SC/SC -{α}SC/SC  +{β} 16* RLSC/SC/S +Elcp/SC- BLcp/SC/S)*SC*S/RLSC+ [β] 16* RLSC/RLSC

= (xP-[α]SC/SC –({α}SC-{β} 16* RLSC/S)/SC+Elcp/SC- BLcp/SC/S)*SC*S/RLSC+ [β] 16* RLSC/RLSC

= (xP-([α]SC+[{α}SC-{β} 16* RLSC/S]SC)/SC -{{α}SC-{β} 16* RLSC/S }SC/SC+Elcp/SC- BLcp/SC/S)*SC*S/RLSC+ [β] 16* RLSC/RLSC

([α] SC+[ ({α} SC – {β} 16* RLSC /S)]SC )/SC becomes OffSet in (H-15) and (H-16) for Chroma

([β] 16* RLSC)/RLSC  becomes RefOffSet in (H-15) and (H-16) for Chroma

{{α}SC-{β} 16* RLSC/S }SC /SC-(Elcp/SC- BLcp/SC/S)  becomes re-sampling phase for Chroma

0≤ {{α}SC-{β} 16* RLSC/S }SC <SC  0≤{{α}SC-{β} 16* RLSC/S }SC /SC<1

-1/SC/S≤(ELcp/SC-BLcp/SC/S) ≤ 1/SC

-1/SC≤ {{α}SC-{β} 16* RLSC/S }SC /SC-(Elcp/SC- BLcp/SC/S) <1+ 1/SC/S

Integer reperentation of re-sampling phase with accuracy 1/16 is

PhaseC = [16*({{α}SC-{β} 16* RLSC/S }SC /SC-(Elcp/SC- BLcp/SC/S) )]   (2)

This value is the range of

For 4:2:0 both current and reference layers SC=RLSC=2 and PhaseC value is the range of

-8≤ PhaseC <16+ 8/S.

Default PhaseC in horizontal interpolation is 0.

Default PhaseC in vertical interpolation (α=β=0, Elcp=Blcp=0,5):

PhaseC = int(4/S-4).                  (3)

Fig. 3 Relative Chroma and Luma sample location.

As shown above PhaseL is non-negative value:

0≤ PhaseL < 32.

It means that PhaseL can be encoded using ue(v) or u(5) format.

For Chroma the range of re-sampling phase is

–8≤PhaseC< 16+8/S.

The value “PhaseC+8” is non-negative:

0≤PhaseC+8<24+8/S.

The magnitude is not-limited, since any scaling factor S is allowed.

The value “PhaseC+8” can be encoded using ue(v).

Restriction for the range of “PhaseC+8” to 6 bits:

0≤PhaseC+8< 64

allows the usage of u(6) format for “PhaseC+8” and precisely accurate enough for S≥1/5 (scaling ratio ≤ 5).

Usage of ue(v) for PhaseC+8 is accurate for any scaling ratio.

Table 1 summarizes syntax elements related to re-sampling phases signaling, ranges, default values and possible formats for re-sampling phase signaling.

Table 1. Syntax elements related to re-sampling phases signaling.

| Syntax elements | Theoretical range | Default value | Variants for signaling |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | A | B | C |
| phase_hor_luma | 0…31 | 0 | ue(v) | u(5) | u(5) |
| phase_ver_luma | 0…31 | 0 | ue(v) | u(5) | u(5) |
| phase_hor_chroma_plus8 | 0…24+8/S | 8 | ue(v) | ue(v) | u(6) |
| phase_ver_chroma_plus8 | 0…24+8/S | int(4/S+4) | ue(v) | ue(v) | u(6) |

Table 2 gives re-sampling phases for typical use cases studied during AhG13 discussion.

Excel template “JCTVC-R0209_AccuracyEvaluation_r2.xlsx” attached to this contribution allows re-sampling process accuracy estimation for suggested re-sampling process design compare to the current specification text.

Excel template “JCTVC-R00209_UseCasesList_r2.xlsx” attached to this contribution summarizes re-sampling parameters settings for various use-cases and estimates bits needed for these parameters sending using se(v) format for offsets and ue(v) or u(5)&u(6) for re-sampling phases.

# Recommendations

Use equations (1) and (2) for explanation re-sampling phases settings in SHVC Test model.

Use u(5) for phase_hor_luma and phase_ver_luma syntax elements for signaling Luma re-sampling phases.

Use 0 as default values for phase_hor_luma, phase_ver_luma (if not presents in bit-stream).

Use u(6) for phase_hor_chroma_plus8 and phase_ver_chroma_plus8 syntax elements for signaling Chroma re-sampling phases.

Use 8 and int(4/S+4) as default values for phase_hor_chroma_plus8 and phase_ver_chroma_plus8 correspondently.

Table 2. The recommendation about re-sampling phases settings for various use cases.

|  | 824-928 |  | 480-720 |  | SVGA-720p |  | 540-720 |  | 720-1080 |  | 480i-720p |  |  |  | 1080i-4kp |  |  |  | 1080i-1080p |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CA H-RXT | EA H-RXT | EA-AHEC | EA-AVBP | CA-AHEC | EA-AVBP | EA H | CA V | EA H | CA V | TF-CA | BF-CA | TF-EA | BF-EA | TF-CA | BF-CA | TF-EA | BF-EA | TF-CA | BF-CA | TF-EA | BF-EA |
| InvScal. Num | 3 | 3 | 2 | 9 | 5 | 5 | 3 | 3 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| InvScal.DeNum | 4 | 4 | 3 | 16 | 6 | 8 | 4 | 4 | 3 | 3 | 3 | 3 | 3 | 3 | 4 | 4 | 4 | 4 | 2 | 2 | 2 | 2 |
| α | -256,5 | -256 | 100 | 0 | 109,5 | 0 | 0 | -0,5 | 0 | -0,5 | -0,5 | -0,5 | 0 | 0 | -0,5 | -0,5 | 0 | 0 | -0,5 | -0,5 | 0 | 0 |
| β | -128,5 | -128 | 0 | 38 | -0,5 | 76 | 0 | -0,5 | 0 | -0,5 | -0,25 | -0,75 | 0 | -0,5 | -0,25 | -0,75 | 0 | -0,5 | -0,25 | -0,75 | 0 | -0,5 |
| EL Size | 928 | 928 | 1280 | 720 | 1280 | 720 | 1280 | 720 | 1920 | 1080 | 720 | 720 | 720 | 720 | 2160 | 2160 | 2160 | 2160 | 1080 | 1080 | 1080 | 1080 |
| BL Size | 824 | 824 | 720 | 480 | 800 | 600 | 960 | 540 | 1280 | 720 | 240 | 240 | 240 | 240 | 540 | 540 | 540 | 540 | 540 | 540 | 540 | 540 |
| EL Ch. phase | 0 | 0 | 0 | 0,5 | 0 | 0,5 | 0 | 0,5 | 0 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 |
| BL Ch. phase | 0 | 0 | 0 | 0,5 | 0 | 0,5 | 0 | 0,5 | 0 | 0,5 | 0,25 | 0,75 | 0,25 | 0,75 | 0,25 | 0,75 | 0,25 | 0,75 | 0,25 | 0,75 | 0,25 | 0,75 |
| PhaseXL | 3 | 0 | 0 | NA | 2 | NA | 0 | NA | 0 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| PhaseYL | NA | NA | NA | 0 | NA | 0 | NA | 3 | NA | 4 | 4 | 28 | 0 | 24 | 8 | 8 | 0 | 0 | 0 | 16 | 0 | 16 |
| PhaseXC | 1 | 0 | 0 | NA | 1 | NA | 0 | NA | 0 | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| PhaseYC | NA | NA | NA | 3 | NA | 2 | NA | 3 | NA | 4 | 4 | 28 | 2 | 26 | 8 | 24 | 4 | 20 | 0 | 16 | 0 | 16 |

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Chen, J. Boyce, Y. Ye, and M. M. Hunnuksela, Y. K. Wang, SHVC Draft 6, JCTVC document no JCTVC-Q1008, April 2014.

- E.Alshina, AhG13: Methodology for evaluation an accuracy of re-sampling process, JCTVC document no JCTVC-Q0203, April 2014.

- E. Alshina, J. Chen,  P. Topiwala, T. Yamamoto, Y. Ye, AhG13: JCT-VC AHG report: SHVC inter-layer re-sampling process (AHG13), JCTVC document no JCTVC-R0013, June 2014