| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July, 2014 | Document: JCTVC-R0211 M33852 |
| --- | --- |

| Title: | SCCE1: Cross-verification of test1.2 results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo Drive San Diego, CA 92127 USA | Email: | minhua@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for SCCE1 test1.2 on intra block copy provided in JCTVC-R0141.  Three local search range settings (i.e. 3CTUs, 4CTUs and 4CTUs + 4 upper lines case) were tested. With a reasonable amount of cache memory, the proposed designs can improve the IBC coding efficiency without incurring memory bandwidth increase. It is asserted and the results match the ones reported by proponents, and the source code is also checked to confirm its consistency with the proposal description.

# Introduction

This document reports cross-check results for the SCCE1 test1.2 [1, 3] on the local intra block copy (IBC) with the following three search range settings:

- The current CTU and two neighboring CTUs to the left (i.e. 3CTUs case).

- The current CTU and three neighboring CTUs to the left (i.e. 4 CTUs case).

- The current CTU, three neighboring CTUs to the left and four upper sample rows (i.e. 4CTUs + 4 upper lines case).

Just like the IBC design with 2CTUs (current CTU plus the left neighboring CTU) search range, all the block sizes are searched in the above three settings.

To avoid memory bandwidth increase and need of buffering pre-filtered reconstructed pictures off-chip, a decoder would need to prepare a sufficient amount of cache memory for the enlarged IBC local search area.   For 4:4:4 chroma format and 8-bit bit-depth, for example, the above three IBC designs would need additional cache memory size (in bytes) of 24576, 36864 and 82944 (assume 4Kx2K picture size and no memory sharing with the de-blocking filter), respectively.

# Test Settings and Conditions

The simulations of this document have used software released by Huawei, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [2] are followed.

# Experimental results

The experimental results are summarized in Table 1 to Table 3 for lossy coding, and Table 4 to Table 6 for lossless coding, respectively. The anchor is SCM1 in which full frame IBC is used. It is verified that those results match what proponents have reported. Please be advised that runtime reported here may not be accurate due to heterogeneous nature of the LSF farm.

Table 1. Experimental results on lossy coding with IBC search range setting of 3CTUs

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 25.5% | 25.2% | 25.2% |
| RGB, text & graphics with motion,720p | 15.8% | 15.3% | 15.4% |
| RGB, mixed content, 1440p | 11.7% | 11.6% | 11.7% |
| RGB, mixed content, 1080p | 17.0% | 17.1% | 16.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 25.9% | 25.4% | 25.3% |
| YUV, text & graphics with motion,720p | 15.7% | 15.2% | 15.1% |
| YUV, mixed content, 1440p | 12.9% | 12.8% | 12.8% |
| YUV, mixed content, 1080p | 18.4% | 18.5% | 18.2% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 12.9% | 12.6% | 12.6% |
| RGB, text & graphics with motion,720p | 10.8% | 10.5% | 10.4% |
| RGB, mixed content, 1440p | 6.6% | 6.4% | 6.5% |
| RGB, mixed content, 1080p | 10.1% | 10.2% | 10.1% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 12.5% | 12.2% | 12.1% |
| YUV, text & graphics with motion,720p | 10.6% | 10.5% | 10.4% |
| YUV, mixed content, 1440p | 8.0% | 7.9% | 7.9% |
| YUV, mixed content, 1080p | 12.3% | 11.1% | 11.5% |
| YUV, Animation, 720p | 0.1% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 8.2% | 7.7% | 8.0% |
| RGB, text & graphics with motion,720p | 4.2% | 4.1% | 4.0% |
| RGB, mixed content, 1440p | 3.8% | 3.3% | 3.5% |
| RGB, mixed content, 1080p | 3.5% | 3.6% | 3.1% |
| RGB, Animation, 720p | -0.1% | -0.1% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 7.5% | 7.3% | 7.2% |
| YUV, text & graphics with motion,720p | 3.6% | 3.6% | 3.7% |
| YUV, mixed content, 1440p | 4.3% | 3.9% | 4.1% |
| YUV, mixed content, 1080p | 5.0% | 1.6% | 3.1% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.3% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 105% |  |  |

Table 2. Experimental results on lossy coding with IBC search range setting of 4CTUs

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 21.3% | 21.0% | 21.0% |
| RGB, text & graphics with motion,720p | 12.9% | 12.4% | 12.5% |
| RGB, mixed content, 1440p | 10.2% | 10.1% | 10.3% |
| RGB, mixed content, 1080p | 15.9% | 15.9% | 15.5% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 21.7% | 21.2% | 21.0% |
| YUV, text & graphics with motion,720p | 12.9% | 12.3% | 12.2% |
| YUV, mixed content, 1440p | 11.3% | 11.2% | 11.2% |
| YUV, mixed content, 1080p | 17.3% | 17.3% | 17.1% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 134% |  |  |
| Dec Time[%] | 112% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 10.7% | 10.4% | 10.4% |
| RGB, text & graphics with motion,720p | 8.7% | 8.4% | 8.4% |
| RGB, mixed content, 1440p | 5.9% | 5.8% | 5.9% |
| RGB, mixed content, 1080p | 9.5% | 9.6% | 9.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.4% | 10.1% | 10.1% |
| YUV, text & graphics with motion,720p | 8.6% | 8.4% | 8.3% |
| YUV, mixed content, 1440p | 7.3% | 7.1% | 7.1% |
| YUV, mixed content, 1080p | 11.6% | 10.3% | 10.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 6.9% | 6.5% | 6.7% |
| RGB, text & graphics with motion,720p | 3.6% | 3.3% | 3.5% |
| RGB, mixed content, 1440p | 3.6% | 3.1% | 3.4% |
| RGB, mixed content, 1080p | 3.3% | 3.3% | 2.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 6.3% | 6.1% | 6.0% |
| YUV, text & graphics with motion,720p | 3.2% | 2.9% | 2.6% |
| YUV, mixed content, 1440p | 4.2% | 3.4% | 3.5% |
| YUV, mixed content, 1080p | 4.8% | 1.8% | 2.8% |
| YUV, Animation, 720p | 0.0% | -0.3% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3. Experimental results on lossy coding with IBC search range setting of 4CTUs + 4 upper lines

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 19.2% | 18.9% | 18.9% |
| RGB, text & graphics with motion,720p | 12.0% | 11.5% | 11.6% |
| RGB, mixed content, 1440p | 9.4% | 9.2% | 9.3% |
| RGB, mixed content, 1080p | 14.5% | 14.5% | 14.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 19.6% | 19.1% | 19.0% |
| YUV, text & graphics with motion,720p | 12.0% | 11.4% | 11.4% |
| YUV, mixed content, 1440p | 10.3% | 10.2% | 10.2% |
| YUV, mixed content, 1080p | 15.9% | 16.0% | 15.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 123% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 9.8% | 9.5% | 9.5% |
| RGB, text & graphics with motion,720p | 8.1% | 7.9% | 7.8% |
| RGB, mixed content, 1440p | 5.4% | 5.4% | 5.4% |
| RGB, mixed content, 1080p | 8.8% | 8.8% | 8.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 9.6% | 9.3% | 9.2% |
| YUV, text & graphics with motion,720p | 8.1% | 7.9% | 7.9% |
| YUV, mixed content, 1440p | 6.7% | 6.4% | 6.4% |
| YUV, mixed content, 1080p | 10.9% | 9.6% | 10.0% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 6.3% | 5.9% | 6.1% |
| RGB, text & graphics with motion,720p | 3.4% | 3.1% | 3.2% |
| RGB, mixed content, 1440p | 3.2% | 2.9% | 3.1% |
| RGB, mixed content, 1080p | 3.2% | 3.2% | 2.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 5.8% | 5.6% | 5.6% |
| YUV, text & graphics with motion,720p | 3.0% | 2.7% | 3.1% |
| YUV, mixed content, 1440p | 3.9% | 3.4% | 3.8% |
| YUV, mixed content, 1080p | 4.3% | 0.5% | 2.1% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 95% |  |  |

Table 4. Experimental results on lossless coding with IBC search range setting of 3CTUs

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -24.2% | -23.1% | -40.1% | -7.9% |
| RGB, text & graphics with motion,720p | -4.2% | -7.8% | -23.5% | -0.5% |
| RGB, mixed content, 1440p | -11.7% | -10.7% | -15.7% | -5.7% |
| RGB, mixed content, 1080p | -8.8% | -8.8% | -8.8% | -8.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -25.7% | -24.8% | -44.2% | -8.5% |
| YUV, text & graphics with motion,720p | -4.8% | -9.5% | -30.6% | -0.6% |
| YUV, mixed content, 1440p | -11.7% | -11.0% | -15.5% | -6.6% |
| YUV, mixed content, 1080p | -9.8% | -9.8% | -9.8% | -9.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |  |
| Dec Time[%] | 111% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -16.5% | -16.0% | -26.9% | -3.1% |
| RGB, text & graphics with motion,720p | -0.9% | -4.9% | -18.0% | -0.2% |
| RGB, mixed content, 1440p | -2.1% | -2.2% | -3.9% | -0.4% |
| RGB, mixed content, 1080p | -1.6% | -1.6% | -1.6% | -1.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -16.1% | -17.1% | -29.9% | -3.9% |
| YUV, text & graphics with motion,720p | -1.0% | -6.3% | -23.6% | -0.3% |
| YUV, mixed content, 1440p | -2.1% | -2.3% | -4.0% | -0.5% |
| YUV, mixed content, 1080p | -1.9% | -1.9% | -1.9% | -1.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -16.3% | -12.3% | -18.1% | -2.5% |
| RGB, text & graphics with motion,720p | -0.6% | -2.0% | -6.9% | -0.2% |
| RGB, mixed content, 1440p | -1.2% | -1.2% | -2.3% | -0.1% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -15.6% | -13.1% | -18.3% | -3.2% |
| YUV, text & graphics with motion,720p | -0.5% | -2.5% | -9.0% | -0.2% |
| YUV, mixed content, 1440p | -1.2% | -1.3% | -2.5% | -0.1% |
| YUV, mixed content, 1080p | -0.9% | -0.9% | -0.9% | -0.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 97% |  |  |  |

Table 5. Experimental results on lossless coding with IBC search range setting of 4CTUs

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -19.9% | -18.9% | -32.1% | -6.4% |
| RGB, text & graphics with motion,720p | -3.6% | -6.4% | -19.1% | -0.4% |
| RGB, mixed content, 1440p | -11.3% | -10.1% | -15.5% | -4.8% |
| RGB, mixed content, 1080p | -8.4% | -8.4% | -8.4% | -8.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -21.0% | -20.2% | -35.5% | -6.6% |
| YUV, text & graphics with motion,720p | -4.0% | -7.8% | -24.7% | -0.4% |
| YUV, mixed content, 1440p | -11.2% | -10.4% | -15.3% | -5.5% |
| YUV, mixed content, 1080p | -9.3% | -9.3% | -9.3% | -9.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 120% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -14.2% | -13.2% | -21.5% | -2.5% |
| RGB, text & graphics with motion,720p | -0.8% | -4.0% | -14.6% | -0.2% |
| RGB, mixed content, 1440p | -2.0% | -2.1% | -3.8% | -0.4% |
| RGB, mixed content, 1080p | -1.6% | -1.6% | -1.6% | -1.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -13.8% | -14.0% | -23.8% | -3.0% |
| YUV, text & graphics with motion,720p | -0.8% | -5.1% | -18.9% | -0.1% |
| YUV, mixed content, 1440p | -2.0% | -2.2% | -3.9% | -0.4% |
| YUV, mixed content, 1080p | -1.8% | -1.8% | -1.8% | -1.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  |  |
| Dec Time[%] | 104% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -14.0% | -10.2% | -15.6% | -2.0% |
| RGB, text & graphics with motion,720p | -0.5% | -1.7% | -5.7% | -0.1% |
| RGB, mixed content, 1440p | -1.1% | -1.2% | -2.2% | -0.1% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -13.4% | -10.7% | -15.2% | -2.6% |
| YUV, text & graphics with motion,720p | -0.5% | -2.1% | -7.4% | -0.2% |
| YUV, mixed content, 1440p | -1.2% | -1.3% | -2.4% | -0.1% |
| YUV, mixed content, 1080p | -0.8% | -0.8% | -0.8% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 92% |  |  |  |

Table 6. Experimental results on lossless coding with IBC search range setting of 4CTUs + 4 upper lines

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -18.3% | -17.4% | -30.2% | -5.3% |
| RGB, text & graphics with motion,720p | -3.1% | -5.9% | -18.1% | -0.3% |
| RGB, mixed content, 1440p | -11.0% | -9.9% | -15.4% | -4.3% |
| RGB, mixed content, 1080p | -8.1% | -8.1% | -8.1% | -8.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -19.3% | -18.7% | -33.4% | -5.5% |
| YUV, text & graphics with motion,720p | -3.5% | -7.2% | -23.5% | -0.3% |
| YUV, mixed content, 1440p | -10.9% | -10.1% | -15.2% | -5.0% |
| YUV, mixed content, 1080p | -8.9% | -8.9% | -8.9% | -8.9% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 119% |  |  |  |
| Dec Time[%] | 100% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -13.2% | -12.3% | -20.2% | -2.3% |
| RGB, text & graphics with motion,720p | -0.7% | -3.8% | -13.9% | -0.1% |
| RGB, mixed content, 1440p | -1.9% | -2.0% | -3.7% | -0.3% |
| RGB, mixed content, 1080p | -1.5% | -1.5% | -1.5% | -1.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -12.8% | -13.1% | -22.4% | -2.7% |
| YUV, text & graphics with motion,720p | -0.8% | -4.8% | -18.0% | -0.2% |
| YUV, mixed content, 1440p | -2.0% | -2.1% | -3.9% | -0.4% |
| YUV, mixed content, 1080p | -1.7% | -1.7% | -1.7% | -1.7% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 95% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | -13.0% | -9.5% | -14.4% | -1.9% |
| RGB, text & graphics with motion,720p | -0.5% | -1.6% | -5.4% | -0.1% |
| RGB, mixed content, 1440p | -1.1% | -1.1% | -2.2% | -0.1% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -12.4% | -9.9% | -14.1% | -2.3% |
| YUV, text & graphics with motion,720p | -0.4% | -2.0% | -7.0% | -0.1% |
| YUV, mixed content, 1440p | -1.1% | -1.2% | -2.4% | -0.1% |
| YUV, mixed content, 1080p | -0.8% | -0.8% | -0.8% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  |  |
| Dec Time[%] | 93% |  |  |  |

# Comments

The results revealed that search range increase is generally beneficial to IBC coding efficiency on screen contents.  Since the IBC search range setting has a profound impact on memory bandwidth, off-chip memory size and on-chip cache size, it is recommended to consider the results of those IBC settings together with the other IBC designs in the same category to determine an IBC design that would result in a good trade-off between coding efficiency and implementation complexity.

# References

[1]	J. Sole, S. Liu, “HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions”. JCT-VC Document, JCTVC-Q1121, Valencia, ES, April 2014.

[2]  H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests”,  JCT-VC Document, JCTVC-Q1015, Valencia, ES, April 2014.

[3] W. Wang, Z. Ma, M. Xu, H. Yu, “SCCE1: Result of Test 1.2 for IBC with constrained buffers from left CTUs”, JCT-VC Document, JCTVC-R0141, Sapporo, JP, 30 June – 9 July, 2014

# Patent rights declaration(s)

Broadcom Corporation does not have current or pending patent rights relating to the technology described in this contribution.