| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0213 |
| --- | --- |

| Title: | Non-SCCE3: Palette and deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Chao Pang, Wei Pu, Rajan Joshi, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1 858 845 2429 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The palette methods in SCCE3 show good performance for screen content. The deblocking filter is applied for these palette coded blocks. However, generally, filtering and smoothing processes are avoided for screen content coding. It is proposed to not deblock CUs coded with palette mode in the same way that PCM CUs and lossless CUs (i.e., cu_transquant_bypass_flag enabled) are not deblocked. Allegedly, this saves some complexity, avoids unwanted filtering and achieves BD-rate reductions (in the range of 0.0% to -0.1%). Additionally, a flag at SPS level could be included to enable/disable deblocking of palette coded CUs.

# Introduction

During the HEVC and HEVC Range Extensions development, filtering and smoothing have been generally avoided for the coding of screen content, as in the following examples:

- Transform skip avoids the DCT averaging and shows very good gains in SCC

- Flag intra_smoothing_disabled_flag was introduced in HEVC RExt to disable the filtering of intra prediction samples

- Interpolation filters are not used in intra block copying

- Several proposals in HEVC RExt showed performance gains for SCC when filtering processes are disabled

Another source of smoothing is the deblocking filter, which is still applied in combination with tools that are strictly targeting the improvement of screen content coding.

# Palette and deblocking

Palette mode is suitable to code regions with few number of colors; flat areas, edges and simple textures. This type of regions should not generally be filtered. However, the default behaviour of palette modes is to treat palette coded areas as any other intra mode, thus strong deblocking filter might be applied. The consequence is that edges might be smoothed.

The top part of Figure 1 shows the first frame of sc_console coded at QP 22 with SCCE3 software. The CUs using palette are highlighted. The bottom part of the Figure 1 highlights the palette pixels affected by the deblocking filter (in white). In the enlarged fragments shown in Figure 2, it is clear that deblocking filter fails to detect real edges, which are being filtered. For example, the horizontal grid lines and part of the text are being deblocked.

Figure 1. Top) First frame of sc_console at QP 22 with the palette coded areas highlighted in green. Bottom) Same frame showing pixels modified by the deblocking filter in palette.

Figure 2. Enlarged fragments showing the real edges (in red circles) distorted by the deblocking filter.

# Proposal

HEVC disables the deblocking filter for PCM CUs and CUs with cu_transquant_bypass_flag equal 1. It is proposed to use this mechanism in order to also disable the deblocking filter for CUs coded with palette mode.

Additionally, a flag could be included at the SPS to enable/disable the deblocking filter for palette modes, in the same way that is done for the PCM CUs, so that encoders have the flexibility to use deblocking filter for palette mode.

# Performance

The simulation results for the proposed method are provided on the top of the SCCE3 software, while other common test conditions for SCC [1] are applied. Encoding/decoding times are not reliable.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.2% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | -0.1% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | 0.0% | -0.2% | -0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 110% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.1% |
| RGB, text & graphics with motion,720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.0% | 0.1% | -0.2% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | -0.1% | -0.4% | -0.3% |
| YUV, Animation, 720p | -0.1% | 0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 113% |  |  |

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, ES, 27 March – 4 April 2014.

# WD Text

Base text is the 	High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 7.

## Disabling deblocking for palette mode

##### 8.7.2.5.7 Filtering process for a luma sample

…

When nDp is greater than 0 and one or more of the following conditions are true, nDp is set equal to 0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xP0 ][ yP0 ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

–	palette_mode_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

When nDq is greater than 0 and one or more of the following conditions are true, nDq is set equal to 0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQ0 ][ yQ0 ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

–	palette_mode_flag  of the coding unit that includes the coding block containing the sample q0 is equal to 1.

##### 8.7.2.5.8 Filtering process for a chroma sample

…

When one or more of the following conditions are true, the filtered sample value, p0′ is substituted by the corresponding input sample value p0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xP0 * SubWidthC ][ yP0 * SubHeightC ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

–	palette_mode_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1.

When one or more of the following conditions are true, the filtered sample value, q0′ is substituted by the corresponding input sample value q0:

–	pcm_loop_filter_disabled_flag is equal to 1 and pcm_flag[ xQ0 * SubWidthC ][ yQ0 * SubHeightC ] is equal to 1.

–	cu_transquant_bypass_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

–	palette_mode_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1.

## SPS flag

palette_loop_filter_disabled_flag specifies whether the deblocking filter process is disabled on reconstructed samples in a coding unit with palette_mode_flag equal to 1 as follows:

–	If palette_loop_filter_disabled_flag is equal to 1, the deblocking filter process on the reconstructed samples in a coding unit with palette_mode_flag equal to 1 is disabled.

–	Otherwise (palette_loop_filter_disabled_flag value is equal to 0), the deblocking filter process on the reconstructed samples in a coding unit with palette_mode_flag equal to 1 is not disabled.

When palette_loop_filter_disabled_flag is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).