| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0215 |
| --- | --- |

| Title: | Non-SCCE3: WD text fixes for palette |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Vadim Seregin, Wei Pu, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1 858 845 2429 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Some alleged inconsistencies in the SCCE3 palette design have been detected and fixes are proposed. The issues include the SPS flag for palette mode, the signalling of the maximum palette size, and the signalling of the number of new entries.

# SPS flag to enable palette

The SPS palette_enabled_flag is part of the SCC extensions, not the Range Extensions. This is to be properly reflected in the section 7.3.2.2 General sequence parameter RBSP syntax. Following changes are suggested:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| sps_range_extensions_flag | u(1) |
| sps_scc_extensions_flag | u(1) |
| sps_extension_67bits | u(67) |
| } |  |
| if( sps_range_extensions_flag ) |  |
| sps_range_extensions( ) |  |
| if( sps_scc_extensions_flag ) |  |
| sps_scc_extensions( ) |  |
| if( sps_extension_67bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.3.2.2.2 Sequence parameter set scc syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_enabled_flag | u(1) |
| } |  |

# Maximum palette size signalling

A known issue in the WD text of palette is that the maximum palette size is not defined. The software uses a macro (MAX_PLT_SIZE), which sets the maximum size to 31.

It is suggested to signal the maximum palette size at SPS or PPS level. For example, it can be signalled at the PPS like the maximum size of transform skip as follows.

- 7.3.2.3.1 General picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_extension_present_flag | u(1) |
| if( pps_extension_present_flag ) { |  |
| pps_range_extensions_flag | u(1) |
| pps_scc_extensions_flag | u(1) |
| pps_extension_67bits | u(67) |
| } |  |
| if( pps_range_extensions_flag ) |  |
| pps_range_extensions( ) |  |
| if( pps_scc_extensions_flag ) |  |
| pps_scc_extensions( ) |  |
| if( pps_extension_67bits ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.3.2.3.3 Picture parameter set scc extensions syntax

| pps_scc_extensions( ) { | Descriptor |
| --- | --- |
| if( palette_enabled_flag ) |  |
| max_palette_size | ue(v) |
| } |  |

max_palette_size specifies the maximum number of entries the palette may have. When not present, the value of max_palette_size is inferred to be equal to 0. The value of max_palette_size shall be in the range of 0 to 31, inclusive.

Note that palette mode still works if the number of entries is 0: all values are signalled as escape values. Alternatively, the case in which the maximum palette size is equal to 0, then palette mode could be disabled:

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( palette_enabled_flag && max_palette_size > 0 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS, nCbS ) |  |
| … |  |

# Number of signalled entries

The syntax element palette_num_signalled_entries specifies the number of entries in the palette that are explicitly signalled for the current coding unit. The current text and software has the following issues:

- Text and software do not match

  - Text uses truncated unary while software a unary code

- Text allows for signalling of more entries than the maximum palette size

- Text uses truncated unary, but the truncation limit is not reached

- In software, the unary code wastes a bit is when the palette size reaches maximum palette size

- In software, the condition is set wrongly to decode bins as long as the size is less or equal to the maximum palette size (this should be strictly less than maximum palette size)

It is propose to align text and software by using truncated unary and setting properly cMax as follows in the WD text:

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_num_signalled_entries | TR | cMax = max_palette_size - numPredPreviousPalette, cRiceParam = 0 |
|  | palette_entries | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | palette_escape_val | 9.3.3.10 | cIdx |
|  | palette_run_type_flag | FL | cMax = 1 |
|  | palette_index | FL | cMax = palette_size |
|  | palette_run | 9.3.3.11 | - |

This alignment has negligible impact in the performance of the SCCE3 software. Using common test conditions [1] and SCCE3 as anchor, the BD-rate is:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 100% |  |  |

# References

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” JCTVC-Q1015, Valencia, ES, 27 March – 4 April 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).