| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0216 |
| --- | --- |

| Title: | Compression performance of StEM content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chad Fogg 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | :Email: | chadfogg@gmail.com |
| Source: | MovieLabs |  |  |

_____________________________

# Abstract

The performance of AVC High 10 is compared to HEVC Main10 across four film sequences that were professionally graded as separate SDR (100-nit Bt.709) and HDR (4000-nit Bt.2020 PQ).tiff files from the same film arhive.  The coded results fit a general trend being investigated by studios that AVC High10 is capable of sufficiently representing HDR/WCG video, albeit at a higher bit-rate than HEVC for comparable objective and subjective qualities. The difference in R/D performance is in proportion to content noise or film grain. The usual ~40% BD-Rate savings has been observed for HDR clips coded as HEVC vs. AVC, but only when test clips are clean, such as oversampled prior to down-sampling and/or fairly noise free. However, even at nearly double the bitrate of HEVC, an AVC High10 bitstream has applications.

# Introduction

StEM sequences  [1] are processed with format conversion tools [2] and coded with the 10bit compiled configuration of a High10 AVC encoder [3] that has been used in the authoring of Blu-Ray titles. Performance measures agree with previous observations [3,4].   R/D plots showing behaviour of the codes are provided in the accompanying slides with this document (JCTVC-R0216r_results.PDF).

Based on the adaptation of AVC High10 in the same manner to HEVC Main10, it is requested that the HDR/WCG code points for ST 2084 in VUI and SEI for ST 2086 in HEVC be copied into the next edition of the AVC specification.

# References

- JCTVC-Q0228, “StEM Content Description”

- JCTVC-R0214

- http://www.videolan.org/developers/x264.html

[3] P. Larbier, “AVC/H.264 Encoding with 4:2:2 for Broadcasting Contribution”  http://x264.nl/x264/10bit_01-ateme_pierre_larbier_422_10-bit.pdf

[4] “Why does 10-bit Save Bandwidth” http://x264.nl/x264/10bit_02-ateme-why_does_10bit_save_bandwidth.pdf

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.