| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0220 |
| --- | --- |

| Title: | AHG13: On reference position calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto Takeshi Tsukuba Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this contribution, three modifications relating to reference position calculation are proposed. Proposal 1 suggests signalling scaled reference layer offsets, reference layer offsets, and resample phases associated with the common reference layer in a single loop. Proposal 2 suggests the variables representing reference layer offsets to be derived using the prediction by the corresponding scaled reference layer offset. Proposal 3 suggests removing clipping in reference position calculation so that whole reference layer picture can be accessed.

# Introduction

Reference position calculation was studied in AHG 13 and they suggested modified text. This contribution proposed several modifications on top of the suggested spec changes in the AHG 13 report.

# Proposal 1: Signalling for reference position calculation

It is proposed to modify syntax relating to reference position calculation. Table 1 shows syntax changes compared to the AGH 13 suggested text. The changes are summarized as follows.

1.	scaled reference offsets, reference layer offsets, and resampling phases syntax sharing the same reference layer index are signalled in the same loop

2.	two present flags: one for a set of reference layer offsets and one for a set of resampling phases

The proposed syntax can signal any combination of reference-position calculation parameters supported by the syntax in AHG 13 text, and have following additional benefits:

1.	In the typical two layer case, the proposed method helps avoiding the same 6 bit reference layer index three times, thus reduces bits.

2.	When all syntax relating to reference position calculation are not needed, which is typical in certain application such as multi view, they can be skipped by 1 bit (ie. by signalling num_scaled_ref_layer_offsets equals to 0).

3.	Number of additional syntax compared to the latest draft is less compared to AHG 13 syntax.

Table 1: Proposed syntax for information on reference position calculation

| pps_multilayer_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_id[ i ] | u(6) |
| scaled_ref_layer_left_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_top_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_right_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| scaled_ref_layer_bottom_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| } |  |
| num_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_ ref_layer_offsets; i++) { |  |
| ref_offset_layer_id[ i ] | u(6) |
| ref_layer_offset_present_flag[ i ] | u(1) |
| if ( ref_layer_offset_present_flag[ i ] ) { |  |
| ref_layer_left_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| ref_layer_top_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| ref_layer_right_offset[scaled_ref_layer_id[ i ] ] | se(v) |
| ref_layer_bottom_offset[ scaled_ref_layer_id[ i ] ] | se(v) |
| } |  |
| } |  |
| num_resample_phase | ue(v) |
| for( i = 0; i < num_resample_phase; i++) { |  |
| resample_phase_layer_id[ i ] | u(6) |
| resample _phase_present_flag[ i ] | u(1) |
| if ( resample_phase_present_flag[ i ]) { |  |
| phase_hor_luma[ scaled_ref_layer_id [ i ] ] | ue(v) |
| phase_ver_luma[ scaled_ref_layer_id [ i ] ] | ue(v) |
| phase_hor_chroma[ scaled_ref_layer_id [ i ] ] | ue(v) |
| phase_ver_chroma[ scaled_ref_layer_id [ i ] ] | ue(v) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

# Proposal 2, Prediction value for reference layer offset

In typical cases, reference region on the current picture and the reference region on the reference picture share the same portion of each corresponding picture. For example, when SNR scalability is used, the reference region relative to the current picture is typically the same as the reference region relative to the reference picture. Fig 1 illustrates such cases.

Fig 1. Typical cases where reference region have the same position and size with respect to the corresponding picture.

In the SNR scalability case mentioned above, exactly the same reference layer offset values need to be signalled in the both current and reference layer. To avoid overhead in the typical case, it is proposed to predict offsets on a reference layer based on the corresponding offsets in the current layer and the picture size ratio between the current layer picture and the reference layer picture.

Reference region offset on the reference layer can be calculated as follows.

RefLayerLeftOffset = 
	(((RefPicWidthInSamplesY * scaled_ref_layer_left_offset[rLIdx]) / CurPicWidthInSamplesY) 
	+ (ref_layer_left_offset[rLIdx])) * RefLayerSubWidthC

Other offsets are derived with the similar way.

# Proposal 3: Clipping in reference position calculation

It is proposed to remove the clipping operation prior to interpolation process. With the clipping, the reference position on a reference picture accessed by resampling process is restricted in the window defined by reference layer offset paramters. When the clipping is removed, the reference position is restricted in the reference picture. Thus, when the reference picture contains meaningful texture outside of the reference layer offset window, those pixels can be used to improve inter-layer texture prediction. Fig 2 illustrates reference position calculation process with or without clipping.

(a) with clipping

(b) remove clipping

Fig 2. Implication of removing clipping in reference position calculation process

Proposed text change for Proposal 3

- H.8.1.4.1.1	Resampling process of luma sample values

Input to this process is the reference luma sample array rlPicSampleL.

Output of this process is the resampled luma sample array rsPicSampleL.

The variables leftStartL, rightEndL, topStartL, and bottomEndL are derived as follows:

leftStartL = ScaledRefLayerLeftOffset
rightEndL = CurPicWidthInSamplesY − ScaledRefLayerRightOffset	(H-1)
topStartL = ScaledRefLayerTopOffset
bottomEndL = CurPicHeightInSamplesY − ScaledRefLayerBottomOffset	(H-2)

The luma samples rsPicSampleL[ xP ][ yP ] with ( xP = 0 ... CurPicWidthInSamplesY − 1, yP = 0 ... CurPicHeightInSamplesY − 1) are derived by invoking the luma sample interpolation process specified in subclause Error: Reference source not found with rlPicSampleL and luma sample location ( xP, yP )( Clip3( leftStartL, rightEndL − 1, xP ), Clip3( topStartL, bottomEndL − 1, yP ) ) given as inputs and rsPicSampleL[ xP ][ yP ] as output.

- H.8.1.4.1.2	Resampling process of chroma sample values

Input to this process is the reference chroma sample array rlPicSampleC,

Output of this process is the resampled chroma sample array rsPicSampleC.

The variables leftStartC, rightEndC, topStartC, and bottomEndC are derived as follows:

leftStartC = ScaledRefLayerLeftOffset / SubWidthC
rightEndC = ( CurPicWidthInSamplesY− ScaledRefLayerRightOffset ) / SubWidthC	(H-3)
topStartC = ScaledRefLayerTopOffset / SubHeightC
bottomEndC = ( CurPicHeightInSamplesY− ScaledRefLayerBottomOffset ) / SubHeightC	(H-4)

The chroma samples rsPicSampleC[ xPC ][ yPC ] with ( xPC = 0 ... CurPicWidthInSamplesC − 1, yPC = 0 ... CurPicHeightInSamplesC − 1) are derived by invoking the chroma sample interpolation process specified in subclause Error: Reference source not found with rlPicSampleC and chroma sample location ( xPC, yPC ) ( Clip3( leftStartC, rightEndC − 1, xPC ), Clip3( topStartC, bottomEndC − 1, yPC ) ) given as inputs and rsPicSampleC[ xPC ][ yPC ] as output.

# Patent rights declaration

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).