| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0225 |
| --- | --- |

| Title: | Non-SCCE4: Constrained run for 1D dictionary coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou Ying Chen Joel Sole Marta Karczewicz | Tel: Email: | +1-858-845-1115 fzou@qti.qualcomm.com cheny@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal presents preliminary results by constraining the current 1D dictionary (string match) methods investigated under the current SCCE4. It is reported that the current 1D dictionary methods may require much more times of memory access from different locations of the reference area compared to e.g., Intra BC. Therefore, it could be beneficial to address more pixels for each string match. In this proposal, the matching length is constrained to be equal or larger than a predefined threshold T, e.g., 4. The simulation results show that the constraint as implemented based on SCCE4 proposals JCTVC-R0098 and JCTVC-R0140 leads to noticeable bitrate increase. It is suggested to have discussions on 1) whether the proposed constraint is needed, if yes, 2) in which extent such a constraint should apply, e.g., what is a suitable number of T? and 3) in which level of reference region such a constraint does not need to apply, e.g., should the constraint for a match from pixels within the same CU and that for a match from pixels outside the current CTU be different?

# Introduction

In the 17th JCT-VC meeting in Valencia, SCCE4 [1] was established to study different sample matching methods. In SCCE4 JCTVC-R0098 [2] and JCTVC-R0140 [3], the sample matching length can be as small as 1 pixel.

## Problem in the 1D dictionary

Although 1D dictionary doesn’t require interpolation, in the worst case scenario, each pixel of the 1D coded CU can be predicted from a different position and none of the reference pixels are adjacent to the each other. In such a case, especially of those reference pixels are not within the current CU or CTU, the memory access bandwidth  is considerably high since the efficiency of 1 pixel access is less than that of accessing e.g., multiple pixels of a block at a time, in average.

Therefore, it is desirable to constrain the minimal number of pixels during matching, such that each memory access can become relatively more efficient.

## Typical memory accesses in HEVC and SCC

Although the cost/efficiency of different types of memory access is implementation dependent, they are considered different in HEVC and SCC, depending on the reference region.

### Intra prediction reference area

The top row and the left column of the current CU are always considered available and accessing pixels within such a reference region does not seem to require any additional memory access.

### Pixels within the current coding unit

Typically, when the current CU is processed, the current CU buffers (original pixel, and reconstruction pixel buffers) are available and stored in Cache. Accessing such fast local buffer is achievable in typical implementations. Therefore, accessing the pixels in the current CU is achievable for 1D dictionary coding.

### Pixels within 2 CTUs

It has been discussed during the development of Range extension as well as Screen Content Coding (SCC), that accessing pixels within the current CTU and the left CTU (if available for reference) are relatively more friendly to implementations.

### Other pixels

Pixels with a relatively far distance than the pixels in the above discussed reference regions are relatively more expensive to access and accessing one block from such a region can be as expensive as accessing a block of the same size from a picture in the decoded picture buffer.

# Proposed matching length constraint on 1D dictionary

## Constrained run on JCTVC-R0098 (Microsoft)

In JCTVC-R0098, two types of 1D dictionary modes are proposed at the CU level. One is normal 1D dictionary mode and the other is reconstruction based 1D dictionary mode. Both modes need to signal the matching length, which is as small as 1 pixel. In the worst case, every pixel in the current CTU (64x64) has a matched reference pixel and 64x64 individual pixel fetching and address calculation are invoked. In JCTVC-R0098, the reference buffer is maintained with a size of 1<<18 for normal dictionary mode and full frame for reconstruction based 1D dictionary mode. Basically, the reference pixel could be located anywhere within the current frame.

Therefore, it is desirable to constrain the minimum number of matching pixels to reduce the memory access and address calculation burden. In this proposal, such constraint is applied to both1D modes in JCTVC-R0098. Specifically, during pixel matching, if the matching length is smaller than a predefined threshold, for instance T, the pixel is forced to be escape mode with the pixel values coded explicitly. Different threshold value T is tested for JCTVC-R0098. When T is 4, (64x64/4) memory access and address calculation are invoked.

## Constrained run on JCTVC-R0140 (Qualcomm)

In JCTVC-R0140, a CU level 1D dictionary mode is proposed with a search range constrained to 2CTU (left CTU and the current CTU). And JCTVC-R0140 shares the same reconstruction buffer with intra BC, with up to 2 CTU search range. When a matching reference pixel is found, it needs to signal the matching length, which is as small as 1 pixel. In the worst case, 64x64 individual pixel fetching and address calculation are invoked. It is proposed to constrain the matching length, such that during pixel matching, if the matching length is smaller than a predefined threshold, for instance T, the pixel is forced to be escape mode with the pixel values coded explicitly. When the constraint run T is applied, (64x64/T) memory access and address calculation are achieved instead of (64x64) in the worst case.

# Simulation

## Results

The proposed scheme is implemented on top of JCTVC-R0098 and JCTVC-R0140 and tested using the common test condition defined in [4]. Table 1 and Table 2 demonstrate the coding performance under full frame intra BC test conditions when different matching length constraints are applied.

Table 1. Results for constrained run based on JCTVC-R0098 in full frame intra BC condition

|  |  |  | JCTVC-R0098 |  | Proposed |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  | All Intra |  |  |  |  |
|  | G/Y | B/U |  | R/V | G/Y | B/U |  | R/V |  |
| RGB, text & graphics with motion, 1080p | -21.3% | -22.1% |  | -22.0% | -18.0% | -18.6% |  | -18.7% |  |
| RGB, text & graphics with motion,720p | -13.0% | -13.4% |  | -13.2% | -8.8% | -9.2% |  | -9.1% |  |
| RGB, mixed content, 1440p | -2.5% | -2.5% |  | -2.5% | -2.1% | -2.2% |  | -2.2% |  |
| RGB, mixed content, 1080p | -6.0% | -6.1% |  | -6.0% | -5.5% | -5.6% |  | -5.5% |  |
| RGB, Animation, 720p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p | -20.7% | -21.3% |  | -20.6% | -17.2% | -17.3% |  | -16.7% |  |
| YUV, text & graphics with motion,720p | -10.4% | -12.7% |  | -14.3% | -7.2% | -7.6% |  | -8.1% |  |
| YUV, mixed content, 1440p | -2.0% | -2.2% |  | -2.1% | -1.8% | -2.0% |  | -1.9% |  |
| YUV, mixed content, 1080p | -5.1% | -5.2% |  | -4.8% | -4.5% | -4.7% |  | -4.4% |  |
| YUV, Animation, 720p | 0.1% | 0.0% |  | 0.0% | 0.1% | 0.0% |  | 0.1% |  |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.1% |  |
| Enc Time[%] | 154% |  |  |  | 153% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  | 102% |  |  |  |  |

Table 2. Results for constrained run based on JCTVC-R0140 in full frame intra BC condition

|  |  |  | JCTVC-R0140 |  | Proposed |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  | All Intra |  |  |  |  |
|  | G/Y | B/U |  | R/V | G/Y | B/U |  | R/V |  |
| RGB, text & graphics with motion, 1080p | -14.4% | -15.0% |  | -14.9% | -7.8% | -8.0% |  | -8.1% |  |
| RGB, text & graphics with motion,720p | -6.4% | -6.2% |  | -6.4% | -2.4% | -2.4% |  | -2.4% |  |
| RGB, mixed content, 1440p | -0.8% | -0.7% |  | -0.8% | -0.4% | -0.4% |  | -0.4% |  |
| RGB, mixed content, 1080p | -1.8% | -1.8% |  | -1.6% | -0.8% | -0.7% |  | -0.7% |  |
| RGB, Animation, 720p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% |  |
| RGB, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% |  |
| YUV, text & graphics with motion, 1080p | -11.9% | -13.3% |  | -12.7% | -7.1% | -7.7% |  | -7.3% |  |
| YUV, text & graphics with motion,720p | -3.9% | -5.2% |  | -6.4% | -2.2% | -2.6% |  | -3.2% |  |
| YUV, mixed content, 1440p | -0.5% | -1.0% |  | -0.8% | -0.4% | -0.7% |  | -0.6% |  |
| YUV, mixed content, 1080p | -1.1% | -1.4% |  | -1.1% | -0.4% | -0.7% |  | -0.6% |  |
| YUV, Animation, 720p | 0.1% | 0.0% |  | 0.0% | 0.1% | 0.0% |  | 0.0% |  |
| YUV, camera captured, 1080p | 0.0% | 0.0% |  | 0.0% | 0.0% | 0.0% |  | 0.0% |  |
| Enc Time[%] | 200% |  |  |  | 200% |  |  |  |  |
| Dec Time[%] | 96% |  |  |  | 97% |  |  |  |  |

## Limitations of the current implementation

Although the proposed method has noticeable gain by apply the constraint, it may be improved from the following aspects.

- Release the constraint for pixels within certain regions. For example, it might happen frequently a match is from reference pixels of the same CTU, in this case, the constraint might not need to apply.

- Apply more restrictive constraint from reference regions that are really expensive to access. For example, when a current match is found outside the current CTU and the left CTU, the threshold T may be set even to 16 to make the efficiency of memory access similar to Intra BC coded 4x4 block.

# Discussions

The proponent feels that the current 1D dictionary, although typically doesn’t increase the decoding time at all, might have an implementation implication that much less efficiency memory bandwidth is required per pixel. Therefore, it is worthwhile to clarify within the JCT-VC group whether this is a concern and resolve it before a 1D dictionary method can be adopted into SCC.

More specifically we would like to clarify the following questions.

- Whether the above mentioned memory bandwidth requirement is a problem at least for full frame 1D dictionary search.

- If 1D dictionary search is constrained to a limited range, e.g., within 2 CTU or CU, could the memory access be much more efficient and be considered as acceptable, similar to the memory copies as in palette mode.

- If the memory bandwidth requirement is an issue, would the proposed constraint solve the problem, especially with a relatively larger number of constrained run? If such a constraint is desirable, could different regions use different minimum number of pixels per run?

- If the memory bandwidth requirement is an issue, whether we encourage other methods to solve it and what is the expected coding efficiency and complexity trade-off, from the decoder perspective?

# References

[1] Y. Chen and J. Xu, “HEVC Screen Content Coding Core Experiment 4 (SCCE4): String matching ofr sample coding”, JCT-VC Document, JCTVC-Q1124, Valencia, Spain, March 2014.

[2] B. Li and J. Xu, “SCCE4: Result of Test 3.1”, JCT-VC Document, JCTVC-R0098, Sapporo, Japan, June 2014.

[3] F. Zou, Y. Chen, J. Sole, M. Karczewicz , “SCCE4: Test 3.2 Pixel based 1D dictionary coding”, ”, JCT-VC Document, JCTVC-R0140, Sapporo, Japan, June 2014.

[4] H. Yu, R. Cohen, K. Rapaka, J. Xu , “Common conditions for screen content coding tests”, JCTVC-Q1015, March 2014

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).