| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0227v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 9th Meeting: Sapporo, JP, 3–9 July 2014 | Document: JCT3V-I0045v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Miscellaneous cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry Adarsh K. Ramasubramonian Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-651-8345 yekuiw@qti.qualcomm.com fhendry@qti.qualcomm.com aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

V2 (revision 1) of this document provides a second solution option for item 4.

This document proposes the following changes:

- Signalling of the profile, tier, and level (PTL) information for the 0-th output layer set (OLS)

- Signalling of bit rate and picture rate information for additional layer sets

- Adding a restriction on update of representation format of the base layer

- Not sending video signal information for externally provided base layer

- Bug fix and simplification for value ranges of num_negative_pics, num_positive_pics, and num_long_term_pics

- Conditional signalling or restriction of vps_vui_bsp_hrd_present_flag

- No restriction on update of separate_color_plane_flag

# Proposal

## Signalling of PTL for the 0-th OLS

The 0-th output layer set (OLS) specified by the active VPS refers to the OLS that contains only the base layer (the layer with nuh_layer_id equal to 0), which is also the only output layer. However, unless the entire bitstream contains only the base layer, the tier and level information of the 0-th OLS is not signalled. When the entire bitstream does contain only the base layer, the tier and level information of the 0-th OLS is signalled in the profile_tier_level( ) syntax structure of the active SPS as well as in the first profile_tier_level( ) syntax structure in the active VPS. However, when the entire bitstream contains more layers than just the base layer, the tier and level information of the 0-th OLS signalled in the profile_tier_level( ) syntax structure of the active SPS as well as in the first profile_tier_level( ) syntax structure in the active VPS would be for the entire bitstream with only the base layer as the output layer.

It is therefore proposed that the PTL information for the 0-th OLS is always signalled in the active VPS as long as the base layer is present in the bitstream.

The syntax of the vps_extension( ) syntax structure is changed as follows, and there is no need to change the semantics:

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| NumOutputLayerSets = num_add_olss + NumLayerSets |  |
| for( i = vps_base_layer_internal_flag ? 0 : 1; i < NumOutputLayerSets; i++ ) { |  |
| if( i >= NumLayerSets ) |  |
| layer_set_idx_for_ols_minus1[ i ] | u(v) |
| if( i > vps_num_layer_sets_minus1 ¦ ¦ ( defaultOutputLayerIdc = = 2 && i > 0 ) |  |
| for( j = 0; j < NumLayersInIdList[ OlsIdxToLsIdx[ i ] ]; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| profile_level_tier_idx[ i ] | u(v) |
| if( NumOutputLayersInOutputLayerSet[ i ] = = 1 && NumDirectRefLayers[ OlsHighestOutputLayerId[ i ] ] > 0 ) |  |
| alt_output_layer_flag[ i ] | u(1) |
| } |  |
| ... |  |
| } |  |

## Signalling of bit rate and picture rate information for additional layer sets

Currently the bit rate and picture rate information is signalled for layer sets, which are specified in the base part of the VPS, but not for the additional layer sets, which correspond to independent sets of layers not containing the base layer and are specified in the VPS extension.

The bit rate and picture rate information should be signalled for all layer sets, including the additional layer sets, as clients may choose to request or consume an additional layer set based on such information.

It is therefore proposed to signal bit rate and picture rate information also for the additional layer sets.

The syntax of the vps_extension( ) syntax structure is changed as follows, and there is no need to change the semantics:

| vps_vui( ){ | Descriptor |
| --- | --- |
| … |  |
| bit_rate_present_vps_flag | u(1) |
| pic_rate_present_vps_flag | u(1) |
| if( bit_rate_present_vps_flag ¦ ¦ pic_rate_present_vps_flag ) |  |
| for( i = vps_base_layer_internal_flag ? 0 : 1; i < NumLayerSets <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= MaxSubLayersInLayerSetMinus1[ i ]; j++ ) { |  |
| if( bit_rate_present_vps_flag ) |  |
| bit_rate_present_flag[ i ][ j ] | u(1) |
| if( pic_rate_present_vps_flag ) |  |
| pic_rate_present_flag[ i ][ j ] | u(1) |
| if( bit_rate_present_flag[ i ][ j ] ) { |  |
| avg_bit_rate[ i ][ j ] | u(16) |
| max_bit_rate[ i ][ j ] | u(16) |
| } |  |
| if( pic_rate_present_flag[ i ][ j ] ) { |  |
| constant_pic_rate_idc[ i ][ j ] | u(2) |
| avg_pic_rate[ i ][ j ] | u(16) |
| } |  |
| } |  |
| … |  |
| } |  |

## A restriction on update of representation format of the base layer

The constraint that when representation format parameters for a layer are updated in an SPS, the updated representation format parameters shall not be greater than the corresponding representation format parameters for that layer that is assigned in VPS only applies to layers that have nuh_layer_id greater than 0. For the base layer, there is no constraint on the value of representation format parameters signalled in the active SPS for that layer.

The objective of the above constraint is to ensure that the values of representation format parameters that are signalled for each layer in the VPS are the largest value of those parameters that decoder should anticipate for that layer. This informs the decoder how much memory should be allocated to handle pictures of each layer, and is useful in session negotiation to determine whether a decoder can decode a bitstream. However, the constraint does not apply to the base layer; hence the value of representation format parameters signalled in the active SPS for the base layer can be greater than the value of the corresponding representation format parameters that is assigned for the base layer in the VPS. This will defeat the purpose of constraining the representation format update for other layers.

Furthermore, when the picture size information in the VPS instead of the SPS is used for level definitions, allowing update of the picture size to be greater than that in the VPS would result in that conforming decoders are not guaranteed to be able to decode the bitstream.

It is therefore proposed to add a constraint to require that the value of a representation format parameter for the base layer in the VPS shall be greater than or equal to the value of the corresponding representation format parameter in all active SPSs for the base layer that refer to the VPS. The following constraint is proposed to be added to the SPS semantics:

When an SPS with nuh_layer_id equal to 0 is the active SPS of a layer with nuh_layer_id equal to 0, it is a requirement of bitstream conformance that the value of syntax elements chroma_format_idc, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, or bit_depth_chroma_minus8 present in the SPS shall be less than or equal to chroma_format_vps_idc, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, or bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ 0 ]-th rep_format( ) syntax structure in the active VPS.

## On video signal information for externally provided base layer

The base layer may be not present in the bitstream but provided externally. When provided externally, signalling of the video signal information for the base layer is not useful. One may argue to use it for session negotiation, however since even the information of PTL, which is the utmost important information for session negotiation, is not signalled, this argument is moot. Yet, in the current signalling of video signal information in the VPS VUI, when vps_num_video_signal_info_minus1 is not present, it will be inferred to be equal to the number of layers, and one video signal information structure would be signalled for each layer ID value, and each layer is assigned to the signalled video signal information structure according to its layer ID value. Consequently, when the base layer is provided externally, there is one signal information structure sent but useless.

In option 1, it is proposed to not send the video signal information for the base layer when it is externally provided. It is noted that the current syntax only allows sending the video signal information in the VPS for layer 0 only when the indices are inferred, while when the indices are explicitly signalled, the video signal information is not sent for layer 0 regardless of whether it is present in the bitstream or not. Thus, not to send the information regardless of whether to infer or to explicitly signal the indices seems to be more consistent.

The syntax is copied below for convenience.

| vps_vui( ){ | Descriptor |
| --- | --- |
| ... |  |
| video_signal_info_idx_present_flag | u(1) |
| if( video_signal_info_idx_present_flag ) |  |
| vps_num_video_signal_info_minus1 | u(4) |
| for( i = 0; i <= vps_num_video_signal_info_minus1; i++ ) |  |
| video_signal_info( ) |  |
| if( video_signal_info_idx_present_flag && vps_num_video_signal_info_minus1 > 0 ) |  |
| for( i = 1; i <= MaxLayersMinus1; i++ ) |  |
| vps_video_signal_info_idx[ i ] | u(4) |
| ... |  |
| } |  |

The semantics of video_signal_info_idx_present_flag, vps_num_video_signal_info_minus1, and vps_video_signal_info_idx[ i ] are changed as follows:

video_signal_info_idx_present_flag equal to 1 specifies that the syntax element vps_num_video_signal_info_minus1 is present and the syntax element vps_video_signal_info_idx[ i ] may be present. video_signal_info_idx_present_flag equal to 0 specifies that the syntax element vps_num_video_signal_info_minus1 and vps_video_signal_info_idx[ i ] are not present.

vps_num_video_signal_info_minus1 plus 1 specifies the number of the following video_signal_info( ) syntax structures in the VPS. When not present, the value of vps_num_video_signal_info_minus1 is inferred to be equal to MaxLayersMinus1 − ( vps_base_layer_internal_flag ? 0 : 1 ).

vps_video_signal_info_idx[ i ] specifies the index, into the list of video_signal_info( ) syntax structures in the VPS, of the video_signal_info( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. When vps_video_signal_info_idx[ i ] is not present, vps_video_signal_info_idx[ i ] is inferred to be equal to ( video_signal_info_idx_present_flag ? 0 : i ). The value of vps_video_signal_info_idx[ i ] shall be in the range of 0 to vps_num_video_signal_info_minus1, inclusive.

When not present, the value of vps_video_signal_info_idx[ i ] is inferred as follows:

–	If video_signal_info_idx_present_flag is equal to 1, vps_video_signal_info_idx[ i ] is inferred to be equal to 0.

–	Otherwise, vps_video_signal_info_idx[ i ] is inferred to be equal to i – ( vps_base_layer_internal_flag ? 0 : 1

In option 2, which is provided in response to earlier reviews of this item, syntax and semantics are changed to allow for either sending or not sending the video signal information for the base layer when it is externally provided, and also signalled for layer 0 when it is present in the bitstream. When video signal information is present, the indices are always explicitly signalled.

| vps_vui( ){ | Descriptor |
| --- | --- |
| ... |  |
| vps_num_video_signal_info_minus1 | u(4) |
| for( i = 0; i <= vps_num_video_signal_info_minus1; i++ ) |  |
| video_signal_info( ) |  |
| if( !vps_base_layer_internal_flag ) |  |
| vsi_for_layer0_present_flag | u(1) |
| for( i = vsi_for_layer0_present_flag ? 0 : 1; i <= MaxLayersMinus1; i++ ) |  |
| vps_video_signal_info_idx[ i ] | u(4) |
| ... |  |
| } |  |

vps_num_video_signal_info_minus1 plus 1 specifies the number of the following video_signal_info( ) syntax structures in the VPS. When not present, the value of vps_num_video_signal_info_minus1 is inferred to be equal to 0.

vsi_for_layer0_present_flag equal to 1 specifies that the syntax element vps_video_signal_info_idx[ 0 ] is present. vsi_for_layer0_present_flag equal to 0 specifies that the syntax element vps_video_signal_info_idx[ 0 ] is not present. When vps_num_video_signal_info_minus1 is present and equal to 0, the value of vsi_for_layer0_present_flag is inferred to be equal to 1.

vps_video_signal_info_idx[ i ] specifies the index, into the list of video_signal_info( ) syntax structures in the VPS, of the video_signal_info( ) syntax structure that applies to the layer with nuh_layer_id equal to layer_id_in_nuh[ i ]. The value of vps_video_signal_info_idx[ i ] shall be in the range of 0 to vps_num_video_signal_info_minus1, inclusive.

## Value ranges for num_negative_pics, num_positive_pics, and num_long_term_pics

The value ranges of the syntax elements num_negative_pics, num_positive_pics, and num_long_term_pics are specified based on a variable maxNumPics.

However, when the short-term RPS is included in an SPS, the nuh_layer_id used in the pseudo code for derivation of maxNumPics is incorrect, as it should be nuh_layer_id of a layer referring to the SPS instead of nuh_layer_id of the SPS. To fix this problem, one more loop, a loop of nuh_layer_id values for all the layers that refer to the SPS, would be needed.

Furthermore, the existing pseudo code is already too complicated for the purpose of specifying a value range, and it is still not the tightest range anyway as the number of pictures not needed for reference but waiting for output is not considered. This is not only complicated editorially and requires effort to understand, but also complicated for conformance testing/checking.

It is proposed to fix the bug by specifying simple value ranges for these three syntax elements.

The general slice segment header semantics in subclause F.7.4.7.1 are changed as follows:

...

num_long_term_pics specifies the number of entries in the long-term RPS of the current picture that are directly signalled in the slice header. When not present, the value of num_long_term_pics is inferred to be equal to 0.

The variable maxNumPics is derived as follows:

maxNumPics = MaxDpbSize - 1
for( olsIdx = 0; olsIdx < NumOutputLayerSets; olsIdx++) {
	lsIdx = OlsIdxToLsIdx[ olsIdx ]
	for( j = 0; j < NumLayersInIdList[ lsIdx ]; j++ )
		if( LayerSetLayerIdList[ lsIdx ][ j ]  = =  nuh_layer_id ) {
			maxSL = MaxSubLayersInLayerSetMinus1[ lsIdx ]
			maxNumPics = Min( maxNumPics, max_vps_dec_pic_buffering_minus1[ olsIdx ][ j ][ maxSL ] )
		}
}

When nuh_layer_id is equal to 0, the sum of NumNegativePics[ CurrRpsIdx ], NumPositivePics[ CurrRpsIdx ], num_long_term_sps, and num_long_term_pics shall be less than or equal to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ]. When vps_extension_flag is equal to 1, the sum of NumNegativePics[ CurrRpsIdx ], NumPositivePics[ CurrRpsIdx ], num_long_term_sps, and num_long_term_pics shall be less than or equal to MaxDpbSize – 1maxNumPics.

...

The short-term reference picture set semantics in subclause F.7.4.8 are changed as follows:

The specifications in subclause 7.4.8 apply, with the following modifications:

The variable maxNumPics is derived as follows:

maxNumPics = MaxDpbSize - 1
for( olsIdx = 0; olsIdx < NumOutputLayerSets; olsIdx++) {
	lsIdx = OlsIdxToLsIdx[ olsIdx ]
	for( j = 0; j < NumLayersInIdList[ lsIdx ]; j++ )
		if( LayerSetLayerIdList[ lsIdx ][ j ]  = =  nuh_layer_id ) {
			maxSL = MaxSubLayersInLayerSetMinus1[ lsIdx ]
			maxNumPics = Min( maxNumPics, max_vps_dec_pic_buffering_minus1[ olsIdx ][ j ][ maxSL ] )
		}
}

num_negative_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture. When nuh_layer_id is equal to 0, the value of num_negative_pics shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ], inclusive. When vps_extension_flag is equal to 1, the value of num_negative_pics shall be in the range of 0 to MaxDpbSize – 1maxNumPics, inclusive.

num_positive_pics specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values greater than the picture order count value of the current picture. When nuh_layer_id is equal to 0, the value of num_positive_pics shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ sps_max_sub_layers_minus1 ] − num_negative_pics, inclusive. When vps_extension_flag is equal to 1, the value of num_positive_pics shall be in the range of 0 to maxNumPics MaxDpbSize – 1 − num_negative_pics, inclusive.

## Conditional sending or restriction of vps_vui_bsp_hrd_present_flag

The signalling of HRD parameters in the VPS VUI is controlled by a gating flag vps_vui_bsp_hrd_present_flag. The value of this flag currently can be set equal to either 1 or 0 regardless of the value of vps_timing_info_present_flag. However, when there is no timing information signalled (i.e. the value of vps_timing_info_present_flag is equal to 0), signalling of HRD parameters in the VPS VUI is meaningless.

Therefore it is proposed to signal vps_vui_bsp_hrd_present_flag only when vps_timing_info_present_flag is equal to 1 and infer the value to be equal to 0 when the flag is not present, or require that the value of the flag shall be equal to 0 when vps_timing_info_present_flag is equal to 0. Conditional signalling is preferred by the proponents.

## On update of separate_color_plane_flag

The representation format syntax structure in the VPS specifies, for each layer, the parameters that are needed to describe some essential properties of the coded picture – including picture width, picture height, bit depth of the luma and chroma components, as well as chroma format and how the colour planes are coded. In SPSs that have nuh_layer_id greater than 0, the representation format parameters are allowed to be updated using the update_rep_format_flag indication. For a layer that has nuh_layer_id greater than 0 and that refers to an SPS with nuh_layer_id greater than 0, when the SPS has update_rep_format_flag equal to 1, the updated representation format parameters are restricted to be less than that signalled in the representation format parameters in the VPS corresponding to that layer.

Of these representation format syntax elements – chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, and bit_depth_chroma_minus8 – all of them except separate_colour_plane_flag specify parameters that have a linear relationship with the value of the syntax element. However, separate_colour_plane_flag does not have such relationship; it describes whether the different colour planes of a picture that refers to the SPS are coded together or coded separately.

The motivation of putting the restriction on the update of representation format parameters was to ensure that resources/capabilities indicated as required by the VPS are not exceeded by an updated SPS. However, neither value 0 nor 1 of the separate_colour_plane_flag indicates any additional data to be signalled or processed compared to the other value, and hence separate_colour_plane_flag should not be considered for this restriction.

It is therefore proposed that separate_colour_plane_flag is removed from the restriction on update of representation format parameters. The changes are given below in red strikethrough (deletion).

–	When update_rep_format_flag is equal to 1, it is a requirement of bitstream conformance that the value of chroma_format_idc, separate_colour_plane_flag, pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, or bit_depth_chroma_minus8 shall be less than or equal to chroma_format_vps_idc, separate_colour_plane_vps_flag, pic_width_vps_in_luma_samples, pic_height_vps_in_luma_samples, bit_depth_vps_luma_minus8, or bit_depth_vps_chroma_minus8, respectively, of the vps_rep_format_idx[ j ]-th rep_format( ) syntax structure in the active VPS, where j is equal to LayerIdxInVps[ layerIdCurr ].

It is suggested to discuss and conclude whether it should be restricted that the value of separate_colour_plane_flag shall be the same in all the rep_format( ) syntax structures in a VPS, or just signal it at most once in a VPS.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).