| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0228 |
| --- | --- |

| Title: | Non-SCCE3: Run-length coding for palette predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Marta Karczewicz, Wei Pu, Rajan Joshi, Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is related to the palette method studied in SCCE3 and method of signaling the palette entries predicted from the palette predictor formed from the palette entries of the previous coded blocks. It is proposed to use run-length coding to indicate which entries are predicted. Additionally, the palette predictor size is increased from 64 to 128.

# Introduction

Palette entries of the current block can be predicted from the palette predictor consisting of the palette entries of the previously palette coded blocks. The default size of the palette predictor is 64. The prediction can be represented with the binary vector of the size equal to the palette predictor size, where 1 indicates that the entry is reused from the predictor, and 0 otherwise.

# Run-length coding

This contribution proposes to use run-length coding of the zero elements in the binary vector with the following steps:

- run-length value equal to 1 indicates end of prediction

- the end of prediction is not signaled for the last 1 in the binary vector

- the number of preceding zero elements is coded for every 1 in the binary vector

- if the number of zero elements is greater than 0, the number plus one is signaled, due to the escape value of 1

- run-length value is coded using 0-order Exponential Golomb code

For example, let consider a binary vector {1100100010000}.

The signaled run-length values are 0-0-3-4-1.

# Simulation results

Run-length coding for the palette prediction was implemented on top of SCCE3 base software and similar to A.8 [1] test was performed based on common test conditions JCTVC-Q 1015. The results are summarized in the Table 1.

Table 1. A.8 (left), proposed method (middle), and proposed method with palette predictor size equal to 128 (right). Anchor is SCCE3base software (full-frame intra block copying).

Additionally, run-length coding for the palette prediction was implemented on top of C4v3 software tested in SCCE3, replacing the single level binary tree from A.8. Additionally, palette predictor size is increased from 64 to 128.

Table 2. C4v3 (left), C4v3 with proposed method (middle), and C4v3 with proposed method and palette predictor size equal to 128 (right). Anchor is SCCE3base software (full-frame intra block copying).

The results show up to 0.3% improvement of the proposed method and up to 0.9% improvement when the palette predictor size is increased to 128.

# Conclusion

Based on the results, it is proposed to adopt run-length method for the palette prediction into the palette method.

# References

[1] M. Karczewicz, W. Pu, V. Seregin, R. Joshi, and J. Sole, “SCCE3: Test A.8 – Improvements on Palette Prediction Vector Signaling”, JCTVC-R0063, 18th Meeting: Sapporo, JP, 30 June – 9 July 2014

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).