| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Sapporo, JP, 30 June – 9 July 2014 | Document: JCTVC-R0229 |
| --- | --- |

| Title: | Non-SCCE3: Palette predictor resetting |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Rajan Joshi, Marta Karczewicz, Wei Pu, Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is related to the palette method studied in SCCE3. In this method, the palette entries of the current CU can be predicted using the palette predictor consisting of the palette entries of the previous coded CUs. In current SCCE3 software, palette predictor is reset at each CTB row. This contribution proposes to do the palette predictor reset in the beginning of each slice and tile, and in the beginning of each CTB row if wavefront is used.  Otherwise, no palette predictor resetting is done. Removal of palette predictor resetting provides up to 1.2% luma BD-rate improvement.

# Introduction

Palette mode is been studied in SCCE3, where many methods of SCCE3 are based on the software distributed among SCCE3 participants and is used as software base. In the base SCCE3 software, palette entries can be predicted using the palette predictor consisting of the palette entries of the previous coded CUs. Additionally, the palette predictor is reset, meaning the length of the predictor is set to 0, at each CTB row.

The motivation of having such a reset probably is to avoid having a dependency on the previous CTB row when wavefront is in use, however this reset is not conditioned on the wavefront and applied also when wavefront is not used. The use of palette resetting shows performance loss.

# Remove palette predictor resetting

To check the impact of the palette predictor resetting, the reset is removed in the SCCE3 C4v3 test and the performance of all intra test based on common test conditions JCTVC-Q1015 is summarized in the Table 1. The more detailed results can be found in the accompanied excel tables.

Table 1. All intra tests of the C4v3 (left) and C4v3 with removed palette predictor reset (right). Anchor is SCCE3 base software (full-frame intra block copying).

As can be seen from the results, the performance loss can be up to 1.2% due to the palette predictor resetting.

To attenuate the loss, it is proposed the following rules for the palette predictor resetting:

- In the beginning of each slice

- In the beginning of each tile

- For each CTB row when wavefront is enabled.

And, do not perform palette predictor resetting in all other cases.

In some encoder designs, it may be preferred to have a parallel processing similar to wavefront for mode decision, in this case dependency on the previous CTB row palette entries may not be desirable. To enable such flexibility at encoder side, it is proposed to have a flag signaled in PPS, which will indicate the palette predictor resetting at each CTB row.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_palette_predictor_reset | u(1) |
| … |  |
| } |  |

pps_palette_predictor_reset equal to 1 specifies palette predictor resetting at every CTB row. pps_palette_predictor_reset equal to 0 specifies the absense of the palette predictor resetting. If pps_palette_predictor_reset is not present, it is inferred to be equal to 0. pps_palette_predictor_reset shall be equal to 1 if entropy_coding_sync_enabled_flag is equal to 1.

# Flexible palette predictor resetting

Proposed in the previous section palette resetting rules are mandated for both encoder and decoder, and decoder has to perform the necessary condition checks to comply with these rules. Moreover, there might be other cases when encoder wants to reset a palette predictor when previous palette entries used as a predictor are not relevant any more, for example this can happen when the content of the picture has changed.

Furthermore, it is proposed to perform a palette prediction resetting, i.e. setting the palette predictor size to 0, at every CU coded in the palette mode where there is no palette entries predicted from the previous CUs. In this case, if encoder wants to break the palette prediction, it simply should not predict any palette entry from the palette predictor, and the burden to control and conditions check would be performed by encoder, on the decoder side it will be only one check whether the current CU has the palette entries predicted from the palette predictor. This can be accomplished without additional syntax element proposed in the first method.

However, to avoid condition checks at decoder side, the palette prediction should have a way to indicate the number of predicted entries or the end of palette prediction independently from the palette predictor size, for example the method proposed in JCTVC-R0228.

The impact of the palette predictor resetting for the CUs having zero predicted palette entries has been simulated on the top of C4v3 test, and results are captured in the Table 2.

Table 2. All intra tests of the C4v3 (left) and C4v3 with proposed palette resetting (right). Anchor is SCCE3 base software (full-frame intra block copying).

# Conclusion

To improve the palette mode performance, it is proposed to disable palette predictor resetting and keep it only in the beginning of each slice and tile, and for each CTB row when wavefront is used. And to have more flexible control on the palette predictor resetting, it is proposed to perform the reset if the palette coded CU does not contain any predicted palette entry.

# Working Draft modification

Method 1

previous_palette_entry_flag[ i ] equal to 1 specifies that the palette entry from the previous used palette is copied. previous_palette_entry_flag [ i ] equals to 0 specifies that the palette entry from the previous used palette is not copied.

The previousPaletteSize is the size of the palette of the previous CU decoded in palette mode. The variable previousPaletteSize is set equal to 0 for the first decoded palette CU of each CTB line when pps_palette_predictor_reset is equal to 1 as well as in the beginning of each slice and tile.

Method 2

previous_palette_entry_flag[ i ] equal to 1 specifies that the palette entry from the previous used palette is copied. previous_palette_entry_flag [ i ] equals to 0 specifies that the palette entry from the previous used palette is not copied.

The previousPaletteSize is the size of the palette of the previous CU decoded in palette mode. The variable previousPaletteSize is set equal to 0 for the first decoded palette CU of each CTB line when all previous_palette_entry_flag[i] are equal to 0.

previous_palette_entry_flag[i] shall be equal to zero for each CTB row if entropy_coding_sync_enabled_flag is equal to 1, in the beginning of each slice and tile.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).